3GPP TSG-RAN WG2 Meeting #127	R2-2406813

Maastricht, Netherlands, Aug 19th – Aug 23rd, 2024

Agenda item:	8.3.4.1

Source:	Qualcomm Incorporated

Title:	RLF prediction and RLF prediction reporting

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# Introduction

In the last RAN2 #126 meeting [1], preliminary discussions on RLF prediction and Handover Failure prediction took place, and the following basic agreements were made.

RAN2 #126 agreements on RLF prediction and Handover Failure prediction [1]:

1	Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR).

2	Study Direct: Directly RLF prediction by AI/ML models.

3	FR2 study will be prioritized for RLF prediction

4	The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction.

5	HOF prediction is down prioritized in our study.  NO simulations/evaluations should be done/submitted

6	RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case.  FFS on expected RLF time and indirect case.

7	No evaluation/simulations are expected for August meeting for RLF

8	Simulation assumption specific to RLF will be discussed in August.  The assumption is that we will reuse RRM simulation assumptions (where possible).

In this contribution, we discuss RLF prediction and RLF prediction reporting.

- We discuss Indirect RLF prediction, the output of an AI/ML model for Indirect RLF prediction, and the KPIs for RLF prediction.

- We discuss the configuration and reporting of RLF prediction.

# RLF prediction

It was agreed in the last RAN2 #126 meeting to focus on the case of RLF due to physical layer problems, i.e., RLF detection that results from the timer T310 expiry, for both the cases of Direct and Indirect RLF prediction. We discuss in this section some aspects of Indirect RLF prediction.

Indirect RLF prediction can be performed on the UE side using an AI/ML model and based on UE measurements of source cell beams (RLM RSs). More specifically, and as was agreed in the last RAN2 #126 meeting, Indirect RLF prediction can be based on the temporal domain source cell predictions of RRM measurements.

An example of Indirect RLF prediction by the UE is shown in Figure 1 below. Based on UE measurements of the source cell beams at historical time instance(s), UE predicts the measurements for a window of time into the future, which we call the prediction window. During the time the timer T310 is running, based on measurement predictions during the prediction window, if UE determines that it is unlikely to have N311 consecutive in-sync indications during the prediction window and before T310 expires, then UE can predict occurrence of RLF.

If the UE is able to predict for a longer time into the future, i.e., the prediction window is longer, UE can predict RLF earlier in time, e.g., before the timer T310 is started, or even before having any out-of-sync indications. It is better if the UE is able to predict the occurrence of RLF as early as possible, because in that case the UE can report this to the network, which may then initiate handover of the UE to a suitable candidate cell thereby preventing the occurrence of RLF. It is assumed in the above discussions that the measurement predictions during the prediction window satisfy the RRM prediction accuracy requirements.

Figure 1: Example of Indirect RLF prediction by the UE

Proposal 1. In simulation evaluation, the AI/ML model for Indirect RLF prediction, which is based on the temporal domain source cell predictions of RRM measurements, produces as output one or more of the following:

- Prediction regarding whether RLF will occur in a certain window of time into the future (the prediction window).

- Predicted time of occurrence of RLF.

- Predicted times of occurrence of RLF and associated probabilities.

  - E.g., Probability of occurrence of RLF is 10% 100ms into the future, 20% 150ms into the future, etc.

As an example, consider Figure 1 above, and suppose that at the time when UE starts timer T310, the UE can predict, based on measurement predictions of the source cell, that over a prediction window duration of time into the future (assumed longer than the timer duration T310) N311 consecutive in-sync indications are unlikely. Then UE can predict at the time T310 is started that it is going to experience an RLF.

The above proposal is similar as what was agreed in the last RAN2 meeting (RAN2 #126) for the Direct RLF prediction case.

For the simulation evaluation for RLF prediction, the simulation assumptions and parameters regarding deployment and BS and UE parameters agreed for RRM measurement prediction should be reused as much as possible (this includes the L1 sampling period and the considered range of values of the prediction window).

Proposal 2. For the simulation evaluation for RLF prediction, RAN2 needs to discuss and agree on the settings of the RLF timers and constants T310, N310, and N311. For FR1, Table 1 below has some suggested values for the simulations.

Table 1: RLF parameters (for FR1)

| Items | Description |
| --- | --- |
| T310 | 1s |
| N310 | 1, 4, 10 |
| N311 | 1 |

Regarding the KPIs for the RLF prediction model, we think that prediction accuracy is an important metric. For the case where the output is a prediction about whether RLF occurs or not, the prediction accuracy metric can be the fraction of the RLFs that the model is able to predict correctly.

The model may also predict the occurrence of RLF erroneously, e.g., it may predict RLF when RLF does not occur. This is a false alarm which may impact system performance since the UE may transmit a report to the network indicating RLF, and the network and the UE may then take actions that are not necessary and may lead to performance deterioration.  Trying to maximize the prediction accuracy as defined above, i.e., the fraction of the RLFs that the model is able to predict accurately, could result in a high false alarm probability. We therefore suggest considering another KPI, which is the prediction accuracy that can be achieved for a given upper bound on the probability of false alarm.

Proposal 3. The following are the possible KPIs to consider for the RLF prediction model.

- Prediction accuracy.

For the case where the output is a prediction about whether RLF occurs or not, the prediction accuracy can be the fraction of the RLFs that the model is able to predict correctly.

- Maximum prediction accuracy that can be achieved for a given upper bound on the probability of false alarm.

# RLF prediction reporting

Network can configure the UE to provide RLF prediction reports.

Proposal 4. Network can configure the UE to provide an RLF prediction report when the UE can predict the occurrence of RLF. The configuration consists of:

- Prediction window time,

- Threshold for probability of occurrence of RLF during the prediction window time.

The start time of the prediction window can be a time after the first out-of-sync indication is detected by the UE.

The UE transmits the RLF prediction report if the probability of occurrence of RLF based on measurement predictions during the prediction window time, exceeds the configured threshold.

Proposal 5. The RLF prediction report provided by the UE to the source gNB includes:

- The predicted time of occurrence of RLF, or information related to the distribution (e.g., CDF) of the predicted time,

- Source cell RRM measurements or measurement predictions.

Figure 2: RLF prediction reporting and its configuration

# Conclusions

Based on the above discussions, we recommend that RAN2 discuss the following observations and proposals.

RLF prediction

Proposal 1. In simulation evaluation, the AI/ML model for Indirect RLF prediction, which is based on the temporal domain source cell predictions of RRM measurements, produces as output one or more of the following:

- Prediction regarding whether RLF will occur in a certain window of time into the future (the prediction window).

- Predicted time of occurrence of RLF.

- Predicted times of occurrence of RLF and associated probabilities.

  - E.g., Probability of occurrence of RLF is 10% 100ms into the future, 20% 150ms into the future, etc.

Proposal 2. For the simulation evaluation for RLF prediction, RAN2 needs to discuss and agree on the settings of the RLF timers and constants T310, N310, and N311. For FR1, Table 1 below has some suggested values for the simulations.

Table 1: RLF parameters (for FR1)

| Items | Description |
| --- | --- |
| T310 | 1s |
| N310 | 1, 4, 10 |
| N311 | 1 |

Proposal 3. The following are the possible KPIs to consider for the RLF prediction model.

- Prediction accuracy.

For the case where the output is a prediction about whether RLF occurs or not, the prediction accuracy can be the fraction of the RLFs that the model is able to predict correctly.

- Maximum prediction accuracy that can be achieved for a given upper bound on the probability of false alarm.

RLF prediction reporting

Proposal 4. Network can configure the UE to provide an RLF prediction report when the UE can predict the occurrence of RLF. The configuration consists of:

- Prediction window time,

- Threshold for probability of occurrence of RLF during the prediction window time.

The start time of the prediction window can be a time after the first out-of-sync indication is detected by the UE.

The UE transmits the RLF prediction report if the probability of occurrence of RLF based on measurement predictions during the prediction window time, exceeds the configured threshold.

Proposal 5. The RLF prediction report provided by the UE to the source gNB includes:

- The predicted time of occurrence of RLF, or information related to the distribution (e.g., CDF) of the predicted time,

- Source cell RRM measurements or measurement predictions.

# References

[1] Chair notes, RAN2 #126 meeting.