3GPP TSG-RAN WG2 Meeting #127	R2-2406817

Maashricht, Netherland, 19th – 23th August, 2024

Agenda item:	6.1.3.2

Source: 	Huawei, HiSilicon

Title: 	Corrections to UE capabilities related to Rel-17 URLLC and RedCap

Document for:	Discussion and Decision

# 1. Introduction

In this contribution, the following 2 Rel-17 UE capabilities with FRx-diff is discussed.

(i) cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17

(ii) multipleCORESET-RedCap-r17

# 2. Discussion

We have noticed some discrepancies between ASN.1 in TS38.331 [2] and TS 38.306 [3] on the following UE capabilities:

(i) cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17

In TS 38.306, the FRx-diff is set to No as follow:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17 Indicates whether the UE supports subband CQI reporting with 4 bits per subband for TN and non-shared spectrum channel access. | UE | No | No | No |

This is aligned with the R1 feature list as follow [1]:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-11 | 4-bits subband CQI | Subband CQI reporting with 4 bits per subband |  | Yes | N/A |  | Per UE | No | No | N/A | This FG is reported for TN and licensed | Optional with capability signaling |

However, in the ASN.1 in TS 38.331, this is implemented as per UE capability with FRx-diff as highlighted yellow as follow in Phy-ParametersFRX-Diff:

Phy-ParametersFRX-Diff ::=                  SEQUENCE {

<<Ommitted>>

twoDifferentTPC-Loop-PUCCH                  ENUMERATED {supported}                      OPTIONAL,

pusch-HalfPi-BPSK                           ENUMERATED {supported}                      OPTIONAL,

pucch-F3-4-HalfPi-BPSK                      ENUMERATED {supported}                      OPTIONAL,

almostContiguousCP-OFDM-UL                  ENUMERATED {supported}                      OPTIONAL,

sp-CSI-RS                                   ENUMERATED {supported}                      OPTIONAL,

sp-CSI-IM                                   ENUMERATED {supported}                      OPTIONAL,

tdd-MultiDL-UL-SwitchPerSlot                ENUMERATED {supported}                      OPTIONAL,

multipleCORESET                             ENUMERATED {supported}                      OPTIONAL,

...,

<<Ommitted>>

[[

-- R1 11-1b: Type 1 HARQ-ACK codebook support for relative TDRA for DL

type1-HARQ-ACK-Codebook-r16                 ENUMERATED {supported}                      OPTIONAL,

-- R1 11-8: Enhanced UL power control scheme

enhancedPowerControl-r16                    ENUMERATED {supported}                      OPTIONAL,

-- R1 16-1b-1: TCI state activation across multiple CCs

simultaneousTCI-ActMultipleCC-r16           ENUMERATED {supported}                      OPTIONAL,

-- R1 16-1b-2: Spatial relation update across multiple CCs

simultaneousSpatialRelationMultipleCC-r16   ENUMERATED {supported}                      OPTIONAL,

cli-RSSI-FDM-DL-r16                         ENUMERATED {supported}                      OPTIONAL,

cli-SRS-RSRP-FDM-DL-r16                     ENUMERATED {supported}                      OPTIONAL,

-- R1 19-3: Maximum MIMO Layer Adaptation

maxLayersMIMO-Adaptation-r16                ENUMERATED {supported}                      OPTIONAL,

-- R1 12-5: Configuration of aggregation factor per SPS configuration

aggregationFactorSPS-DL-r16                 ENUMERATED {supported}                      OPTIONAL,

-- R1 16-1g: Resources for beam management, pathloss measurement, BFD, RLM and new beam identification

maxTotalResourcesForOneFreqRange-r16        SEQUENCE {

maxNumberResWithinSlotAcrossCC-OneFR-r16    ENUMERATED {n2, n4, n8, n12, n16, n32, n64, n128}    OPTIONAL,

maxNumberResAcrossCC-OneFR-r16              ENUMERATED {n2, n4, n8, n12, n16, n32, n40, n48, n64, n72, n80, n96, n128, n256}

OPTIONAL

}                                           OPTIONAL,

-- R1 16-7: Extension of the maximum number of configured aperiodic CSI report settings

csi-ReportFrameworkExt-r16                  CSI-ReportFrameworkExt-r16                  OPTIONAL

]],

[[

twoTCI-Act-servingCellInCC-List-r16         ENUMERATED {supported}                      OPTIONAL

]],

[[

-- R1 22-11: Support of 'cri-RI-CQI' report without non-PMI-PortIndication

cri-RI-CQI-WithoutNon-PMI-PortInd-r16       ENUMERATED {supported}                      OPTIONAL

]],

[[

-- R1 25-11: 4-bits subband CQI for TN and licensed

cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17  ENUMERATED {supported}               OPTIONAL

]],

[[

multipleCORESET-RedCap-r17                  ENUMERATED {supported}                      OPTIONAL

]]

}

Observation#1: The ASN.1 for cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17 is not correctly implemented according to the R1 feature list.

There are 2 ways to solve the issue with no ASN.1 NBC:

Approach 1: Dummify the existing field and create a new UE cap in the common field as part of the late non-critical extension.

Approach 2: Update the field description that the same value is used for both frequency range FR1 and FR2. For example:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17 Indicates whether the UE supports subband CQI reporting with 4 bits per subband for TN and non-shared spectrum channel access. In this release, the same value shall be indicated for the frequency ranges. | UE | No | No | Yes |

As on interoperability, Approach 1 has no interoperability issue, while for Approach 2, there should not be since the ASN.1 is already implemented with FRx-diff and if the UE provides a different value, it can be left to network implementation.

In view that Approach 2 is simpler and does not increase the overhead, we have a slight preference for Approach 2:

Proposal#1: Update the field description that the same value is used for both frequency range FR1 and FR2. For example:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17 Indicates whether the UE supports subband CQI reporting with 4 bits per subband for TN and non-shared spectrum channel access. In this release, the same value shall be indicated for the frequency ranges. | UE | No | No | Yes |

(ii) multipleCORESET-RedCap-r17

As seen highlighted green in the ASN.1 in (i), the multipleCORESET-RedCap-r17 is implemented with FRx -diff in Phy-ParametersFRX-Diff and the reason is to align it with multipleCORESET which is also in Phy-ParametersFRX-Diff. Hence this is correctly implemented in the ASN.1.  However, this is not correctly reflected in TS38.306 as follow where the FR1-FR2-DiFF column is set to ‘No’. This should be corrected.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| multipleCORESET-RedCap-r17 Indicates whether the RedCap UE supports configuration of up to three PDCCH CORESETs in the RedCap specific initial DL BWP when it does not contain CD-SSB and CORESET#0. If this is not supported, the field description of multipleCORESET applies to the RedCap-specific initial BWP. The RedCap UE reporting this capability shall also report multipleCORESET. | UE | No | No | No |

As on interoperability, we think that there should not be since the ASN.1 is already implemented with FRx-diff.

Observation#2: The FR1-FR2-diff for multipleCORESET-RedCap-r17 should be changed to ‘Yes’.  There is no interoperability issue foreseen.

Proposal#2: The field description for multipleCORESET-RedCap-r17 should be updated as follow:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| multipleCORESET-RedCap-r17 Indicates whether the RedCap UE supports configuration of up to three PDCCH CORESETs in the RedCap specific initial DL BWP when it does not contain CD-SSB and CORESET#0. If this is not supported, the field description of multipleCORESET applies to the RedCap-specific initial BWP. The RedCap UE reporting this capability shall also report multipleCORESET. | UE | No | No | Yes |

# 3. Conclusion

It is requested that RAN2 discuss the following observation and proposal:

Observation#1: The ASN.1 for cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17 is not correctly implemented according to the R1 feature list.

Proposal#1: Update the field description that the same value is used for both frequency range FR1 and FR2. For example:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17 Indicates whether the UE supports subband CQI reporting with 4 bits per subband for TN and non-shared spectrum channel access. In this release, the same value shall be indicated for the frequency ranges. | UE | No | No | Yes |

Observation#2: The FR1-FR2-diff for multipleCORESET-RedCap-r17 should be changed to ‘Yes’.  There is no interoperability issue foreseen.

Proposal#2: The field description for multipleCORESET-RedCap-r17 should be updated as follow:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| multipleCORESET-RedCap-r17 Indicates whether the RedCap UE supports configuration of up to three PDCCH CORESETs in the RedCap specific initial DL BWP when it does not contain CD-SSB and CORESET#0. If this is not supported, the field description of multipleCORESET applies to the RedCap-specific initial BWP. The RedCap UE reporting this capability shall also report multipleCORESET. | UE | No | No | Yes |

# 4. References

[1]. R1-2304113 Updated RAN1 UE features list for Rel-17 NR after RAN1#112bis-e

[2] TS38. 331 v17.9.0

[3] TS38.306 v17.9.0