3GPP TSG-RAN WG2 #126bis	R2-2406818

Maastricht, Netherlands, Aug 19th - 23rd 2024

Agenda Item:	8.2.2

Source:	Ericsson

Title:	Functionality aspects for A-IoT

Document for:	Discussion/Decision

# 1	Introduction

This paper discusses aspects on the functionality required for A-IoT devices, including security related questions to SA3, the need for resource allocation, segmentation/reassembly (pending RAN1 progress), QoS handling, higher layer repetition, BSR and SR. Note that in parts were referring to “Reader”, this would apply to either a gNB or an Intermediate node as Reader, described in SID and elsewhere.

# 2	Discussion

## 2.2	Protocol stack and RAN awareness.

### 2.2.1	UP

In the last RAN2 meeting it was agreed that SDAP, PDCP, and RLC will not be supported for A-IoT as in NR; nevertheless, it is still FFS whether and how functionalities typically associated to these layers should be supported. Specifically, AS security and segmentation.

In our understanding, this does not mean that functionality provided by the currently excluded protocol layers (c.f. NR), if needed, would necessarily be supported by MAC. Additionally, a separate layer may be defined, as features are FFS pending agreements in RAN2 and in other working groups (RAN1, SA3 and other). For the time being we propose to assume only PHY and MAC will be the layers in the UP-protocol stack.

Previous agreement on not supporting SDAP, PDCP, RLC does not prevent the definition of a new AS protocol layer in the UP-protocol stack above MAC.

The resulting baseline UP protocol stack is shown in Figure 1.

Figure 1: User plane protocol stack for A-IoT

### 2.2.2	CP

In the last meeting it was agreed that A-IoT will not include RRC connection management, periodic System Information and that A-IoT devices will not support ASN.1 encoding/decoding.

If we assume that MAC will handle the set of functions (denoted as “MAC functions”) typically part of MAC also in legacy, (e.g.: random access procedure, handling of logical channels or padding) there is still several functions for which an entity needs to be defined.

The functions (denoted as “Device Control” functionality) currently at hand are:

- Handling of the inventory procedure at RAN level (i.e.: triggering multiple RA procedures, selecting the appropriate configuration for it, and determining when to terminate the inventory procedure)

- Delivery of PDUs from/to upper layers (e.g.: NAS)

- Delivery of data (e.g.: DL command and reply use case)

- Control signalling to one group or a specific device (as needed)

- Handling of AS identity (if needed, see Section 2.8)

In principle these “Device Control” functions could be incorporated with the MAC layer or alternatively separated into a different, upper layer, similar to what the RRC layer is in legacy NR. Both options have pros and cons that should be studied in RAN2.

Both “Device Control” and “MAC” functions will require control elements. In legacy MAC they are encoded through LCID code points(c.f. MAC CE), and in legacy RRC they are handled by ASN.1 encoding. As a result, we can imagine the method used for the “Device Control” signalling will be similar to the one used in MAC since ASN.1 UPER encoding will not be supported. If the number of needed “MAC” and “Device Control” control elements will be m and n respectively, having them in separate layers will require an overhead of at least log2m+1+log2n bits to encode (+1 to handle a dedicated logical channel directed to the upper layer), while if they are all handled by the same layer, they would require an overhead of log2m+n. In general, handling control elements in a single layer will require a lower number of bits.

Merging the “MAC” and “Device Control” functionalities in a single layer implies a smaller overhead in terms of encoding of control elements.

In terms of complexity, having many functions in the MAC will increase layer complexity. Since this would cause a considerable re-design of MAC layer on network side as functions that typically are handled by RRC would be placed into MAC, whereas separating functional responsibility could simplify the design and specification of layers.

Merging the “MAC” and “Device Control” functionality in a single layer implies a higher complexity in terms of modelling and lower reusability of network-side implementation of such layer.

Study the pros and cons of merging “MAC” and “Device Control” functions into a single layer or in multiple layers.

The CP protocol stack as discussed above is shown in Figure 2.

Figure 2: Control plane protocol stack for A-IoT

### 2.2.3	Context awareness

In our view, A-IoT services including inventory or command are initiated/triggered by CN/application by nature. It thus makes sense to deploy and configure A-IoT (NAS like) in an end-to-end manner, i.e., information and signalling is exchanged between devices and CN/Application directly. This in turn may also lead to less AS complexity while meeting security standards as set for 5GS. In any case, this should be part of the RAN2 study and be included in liaisons with other working groups.

In addition, whether Inventory request message shall be transparent to Reader needs to be discussed and studied.

As discussed in [5], we foresee a few ways of supporting inventory, command and registration from CN-to-RAN perspective. It is important to define the protocol stack of all interfaces across the network to determine exactly what information are available to the reader and what is transferred transparently. For instance, information generated by the XXAP (RAN3 term) would be available to the reader, while information generated by NAS would not.

It is also important to clarify what we mean with “reader” when discussing the FFS about the level of visibility required by the reader. In the general architecture currently depicted in the TR draft the “RAN” is split into two entities (Common Reader Function and AIoT RAN node function) which in topology 1 they both belong to the same device (Reader) but in topology 2 are split in two different devices. In general, information relevant for AIoT radio interface should be available at the Common Reader Function, while information for correct interfacing towards the CN should be available at the AIoT RAN node function.

From AS perspective it is useful for the reader to get some information about the device population and capabilities to optimize and properly execute the inventory procedure or the delivery of commands. Nevertheless, there will likely be a set of minimum capabilities that all devices should satisfy, so the inventory would be configured accordingly as a fallback. Below a summary with the information that are required for the procedure to work, and the information that could enhance the performance but are not strictly necessary.

- Required: AMF identifier for NAS Node selection function (NNSF) (in legacy this is part of 5G-S-TMSI which is sent over RRC), Service type (command or inventory). This is mainly RAN3 and other groups to define, but the assumption of the identifier is useful in RAN2.

- Enhancements: Device population size (inventory), UE capabilities, Session ID (to avoid duplicate replies from devices)

Study whether reader needs to be aware of the below information:

  - AMF identifier for NAS Node selection function (NNSF) (FFS: if 5G AIoT Device Identifier is used instead as in legacy)

  - Device population size

  - Device capabilities (e.g.: device type and/or support for frequency shift)

  - Inventory Session ID/transaction ID

## 2.3	Resource allocation

Upon triggering of an inventory procedure or a command signalling (e.g., received from the CN or AF), the reader needs to trigger a query/inventory procedure (e.g., trigger message, random access parameters, scheduling info etc) towards the intended devices. During the procedure, the reader is expected to provide resource allocation to devices for their UL transmissions.

The resource allocation mechanisms would be different for CBRA and CFRA. The first would allow the access of a device to a random resource out of a larger pool of resources (i.e., randomly selecting a slot instead of a preamble as in legacy), the second would allow the direct scheduling of the device in a dedicated resource, like what happens in NR when an UL grant is provided.

In case of CBRA the device needs to be informed about the pool of resources it can choose randomly from. With the current agreements that at least slotted-ALOHA is supported, this information should at least include the number of slots that are available and possible the offset in time of the first eligible slot. A slot duration can be flexible given that RAN1 agreed to study asynchronous operation as the baseline.

For a CBRA procedure, the reader provides in DL signalling at least pool, or set of resources, from which devices can randomly select their UL transmission resource.

For CFPR, the reader needs to provide dedicated resources to the target devices (one or multiple).

For a CFRA procedure, the reader must provide dedicated resources to a specific device.

## 2.6	Scheduling Request, Segmentation, and other flags

In previous meetings it was agreed that legacy NR BSR/SR will not be supported for A-IoT as well as any form of upper layer ARQ-like retransmission while it is still FFS how to handle segmentation depending on RAN1 agreements.

Nevertheless, the maximum supported TBS from the PHY layer should still be determined. In most cases it is fine to assume that the UL data has a constant or at most predictable size so that the reader can allocate the appropriate TBS to complete the transmission without the need for segmentation, but it is unlikely that this mechanism will be sufficient in all possible scenarios. For instance, in the DL command case, the device reply may have unexpected size due to error on application side. Moreover, depending on RAN1 agreements, it may be the case that the maximum TBS cannot satisfy all possible use cases.

Therefore, it is wise to consider a segmentation mechanism or a method for the device to request more resources to the reader for D2R transmission (similar to Scheduling Request).

There might be cases where segmentation and/or scheduling request mechanisms are needed to satisfy all possible use cases depending on RAN1 agreements on maximum TBS.

The complexity of this mechanism should be reduced to the minimum also in terms of control signaling overhead for obvious reasons.

In legacy NR there are several methods to implement behaviors coherent with the problem under exam:

- Scheduling Request (SR): a special PHY layer message, carried over PUCCH, that simply request an UL Grant to the gNB. If RRC Connection is not established the UE would start a RACH instead

- Buffer Status Report (BSR): a MAC CE with which a UE can inform the gNB on the current amount of pending data with more or less granularity. The gNB can then decide whether or not to provide an UL grant and how large.

- Segmentation: a RLC SDU can be split in multiple PDUs. Two control fields keep track of whether the PDU contains an entire SDU or the first, last or neither first nor last segment (SI field, 2 bits) and the start and final offset in the overall SDU of a specific PDU (SO field, 16 bit + 16 bit)

For A-IoT the most common scenario will be that the first UL grant provided by the reader is sufficient to deliver the whole SDU. For those possible cases for which the grant is not enough, the device should ask for further resources. At the same time, since ARQ-like retransmissions will not be supported, it is reasonable to assume that the device can simply transmit each segment one after the other and it is successful if all the segments were delivered successfully without any other segment retransmission mechanism.

Study how to address the case of one UL grant not being enough to deliver the full SDU assuming in-order delivery and no specific mechanisms to handle failure of delivery and retransmission of some of the segments.

To satisfy these assumptions, a simple MAC CE containing a Boolean flag with a similar meaning as legacy SR may be enough.

## 2.7	Repetition

RAN1 has made the below agreement regarding repetition in previous meetings:

Agreement

Study D2R transmission in the physical layer using repetition

- Note: Discussions regarding higher-layer repetitions are up to RAN2.

Agreement

Define repetition types for study purposes as follows:

- Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times

- Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times

- Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times

- Chip level: Each chip after line coding (if used) or after square wave modulation (if used) is repeated Rchip times

  - NOTE: Equivalent to extending the duration of each chip by Rchip times

Agreement

- For D2R, study at least block-level and bit-level repetition type 1 and type 2.

Repetition as one function to improve transmission reliability needs to be studied and supported. In addition, repetition can be interpreted as a form of a retransmission mechanism. This is an important aspect to note, especially given RAN2 has agreed to not support HARQ and ACK for A-IoT. We think higher layer repetition can be implemented in MAC if to be supported. Like NR, a device can be informed by a Reader via e.g., a L1 or L2 signaling.

The rationale to decide whether repetitions are needed or not should be that a device in cell edge should deliver its packet with a certain success probability. If one transmission in those conditions cannot achieve that probability, then repetitions are needed, regardless of which layer implements them.

Given a minimum SINR (cell edge), repetitions, at any layer, are needed if a single transmission cannot achieve a certain success probability requirement.

A success probability target should be defined at cell edge for a certain TBS as well as a definition of cell edge in terms of minimum SINR

It is in RAN1 scope to study PHY layer repetitions, both block-wise and bit-wise. In principle PHY layer alone may be able to configure this functionality to achieve any desired success rate given a definition of minimum SINR. If this is the case, any transmission that is not received correctly may be considered as failed because the device was “out of coverage” and no particular action should be taken by the 3GPP network provided that the device does not consider itself as inventoried. This does not prevent the application to request again the inventory of such device.

RAN2 repetitions are needed if RAN1 repetitions (if any) cannot achieve the success probability requirement at cell edge.

In case PHY layer does not implement repetitions, or the PHY layer repetitions are not enough to satisfy the requirement or if one repetition is not enough to satisfy the requirement, RAN2 should study a repetition mechanism to achieve the success probability target. It may be convenient to start studying that while waiting for more detailed RAN1 evaluations and agreements on the matter.

RAN2 to study repetitions mechanism in the hypothesis that PHY layer repetitions or just one transmission is not enough to satisfy a success probability requirement at cell edge.

Since data delivery will likely be part of RACH procedure, a simple RAN2 mechanism to implement repetitions may be performing multiple RACH attempts, similarly to what happens in legacy.

RAN2 to consider multiple RACH attempts as a mechanism to implement repetitions.

## 2.8	Device Addressing and AS identifier handling

The UE should be able to determine if a message received on the shared DL channel is directed to itself or not.

In legacy this is achieved both by having a broadcast transport channel (BCH), or by scrambling the PDCCH associated with a DL transmission using an identifier (RNTI) that the MAC entity in the UE monitors (if the RNTI matches the ones monitored by the UE, the following DL transmission is directed to the UE).

A similar functionality should be supported for A-IoT since the DL channel is also shared, but it may be simplified considering the different transmission scheme.

Study how the A-IoT device determines if a DL transmission in the shared downlink channel (PRDCH) is directed to itself and whether it should be MAC or PHY responsibility.

For the remainder of the section, we call “AS ID” the information needed to distinguish transmission from/to different devices from MAC perspective. (i.e.: it is out of scope higher-layer perspective such as NAS, Application or inventory as a whole). Also, we distinguish transmissions happening before and after the generation of the AS ID.

### 2.8.1	Premise on Logical Channels

RAN1 has agreed that PHY layer will support PRDCH for DL data transmission, PDRCH for UL data transmission, and not to support a broadcast physical channel (PBCH). This does not necessarily imply a certain set of supported transport channels are concluded for use in the interface between PHY and MAC layers.

In legacy NR the MAC PDUs received from the BCH are processed assuming a transparent MAC PDU format which does not add any overhead information. This is typically used for MIB delivery. In addition, in NR, PCH is used for paging message delivery. Whether the BCH and the PCH is needed would be depending on RAN1 decision.

DL-SCH (mapped to PRDCH) and UL-SCH (mapped to PDRCH) is the minimum set of transport channels provided by PHY layer.

It is important to notice, though, that some messages in DL should be transmitted in a broadcast manner. An example of this is the initial trigger message targeting all devices in reader coverage. For modelling purposes, with no necessary implication on device complexity, it is convenient to define a Broadcast channel (BCH).

Notice that even if an inventory request targets a specific device, this message may be generated by layers above MAC and/or without knowledge if the device is present in a certain coverage area. Therefore, all devices should check if the request applies to them or not (i.e.: it is a broadcast message).

Similarly, some messages during RACH are sent in a contention-based fashion and should be handled differently than contention-free UL transmission. It is again convenient to define a logical channel “RACH” to group all messages transmitted in a contention-based manner.

DL-SCH (mapped to PRDCH), BCH (mapped to PRDCH), UL-SCH (mapped to PDRCH) and RACH (mapped to PDRCH) logical channels are defined at least for modelling purposes (not necessarily with complexity impact).

Table 1: Logical-to-Physical channel mapping and messages exchanged in each logical channel.

| PHY Channel | PRDCH |  | PDRCH |  |
| --- | --- | --- | --- | --- |
| Logical Channel | BCH | DL-SCH | UL-SCH | RACH |
| Messages | - A-IoT Paging message - Inventory Request - A-IoT Msg2 | - A-IoT Paging message - Inventory Request (specific device only) - Command Request - Any other R2D data TX | - Inventory Response - Command Response - Any other D2R data TX | - A-IoT Msg1 (with or without data) |

It is important to notice that the messages in DL-SCH and UL-SCH are the ones that should be delivered in a unicast manner from MAC perspective. Therefore, the addressing problem explained in this section applies only to those messages and logical channels.

Only messages delivered through UL-SCH and DL-SCH are affected by the addressing problem from MAC perspective.

### 2.8.2	Transmissions after acquiring AS ID

Assuming both the device and reader agreed on an AS ID, there should be a method to distinguish transmissions based on this AS ID. We foresee that should be studied in cooperation with RAN1.

For UL transmissions over UL-SCH, the device will receive an UL Grant from the reader, so there is no ambiguity from reader perspective regarding which device is transmitting and thus there is no need to include AS ID in control signalling.

After acquiring the AS ID, the only way a device can transmit in UL is by receiving an UL Grant, therefore the reader always knows which device is transmitting in that resource.

For UL-SCH after acquiring the AS ID, assume implicit addressing (no need for control signalling to distinguish between MAC PDUs from different devices).

For DL-SCH, there might be ambiguities especially considering that any response from the device is triggered by DL messages at any time. We foresee four main options on how to handle the addressing problem in DL:

- If during a procedure only the addressed device is active to transmit/receive messages, there is no ambiguity on which is the addressed device and therefore the addressing may be implicit (i.e.: no need to include the AS ID in PHY or MAC control signaling). For instance, in RFID in a given slot only one device reaches the state after contention resolution where it can transmit data and until a new slot is initiated (with a QueryRep command) all the other devices must remain silent.

- A second option is that the addressing of devices may be handled entirely by the PHY layer, meaning that appropriate control signalling will be included in the physical layer packet (i.e.: not part of MAC PDU) and it will be PHY layer responsibility to pass to the upper layers, through the transport channels, only MAC PDUs that are addressed to the device. In this case no control signalling should be added by MAC layer, but there should be a mechanism to inform the PHY layer on the current AS ID in case the generation of it was higher layer responsibility. In this case BCH and DL-SCH logical channel will have the same behaviour and can be merged into DL-SCH.

- A third option is that the addressing is handled by MAC layer. This means that a MAC CE or a MAC field should be defined to include the AS ID to determine who is the source/target of a certain MAC PDU.

- Hybrid options are also possible. For instance, legacy is more similar to a mix of options b and c. In fact, it’s MAC responsibility to discard or keep MAC PDUs based on the RNTI that was used in PDCCH, but it’s PHY layer signalling that carries the RNTI itself (scrambling of PDCCH) and it’s PHY layer that informs MAC layer which RNTI was used for a certain PDSCH transmission.

For DL-SCH study pros and cons of the following options regarding device addressing problem:

  - Implicit addressing (reader/standard prevent transmission of other devices’ MAC PDUs at the same time). This cannot apply to CFRA.

  - Control information included in PHY PDU, mechanism to determine and assign identifier up to upper layers.

  - Control information included in MAC PDU (e.g., MAC CE), mechanism to determine and assign identifier up to upper layers.

  - Any hybrid of the previous 3 options.

### 2.8.3	Generation, provisioning, and validity of the AS ID

If a device is not storing any AS ID, there is a need to define one during the initial access. Therefore, this sub-section refers to the part of the procedure “before acquiring the AS ID”.

There may be two possible ways the device performs initial access, Contention-Based RACH (CBRA) and Contention-Free RACH (CFRA). CFRA presumes the presence of an agreed AS ID since the reader allows the device to transmit directly in a dedicated resource and it needs to know exactly which device is using that resource. For this reason, the generation of the AS ID must be associated to the CBRA access type.

Generation and distribution of the AS ID must happen during an initial access during or right after a Contention-Based RACH

More specifically, the generation and distribution of the AS ID may happen with one of the following three mechanisms:

- Agreed during CBRA procedure itself.

- Decided by the reader and communicated to the device in the A-IoT paging step.

- Agreed with further signalling after completion of CBRA (i.e.: after contention resolution)

With option c there would be an increased signalling overhead and overall duration of the inventory procedure. Moreover, there are ways to implement options a and b. Unless a clear issue with the other methods is highlighted, we propose to consider only options a and b.

Study how the AS ID is generated and distributed either during the CBRA procedure or generated by the reader.

During CBRA an identifier that is exchanged between reader and device is the randomly generated ID that is then used to resolve contention. A simple way of implementing option a could be to promote this ID to AS ID once contention has been resolved. There would be a non-zero probability that the randomly generated ID is equal to an AS ID that some other device is currently storing, therefore this method does not guarantee unique IDs, but we may argue that if this probability is low enough, the method is acceptable.

On the other hand, in the specific case of inventory request directed to a specific device, the reader may already generate an AS ID and distribute it in the A-IoT paging message together with a higher-layer ID that is actually used by device to react to the request. The device would simply start using it, potentially also to perform immediately a CFRA. Nevertheless, when the network triggers a device-specific command/inventory it doesn’t know with certainty where the device is, if it is present at all. Therefore, this AS ID and potential resources for CFRA must be allocated in all readers that may be serving the device. This consumes more resources than the method described above and it only applies in a subset of all the possible cases.

Another option, more like legacy behaviour, is that A-IoT Msg2 contains both the contention resolution ID (i.e.: a copy of the randomly generated ID from Msg1) and a temporary AS ID that will be promoted to AS ID once contention has been resolved. This is analogous to legacy TC-RNTI transmitted in Msg2 which later becomes C-RNTI after contention resolution.

Regarding the validity of the AS ID, it is not strictly necessary for the device to store it in NVM for the following transactions. In principle each transaction may start with a CBRA and thus a new AS ID may be allocated every time.

Nevertheless, it is in the study scope to consider CFRA, which implies an AS ID was agreed in a previous transaction and stored. Also, avoiding a CBRA every time would improve the performance of the inventory procedure.

If a device can store the AS ID across different transactions, the network can take advantage of it to reduce signalling overhead, otherwise a new AS ID may be generated at every transaction (which prevents the possibility to have CFRA)

If the AS ID can be stored, there should be a mechanism in place to let the network know if the device still has it in memory to select the appropriate initial access procedure. One possibility may be to specify in the standard a clear requirement in terms of lifetime of the AS ID in memory so that a simple timer on the reader can keep track of it. It is unclear, though, it is possible to guarantee this lifetime considering device power limitations.

Study whether a requirement on how long the AS ID can be kept in memory is feasible and reliable.

In the hypothesis that this requirement cannot be trusted, there is a need for an explicit mechanism to inform the reader if the device has or not an AS ID in memory.

If the reader initiates a CFRA or a DL-SCH transmission, and this does not have stored the AS ID anymore, it will simply not respond. Then, it may be up to reader implementation to consider the device as out of coverage or try again with a different method. For example, the reader may then initiate an inventory procedure targeting the specific device. This trigger message would be carried over BCH and thus does not require AS ID. At the same time a CBRA would be triggered so that a new AS ID is generated.

If a reader initiates a procedure assuming AS ID is stored, but it’s not, the device will not reply. Then the reader may start an inventory to a dedicated device with CBRA access

### 2.8.4	Transmissions before acquiring AS ID

It remains to be solved how to distinguish transmissions to different devices before the AS ID has been agreed between reader and device. If the network knows that the device does not have an AS ID (see Section 2.8.3), and considering that from previous discussions only messages transmitted over DL-SCH have this problem, we can see from Table 1 that:

- A-IoT Paging message: this can be delivered via BCH instead.

- Inventory Request (specific device only): this can be delivered via BCH instead.

- Command Request: this is sent only after inventory has been completed or with CFRA which presumes AS ID is already present

- Any other R2D data TX: same as Command Request.

With the other assumptions in this section, there is no need to handle addressing problem for messages transmitted over DL-SCH before acquiring AS ID.

Details on how to handle transmissions over RACH is handled as part of Random Access discussion.

## 2.9	Other MAC aspects

In the previous meeting some companies proposed the concept of not having octet aligned MAC PDU (or upper layer SDUs). It may be that the amount of control information to be sent is very low, just a few bits. In such case it might be beneficial to avoid octet alignment to avoid padding bits.

This discussion of course depends on how flexible and generic the TBS allocation is expected to be, also in future extensions.

Nevertheless, typically the length of a SDU is given in octet in NR. Avoiding octet-alignment means that now the length should be given in bits, which would increase the amount of signaling bits for indicating the TBS size.

- As a result, It is not obvious, without other assumptions, to determine if bit-alignment would lead to a more efficient MAC PDU encoding than octet-alignment.

Moreover, considering device limitations, memory works by addressing words of various length, typically a byte. Therefore, it might create excessive complexity on device side to have bit-aligned buffers and stored data.

Also, current network design assumes octet alignment. Changing this may create excessive implementation effort on network side.

Bit-alignment may cause excessive complexity and implementation effort on both device and network size due to the normal functioning of memory and existing implementations.

Include in the TR that bit-aligned MAC PDUs is out of the scope of the study.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Previous agreement on not supporting SDAP, PDCP, RLC does not prevent the definition of a new AS protocol layer in the UP-protocol stack above MAC.

Observation 2	Merging the “MAC” and “Device Control” functionalities in a single layer implies a smaller overhead in terms of encoding of control elements.

Observation 3	Merging the “MAC” and “Device Control” functionality in a single layer implies a higher complexity in terms of modelling and lower reusability of network-side implementation of such layer.

Observation 4	There might be cases where segmentation and/or scheduling request mechanisms are needed to satisfy all possible use cases depending on RAN1 agreements on maximum TBS.

Observation 5	Given a minimum SINR (cell edge), repetitions, at any layer, are needed if a single transmission cannot achieve a certain success probability requirement.

Observation 6	A success probability target should be defined at cell edge for a certain TBS as well as a definition of cell edge in terms of minimum SINR

Observation 7	RAN2 repetitions are needed if RAN1 repetitions (if any) cannot achieve the success probability requirement at cell edge.

Observation 8	Only messages delivered through UL-SCH and DL-SCH are affected by the addressing problem from MAC perspective.

Observation 9	After acquiring the AS ID, the only way a device can transmit in UL is by receiving an UL Grant, therefore the reader always knows which device is transmitting in that resource.

Observation 10	Generation and distribution of the AS ID must happen during an initial access during or right after a Contention-Based RACH

Observation 11	If a device can store the AS ID across different transactions, the network can take advantage of it to reduce signalling overhead, otherwise a new AS ID may be generated at every transaction (which prevents the possibility to have CFRA)

Observation 12	If a reader initiates a procedure assuming AS ID is stored, but it’s not, the device will not reply. Then the reader may start an inventory to a dedicated device with CBRA access

Observation 13	With the other assumptions in this section, there is no need to handle addressing problem for messages transmitted over DL-SCH before acquiring AS ID.

As a result, It is not obvious, without other assumptions, to determine if bit-alignment would lead to a more efficient MAC PDU encoding than octet-alignment.

Observation 14	Bit-alignment may cause excessive complexity and implementation effort on both device and network size due to the normal functioning of memory and existing implementations.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Study the pros and cons of merging “MAC” and “Device Control” functions into a single layer or in multiple layers.

Proposal 2	Study whether reader needs to be aware of the below information:

a.	AMF identifier for NAS Node selection function (NNSF) (FFS: if 5G AIoT Device Identifier is used instead as in legacy)

b.	Device population size

c.	Device capabilities (e.g.: device type and/or support for frequency shift)

d.	Inventory Session ID/transaction ID

Proposal 3	For a CBRA procedure, the reader provides in DL signalling at least pool, or set of resources, from which devices can randomly select their UL transmission resource.

Proposal 4	For a CFRA procedure, the reader must provide dedicated resources to a specific device.

Proposal 5	Study how to address the case of one UL grant not being enough to deliver the full SDU assuming in-order delivery and no specific mechanisms to handle failure of delivery and retransmission of some of the segments.

Proposal 6	RAN2 to study repetitions mechanism in the hypothesis that PHY layer repetitions or just one transmission is not enough to satisfy a success probability requirement at cell edge.

Proposal 7	RAN2 to consider multiple RACH attempts as a mechanism to implement repetitions.

Proposal 8	Study how the A-IoT device determines if a DL transmission in the shared downlink channel (PRDCH) is directed to itself and whether it should be MAC or PHY responsibility.

Proposal 9	DL-SCH (mapped to PRDCH) and UL-SCH (mapped to PDRCH) is the minimum set of transport channels provided by PHY layer.

Proposal 10	DL-SCH (mapped to PRDCH), BCH (mapped to PRDCH), UL-SCH (mapped to PDRCH) and RACH (mapped to PDRCH) logical channels are defined at least for modelling purposes (not necessarily with complexity impact).

Proposal 11	For UL-SCH after acquiring the AS ID, assume implicit addressing (no need for control signalling to distinguish between MAC PDUs from different devices).

Proposal 12	For DL-SCH study pros and cons of the following options regarding device addressing problem:

a.	Implicit addressing (reader/standard prevent transmission of other devices’ MAC PDUs at the same time). This cannot apply to CFRA.

b.	Control information included in PHY PDU, mechanism to determine and assign identifier up to upper layers.

c.	Control information included in MAC PDU (e.g., MAC CE), mechanism to determine and assign identifier up to upper layers.

d.	Any hybrid of the previous 3 options.

Proposal 13	Study how the AS ID is generated and distributed either during the CBRA procedure or generated by the reader.

Proposal 14	Study whether a requirement on how long the AS ID can be kept in memory is feasible and reliable.

Proposal 15	Include in the TR that bit-aligned MAC PDUs is out of the scope of the study.

# References

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA, Maastricht, Netherlands, March 18-21, 2024

RAN2#125bis Chair Notes, April 2024

RAN1#116bis R19 Ambient IoT Chair notes, April 2024

TS 22.369, “Service requirements for ambient power-enabled IoT”, 3GPP, March 2024

R3-24xxxx, “AIoT RAN CN interface details”, Ericsson, August 2024

## Annex.	 Current Agreements

RAN2 has made the below relevant agreements in RAN2#125-bis

Agreements (AI 8.2.2 Stage 2 General aspects)

- Unless explicitly stated all agreements apply to all device types and for both topologies.

- From RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2.

- RAN2 will support two use cases, “inventory” and “command”.  The definition, detailed wording is FFS

- Baseline procedure:
Step A: Based on the service request, the reader sends the Initial Trigger Message indicating device(s) that need to respond; Details FFS
Step B: Triggered device(s) performs the random access-like procedure, if needed; Details FFS
Step C: The device may perform the data communication with the reader as needed,: Details FFS

- We will study the support of both “inventory” and “command” in the same procedure.

- FFS if Initial Trigger Message can also include “command”.

- RAN2 will continue the study of ambient IoT assuming no support of AS security until SA3 provides further input.

Agreement (AI 8.2.3.2 User Plane)

- SDAP is not supported for UP protocol stack.

- PDCP layer is not needed.  FFS how to handle AS security (if needed pending SA3 dicsussion) and any other really needed functionalities.

- RLC layer is not needed.   FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size).  RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed.

- No HARQ and RLC AM

- FFS about the level of visibility required by the reader and what information is necessary for AS layer operations.

- RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL).  FFS how to handle the general QoS requirements from SA2

Agreement (AI 8.2.5 Random Access)

- RAN2 confirms slotted-ALOHA is the baseline for Ambient IoT random access

- We will study the support for access triggering for a single device, group of devices, or all devices.    RAN2 to discuss the contention-based and contention-free access procedures and detailed solutions.

- Random Access is triggered by the reader

- Reader provides the information that the device needs to respond to the random access trigger.  FFS what those parameters are

- Study the solution and benefits of both 2-step like random access procedure and 4-step like random access procedure.  FFS the details on each procedure and how we call it.

- Handling of contention resolution failure and access failure at the device will be studied in RAN2, including failure detection and re-access.  FFS details

- For the very first access message from the device to reader in random access an ID is included.  RAN2 to discuss whether a temporary identifier is included, or the permanent device ID is included (considering other WGs input as well).

Agreement for CP aspects

- RRC connection management is not supported.FFS how the resource configuration is provided to the device (if needed based on RAN1 progress)

- RRM L3 measurement reporting is not supported by Ambient IoT devices.

- RAN2 assumes, AIoT devices are not required to support ASN.1 encoding/decoding.

- Periodical System information and MIB are not supported by AIoT devices. This doesn’t preclude any RAN1 defined broadcast signals.

- RAN2 assumes that RRC layer is not necessary between the reader and the device.   RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

RAN2#126 agreements:

Agreements

- As baseline, the “inventory only” case is supported by the procedure:
Step A: A-IoT paging;
Step B: Device ID transmission (via Random Access or without using RA).  Details are FFS

- As baseline, the “inventory and command” case is supported by the procedure:
Step A: A-IoT paging;
Step B: Device ID transmission (via Random Access or without using RA).  Details are FFS 
Step C: reader to device data transmission (e.g. the R2D command), and
Step D: corresponding device to reader data transmission (e.g. the feedback).  FFS whether this is optional, pending other WG discussions.   
Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message.  This doesn’t mean that inventory and command are received by the reader at the same time from upper layer.

- From RAN2 point of view we will study “Command only” use case.  
FFS the options on how to support it :
Initial trigger message from the reader contains the command.  Final feasibility depends on SA2 and SA3 work/conclusions.

Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

Agreements on functionality

1	Multiple “AIoT logical channels” for upper layer data are not supported.   FFS if AIoT logical channel concept is used depending on final modeling issue.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

4	AS-layer (above PHY layer) RLC-like sretransmission/repetition is not supported.  This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective.   FFS how we handle segmentation case (if needed)

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. - What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

- The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

- “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.   “Msg2” usage/presence can be further discussed

Agreement

-	From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).