3GPP TSG-RAN WG2 Meeting #127	                                         R2-2406819

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source: 	Xiaomi

Title:  	Discussion on Inter-CU LTM

Agenda Item:	8.6.2	Inter-CU LTM

Document for:	Discussion and Decision

Introduction

A Revised WID on NR mobility enhancements Phase 4 [1] has been approved. In the WID, one of the objectives is to specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM), as shown below.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

In this contribution, we provide our understandings on inter-CU LTM.

Discussion

  - Inter-CU LTM when DC is not configured

Signalling flows

For stage-2 signaling flows and procedures, RAN2 reached the following agreements [2]:

Stage-2 signaling flows and procedures 1. The preparation of inter-CU LTM configuration is initiated by the source gNB-CU. 2. For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM. 3. The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells. 4. In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

For inter-CU LTM, more than one candidate gNB-CUs will involve in the mobility flow. Based on the overall procedure for Rel-18 intra-CU LTM [3], the signalling procedure for Rel-19 inter-CU LTM is illustrated in Figure 1 and includes the following three phases:

- Phase 1 (LTM Preparation): Based on the L3 RRM measurement report, the source gNB (source gNB-CU) decides candidate cells and initiates the inter-node interactions for inter-CU LTM preparation. After the interactions, the source gNB (source gNB-CU) provides the LTM configuration to UE with the RRC configuration of multiple candidate cells.

- Phase 2 (Initial LTM execution): As in Rel-18 intra-CU LTM, the UE sends the L1 measurement report to the source gNB (source gNB-DU). And after the reception of the cell switch command MAC CE, the UE switches to one candidate gNB (e.g., C-gNB 1). To support RACH-less LTM, DL and UL early synchronization with candidate cells can be performed before receiving the cell switch command.

- Phase 3 (Subsequent LTM execution): In subsequent LTM execution phase, the similar steps as steps 8~15 are performed. And the subsequent LTM is triggered by the current serving gNB, which is one candidate gNB (e.g., C-gNB 1).

According to the signalling procedure for Rel-19 inter-CU LTM illustrated in Figure 1, the signalling flows between UE and gNB can follow the Rel-18 intra-CU LTM.

Proposal 1: For Rel-19 inter-CU LTM, the signalling flows between UE and gNB can follow Rel-18 intra-CU LTM and the signalling procedure for inter-CU LTM illustrated in Figure 1 can be considered.

Figure 1: Signalling procedure for inter-CU LTM

Reference configuration

In Rel-18 LTM, to support the delta configuration, the reference configuration for LTM candidate configurations has been configured. And the LTM configuration (ltm-Config-r18) includes one separate reference configuration (ltm-ReferenceConfiguration-r18) for LTM candidates and add/mod/release lists (ltm-CandidateToReleaseList-r18 and ltm-CandidateToAddModList-r18) to configure the candidate configuration for each LTM candidate.

RAN2 has discussed how to configure the reference configuration for inter-CU LTM and two options have been discussed at the last meeting:

- Option 1: Single reference configuration as Rel-18 LTM

- Option 2: Multiple reference configuration, e.g., a reference configuration per CU

One single reference configuration is the simple method as the Rel-18 LTM. But some opponents think the coordination between multiple CUs may be needed for the option 1 and each CU may have different capabilities or default configurations. In effect, option 2 may not be better in the reduction of CU-CU coordinations. Multiple reference configurations associated with candidate CUs may also need to be sent to the source CU. Moreover, in Rel-18, the LTM candidate configuration can be the complete configuration. If some candidate CUs might not want to generate the candidate configuration based on the single reference configuration, the candidate configuration can be configurated as a complete configuration with the complete flag (ltm-ConfigComplete-r18).

Proposal 2: A single reference configuration is configured for inter-CU LTM and the Rel-18 reference configuration (ltm-ReferenceConfiguration-r18) for intra-CU LTM is reused to configure it for inter-CU case.

Proposal 3: As in Rel-18, the candidate configuration of inter-CU LTM can also be a complete candidate configuration and the complete configuration flag (ltm-ConfigComplete-r18) for Rel-18 case can be reused.

Early sync phase

RAN2 has discussed the early sync phase in previous meetings, and reached the following agreements [2][4]:

Agreements-RAN2#125bis: - Early DL and UL sync is also supported for inter-CU LTM. Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval. - PDCCH ordered early RACH is supported for inter-CU LTM. - For early TA acquisition, Rel-18 option is baseline. FFS for RAR based option. Agreements-RAN2#126: - RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

To support Early UL sync, there are two options for intra-CU LTM, as followings:

- Early TA acquisition

- UE based TA measurement

Early TA acquisition has been supported for inter-CU LTM. Whether to support UE based TA measurement for inter-CU cases has been discussed. To support UE based TA measurement may need to meet some synchronization requirement. Although, RAN4 already defined UE requirement for UE based TA measurement for intra-CU LTM, whether the same requirements are applied for Rel-19 inter-CU LTM needs to be confirmed by RAN4. And from RAN2 perspective, supporting UE based TA measurement is beneficial for inter-CU LTM, which can reduce the cell switch interruption when the UE doesn’t perform early TA acquisition. Hence, from RAN2 perspective, UE-based TA measurement for inter-CU LTM could be considered, but it should be finally decided by RAN4.

Proposal 4: From RAN2 perspective, supporting UE-based TA measurement is beneficial for inter-CU LTM, but it should be finally decided by RAN4.

In previous meeting, RAN2 thinks “Early DL sync using CSI-RS should be considered, pending RAN1 approval.” In the WID [1], RAN1 will specify support for CSI-RS measurements for LTM (Objective 2), but RAN1 is not included in the scope of inter-CU LTM (Objective 1) . Hence, RAN2 needs to send an LS to RAN1 to ask whether the Early DL sync using CSI-RS can be supported and trigger the discussion in RAN1.

Proposal 5: Send an LS to RAN1 to ask whether the Early DL sync using CSI-RS can be supported.

Security issue

To support subsequent LTM for the inter-CU LTM, how to handle the security key upon cell switch needs to be discussed as captured in the WID [1]. In the previous meeting, RAN2 has sent an LS to SA3 including many potential solutions and asked SA3 the feasibility of these potential solutions for the security key handling of inter-CU LTM.

The LS form SA3 [5] indicates that SA3 agreed a SA3 WID [6] to work on this topic and will provide feedback to RAN2 as SA3 makes any progress. Hence, RAN2 can wait for SA3’s progress and then study the RAN2’s impact based on the reply of SA3.

Proposal 6: For the solution of the security key handling in inter-CU LTM, RAN2 can wait for the progress of SA3.

L2 handling

In previous meetings [2] [4], RAN2 agreed that upon inter-CU LTM execution, UE performs MAC reset, RLC re-establishment, PDCP re-establishment and security key update and supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

According to the progress in Rel-19 inter-CU LTM and Rel-18 intra-CU LTM, L2 handling for intra-CU LTM and inter-CU LTM, are in the table below:

| Cases L2 Handling | Intra-CU LTM |  | Inter-CU LTM |
| --- | --- | --- | --- |
|  | Intra-DU LTM | Intra-CU inter-DU LTM | - |
| MAC reset | Yes | Yes | Yes |
| RLC re-establishment | No | Yes | Yes |
| PDCP handling | No | PDCP data recovery | PDCP re-establishment |
| Security key update | No | No | Yes |

Therefore, for L2 handling, when UE is configured with mixture of intra-CU and inter-CU LTM, UE needs to know the types of LTM. And RAN2 has discussed whether the new cell set ID can be introduced to indicate the inter-CU and intra-CU LTM for the mixture of intra-CU and inter-CU case and RAN2 also discussed whether the Rel-18 NoResetID can be reused when the security key update is not required.

For the first issue, how to differ intra-CU LTM or inter-CU LTM executed by UE can be indicated by whether the security key update is required. And if the security key update is required for the LTM execution, UE shall perform MAC reset, RLC re-establishment, and PDCP re-establishment. And how to indicate the security key update is required for the LTM execution can be discussed in SA3 WID [6]. As in P6, we can wait for SA3’s progress.

Proposal 7: If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. For the issue how to indicate that the security key update is required, we can wait for SA3’s progress on the security of inter-CU LTM.

For the second issue, when the UE performs intra-CU LTM and the security key update is not required, the Rel-18 mechanism can be reused. And whether to perform L2 reset is based on the NoResetID. i.e., when performing the LTM cell switch procedure, if the value of field ltm-NoResetID of the target cell is not equal to the value of ltm-ServingCellNoResetID of the serving cell, the UE performs PDCP data recovery and RLC re-establishment.

Proposal 8: If the security key update is not required, reuse the Rel-18 L2 handling mechanism and the Rel-18 NoResetID is reused to determine whether to perform RLC re-establishment and PDCP data recovery when the UE performs the LTM cell switch procedure.

Inter-CU LTM and L3 Mobility

In the previous meeting, RAN2 has agreed the mixture of intra-CU and inter-CU LTM is supported and Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch [2].

In Rel-18 intra-CU LTM, the following items has been supported for the coexistence of intra-CU LTM and L3 Mobility：

- For L3 HO

  - L3 handover may happen while LTM is configured/evaluated/used.

  - During network triggered L3 HO, the UE does not autonomously release the LTM configuration.

  - The RRCReconfiguration message to execute an L3 HO procedure may reconfigure (setup, release) the LTM configuration.

- For CHO

  - While configured with LTM configurations, the UE can also be configured with CHO. And before any CHO execution condition is satisfied, upon reception of LTM cell switch command MAC CE, the UE executes the LTM cell switch procedure.

For the inter-CU case, whether above items are also applicable or not can be discussed. One goal of the subsequent LTM mobility procedures is aiming to reduce signalling overhead. Extending above items to the inter-CU case can avoid RRC configuration after mobility procedure and UE can continuously perform the LTM cell switch after the L3 mobility. In addition, to support these items can also be beneficial to the flexibility of configuration.

Proposal 9: L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured, and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- Before any CHO execution condition is satisfied, upon reception of inter-CU LTM cell switch command MAC CE, the UE executes the inter-CU LTM cell switch procedure.

  - Inter-CU LTM when DC is configured

In the WID, two secondary priority cases need to be discussed for the Inter-CU LTM when DC is configured, as show below:

- When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

Inter-CU LTM where CU is acting as SN and MN is unchanged

In the legacy inter-SN CPC, both MN initiated inter-SN CPC and SN initiated inter-SN CPC are supported. As in inter-SN CPC, for the cases inter-SN LTM without MN change, two sub-cases can be considered:

- Sub-Case 1: MN initiated inter-SN LTM

- Sub-Case 2: SN initiated inter-SN LTM

For the MN initiated inter-SN LTM, the MN-CU initiates the preparation of inter-SN LTM and the MN-DU triggers the inter-SN LTM via the Cell Switch Command MAC CE.

For the SN initiated inter-SN LTM, the source SN-CU initiates the preparation of inter-SN LTM and the source SN-DU triggers the inter-SN LTM via the Cell Switch Command MAC CE.

Proposal 10: For inter-SN LTM without MN change, two sub-cases can be considered:

- Sub-Case 1: MN initiated inter-SN LTM

  - the MN-CU initiates the preparation of inter-SN LTM and the MN-DU triggers the inter-SN LTM

- Sub-Case 2: SN initiated inter-SN LTM

  - the source SN-CU initiates the preparation of inter-SN LTM and the source SN-DU triggers the inter-SN LTM

As in the legacy mobility for SN change, the candidate/target configuration is provided to UE in MN format. For inter-SN LTM, the RRC reconfiguration message containing the inter-SN candidate configuration provided to the UE is in MN format.

Proposal 11: For inter-SN LTM, the RRC reconfiguration message containing the inter-SN candidate configuration provided to the UE is in MN format.

For inter-CU LTM in non-DC case, RAN2 has agreed the mixture of intra-CU and inter-CU LTM is supported. For inter-SN LTM, RAN2 can discuss whether the mixture of intra-SN and inter-SN LTM can be supported.

Proposal 12: For inter-SN LTM without MN change, RAN2 to discuss whether to support the mixture of intra-SN and inter-SN LTM.

Inter-CU LTM where CU is acting as MN and SN is unchanged or SN is released

For the second cases, there are two sub-cases:

- Sub-Case 1: Inter-CU LTM where CU is acting as MN and SN is unchanged

- Sub-Case 2: Inter-CU LTM where CU is acting as MN and SN is released

Both sub-case 1 and 2 have been captured in the WID. Firstly, RAN2 can discuss whether the sub-case 1 and sub-case 2 can be configured to UE at the same time.

Proposal 13: RAN2 to discuss whether both inter-MN LTM with SN un-change and inter-MN LTM with SN release can be configured to UE simultaneously.

Conclusions

According to the analysis given above, we have the following proposals:

2.1	Inter-CU LTM when DC is not configured

Signalling flows

Proposal 1: For Rel-19 inter-CU LTM, the signalling flows between UE and gNB can follow Rel-18 intra-CU LTM and the signalling procedure for inter-CU LTM illustrated in Figure 1 can be considered.

Reference configuration

Proposal 2: A single reference configuration is configured for inter-CU LTM and the Rel-18 reference configuration (ltm-ReferenceConfiguration-r18) for intra-CU LTM is reused to configure it for inter-CU case.

Proposal 3: As in Rel-18, the candidate configuration of inter-CU LTM can also be a complete candidate configuration and the complete configuration flag (ltm-ConfigComplete-r18) for Rel-18 case can be reused.

Early sync phase

Proposal 4: From RAN2 perspective, supporting UE-based TA measurement is beneficial for inter-CU LTM, but it should be finally decided by RAN4.

Proposal 5: Send an LS to RAN1 to ask whether the Early DL sync using CSI-RS can be supported.

Security issue

Proposal 6: For the solution of the security key handling in inter-CU LTM, RAN2 can wait for the progress of SA3.

L2 handling

Proposal 7: If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. For the issue how to indicate that the security key update is required, we can wait for SA3’s progress on the security of inter-CU LTM.

Proposal 8: If the security key update is not required, reuse the Rel-18 L2 handling mechanism and the Rel-18 NoResetID is reused to determine whether to perform RLC re-establishment and PDCP data recovery when the UE performs the LTM cell switch procedure.

Inter-CU LTM and L3 Mobility

Proposal 9: L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured, and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- Before any CHO execution condition is satisfied, upon reception of inter-CU LTM cell switch command MAC CE, the UE executes the inter-CU LTM cell switch procedure.

2.2	Inter-CU LTM when DC is configured

Inter-CU LTM where CU is acting as SN and MN is unchanged

Proposal 10: For inter-SN LTM without MN change, two sub-cases can be considered:

- Sub-Case 1: MN initiated inter-SN LTM

  - the MN-CU initiates the preparation of inter-SN LTM and the MN-DU triggers the inter-SN LTM

- Sub-Case 2: SN initiated inter-SN LTM

  - the source SN-CU initiates the preparation of inter-SN LTM and the source SN-DU triggers the inter-SN LTM

Proposal 11: For inter-SN LTM, the RRC reconfiguration message containing the inter-SN candidate configuration provided to the UE is in MN format.

Proposal 12: For inter-SN LTM without MN change, RAN2 to discuss whether to support the mixture of intra-SN and inter-SN LTM.

Inter-CU LTM where CU is acting as MN and SN is unchanged or SN is released

Proposal 13: RAN2 to discuss whether both inter-MN LTM with SN un-change and inter-MN LTM with SN release can be configured to UE simultaneously.

Reference

[1] RP-241515, “Revised WID: NR mobility enhancements Phase 4”, Apple Inc, China Telecom

[2] Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

[3] 3GPP TS 38.300, “NR and NG-RAN Overall Description”

[4] Report of 3GPP TSG RAN WG2 meeting #125bis, Changsha, China

[5] S3-242400, “Reply LS to RAN2 on security handling for inter-CU LTM in non-DC cases”

[6] S3-242401, “New WID on security aspects of NR mobility enhancement”