3GPP TSG-RAN WG2 Meeting #127														                  R2-2406821

Masstricht, Netherlands, Aug 19th – 23rd, 2024                                      Revision of R2-2405132

Agenda item:	8.9.2

Source:	MediaTek Inc.

Title:	RAN2 impact on S&F mode

Document for:	Discussion and Decision

# Introduction

Rel-19 IoT NTN work item was revised in RAN#104 [1], one of the objectives is to support S&F operation, as below:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

A study report, 23.700-29 [2], on the integration of satellite components in the 5G architecture Phase 3 has been released by 3GPP SA2. The conclusion for Key Issue #2: Support of Store and Forward Satellite operation has been made. Two types of system architecture can be adopted by operation deployment: a full CN onboard the satellite or a Split MME architecture.

In this contribution, we share our view on S&F mode, incorporating input from SA2.

# Discussion

S&F indication

In RAN2#125bis, it was agreed that IoT NTN network should be able to inform UE whether S&F Satellite operation is applied. Furthermore, in RAN2#126, it was agreed that the S&F indication can be provide by SIB, and no NAS indication is needed.

RAN2#125bis Agreement:

- An IoT NTN network shall be able to inform UE(s) whether S&F Satellite operation is applied, either via NAS or AS (wait for SA2 progress on this)

RAN2#126 Agreement:

- S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

Regarding the details of the S&F indication, we believe it is better to indicate whether the cell supports the S&F operation, rather than the current feeder link availability status. For a Rel-19 UE, this allows the UE to decide whether to prefer a S&F operation-enabled cell during cell selection. The preference can be made in AS or NAS, depending on the discussion progress in SA2/CT1.

Proposal 1: The S&F indication specifies whether the cell supports the S&F operation.

In addition to the S&F indication, the current feeder link status can also be broadcasted. When the feeder link is available, the real-time transmission is still possible for a cell that has S&F mode enabled. With this information, the UE will be able to categorize the cell as the second priority after the cell without S&F mode during cell selection. Alternatively, the UE will have the opportunity to initiate a real-time transaction, such as establishing a TCP connection at the application layer. This information could also be notified to the users, allowing them to know when it is possible to make real-time communications, such as voice calls.

It also can be beneficial in many aspects if UE knows when the cell will have or lose the feeder link.

Proposal 2: In addition to S&F operation indication, the current feeder link status can be broadcast in a S&F operation-enabled cell via system information.

Proposal 3: The times when the feeder link will become available or unavailable can be broadcast in an S&F operation-enabled cell via system information.

S&F barring bit

SA2 has discussed whether the legacy UEs can be supported in S&F mode. As observed in the study report, for the split MME architecture, the UE needs to indicate support for S&F mode to the MME following existing NAS capability. For the full CN onboard the satellite, the UE needs to be aware that a satellite supports S&F mode. Therefore, we believe SA2 does not support legacy UEs in S&F mode.

However, there could be an exception. When the feeder link is available, the cell functions no differently than a non-S&F mode cell. In this case, allowing legacy UE to access the cell can be a lifesaver for those legacy UEs have only coverage from an S&F mode cell.

Therefore, a (legacy) UE does not support S&F mode should still be able to access an S&F mode-enabled cell when feeder link is available.

Proposal 4: Legacy UEs (Rel-17 and Rel-18) and Rel-19 UEs that do not support S&F mode can access the cell in S&F mode when the feeder link is available.

In RAN2#126, we reached the following agreement:

RAN2#126 agreement

- RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

To prevent legacy UEs from accessing the S&F mode cell, the legacy bits in SIB1 cellBarred and cellBarred-NTN can be used. Therefore, to allow legacy UEs to access the S&F mode cell when the feeder link is available, cellBarred-NTN can be temporarily open.

Proposal 5: A cell operating in S&F mode should set the cellBarred-NTN-r17 to barred when the feeder link is not available and set the cellBarred-NTN-r17 to notbarred when the feeder link becomes available.

RAN2 has discussed introducing a new barring bit for Rel-19 UEs. However, with the agreed S&F mode indication bit, Rel-19 UEs can already be barred if needed. If S&F mode indication is set to True, Rel-19 UEs that support S&F mode must ignore the cellBarred-NTN to access the cell. If the network wants to bar Rel-19 UEs from accessing the cell, it simply does not set the S&F mode bit. In this case Rel-19 UE will subject to the cellBarred-NTN bit, just like legacy UEs.

Proposal 6: Rel-19 UEs that support S&F mode should ignore the cellBarred-NTN-r17 if the S&F indication in the SIB is set to true.

Cell selection with satellite ID

In the study report [2], for the split MME architecture, a wait time and a list of satellite IDs will be provided to UE to re-attempt the Attach/TAU procedure.

23.700-29 v1.0.0 clause 8.2 b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB.

For the full CN onboard the satellite, a S&F monitoring list of satellites IDs will be optionally provided to the UE during the attach/TAU. The UE uses the satellites in the S&F monitoring list for MO/MT data/signalling with the CN. Specifically, the S&F monitoring list may assist the UE in retrieving MT data.

23.700-29 v1.0.0 clause 8.2 - Optionally the MME provides the UE with a S&F monitoring list of satellites IDs, during attach/TAU. The UE uses the satellites in the S&F monitoring list for MO/MT data/signalling with the CN. The S&F monitoring list can be determination by the CN. How network determines the S&F monitoring list is outside the scope of 3GPP in this release of specification. NOTE 8: The S&F monitoring list may assist the UE in retrieving MT data.

To support these functions, NAS can indicate a list of satellite IDs to AS during the PLMN selection and cell selection. Currently, SIB31, SIB32, and SIB33 contain satellite IDs. SIB32 includes a frequency list for a given satellite ID, and the corresponding frequency of the satellite ID in SIB33 can be found in SIB3 or SIB5. The frequency of satellite in SIB31 is the frequency of the serving cell. The UE can determine the frequency of the cell from the satellite ID. When NAS indicates the satellite ID to AS, the UE can first find the corresponding cell frequency from the SIBs and search the cell on that frequency. If a cell is found on that frequency, the UE may check the satellite ID to see if it matches the one requested by NAS.

Alternatively, NAS can request the UE to report the satellite ID of the found cell during the PLMN selection and cell selection.

The wait time can be used in NAS to trigger the PLMN selection or cell selection.

Proposal 7: NAS can indicate a list of satellite IDs to AS during the PLMN selection and cell selection. The UE AS will attempt to camp on the cell provided by satellite IDs.

Proposal 8: NAS can request UE to report the satellite ID of found cell during the PLMN selection and cell selection.

Proposal 9: Satellite IDs in SIB31, SIB32, and SIB33 can be reused for providing the mapping between satellite ID and frequency information.

To support cell search for a specific satellite when the UE is not camping on a cell, the UE may store all the known satellite ID and frequency mapping information for each PLMN.

Proposal 10: The UE stores satellite ID and frequency mapping information for each PLMN.

For the full CN onboard the satellite, an S&F monitoring list of satellites IDs can assist the UE in retrieving MT data. To retrieve the MT data, the UE will autonomously initiate a RRC connection establishment. This RRC connection establishment is not for MO data/signalling, nor is it for MT data/signalling, as there is no paging message and no need to initiate any procedure with the core network. Therefore, a new connection establishment cause is needed so that the satellite can recognize that this connection is solely for retrieving the stored DL data to the UE. Network would only forward the stored DL data to UE, if any, and there will be no attempt to access the feeder link.

Proposal 11: Introduce a new connection establishment cause: “retrieve MT data”, for UE-initiated connection establishment to retrieve MT data.

# Conclusion

S&F indication

Proposal 1: The S&F indication specifies whether the cell supports the S&F operation.

Proposal 2: In addition to S&F operation indication, the current feeder link status can be broadcast in a S&F operation-enabled cell via system information.

Proposal 3: The times when the feeder link will become available or unavailable can be broadcast in an S&F operation-enabled cell via system information.

S&F barring bit

Proposal 4: Legacy UEs (Rel-17 and Rel-18) and Rel-19 UEs that do not support S&F mode can access the cell in S&F mode when the feeder link is available.

Proposal 5: A cell operating in S&F mode should set the cellBarred-NTN-r17 to barred when the feeder link is not available and set the cellBarred-NTN-r17 to notbarred when the feeder link becomes available.

Proposal 6: Rel-19 UEs that support S&F mode should ignore the cellBarred-NTN-r17 if the S&F indication in the SIB is set to true.

Cell selection with satellite ID

Proposal 7: NAS can indicate a list of satellite IDs to AS during the PLMN selection and cell selection. The UE AS will attempt to camp on the cell provided by satellite IDs.

Proposal 8: NAS can request UE to report the satellite ID of found cell during the PLMN selection and cell selection.

Proposal 9: Satellite IDs in SIB31, SIB32, and SIB33 can be reused for providing the mapping between satellite ID and frequency information.

Proposal 10: The UE stores satellite ID and frequency mapping information for each PLMN.

Proposal 11: Introduce a new connection establishment cause: “retrieve MT data”, for UE-initiated connection establishment to retrieve MT data.

# References

RP-241624, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)

TR 23.700-29 v.1.0.0. Study on integration of satellite components in the 5G architecture Phase 3