3GPP TSG-RAN WG2 Meeting #127	R2-2406822

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.11.2

Source:	Nokia

Title:	Random Access Aspect of SBFD

WID/SID:	NR_duplex_evo-Core - Release 19

Document for:	Discussion and Decision

# Introduction

In RAN#102 meeting, a Work Item (WI) on Evolution of NR duplex operation: Subband full duplex (SBFD) was approved [1]. The work item description has been updated in RAN#104 meeting and contain the following objectives [RP-241614]:

For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4] - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling

# SBFD Random Access Operation

For initial access, the UE selects the index of the preferred SSB index and decodes the associated PBCH for MIB, and obtains relevant information for decoding the SIB1, from where it obtains the RACH configuration for sending Msg1. The RACH configuration contains, among others, the PRACH configuration index, the frequency domain multiplexing of the RACH occasions (ROs), the frequency offset of the ROs and information about the power ramp up during the RACH procedure. For UEs in RRC_CONNECTED, dedicated RACH configuration, including CFRA, can be indicated by the network. If the CFRA configuration is missing, UEs in RRC_CONNECTED use the common RACH configuration for CBRA.

To enable support of initial access in SBFD, i.e., in UL subband, RAN1 has been discussing the above aspects of random access procedures in SBFD since RAN1#116 and has made agreements as listed in the Appendix 1. In fact, RAN1 observed that support of initial access in SBFD provides at least one or more following benefits: reduced random access latency, reduced PRACH collision probability or allowing more contiguous frequency resources for PUSCH in UL slots, improved coverage of PRACH with sparse UL resources , and increased cell range of PRACH with sparse UL resources. As mentioned in the section 1, WI was updated in RAN#104 [RP-241614] to specify SBFD operation to support random access in SBFD symbols by UEs in RRC_IDLE/INACTIVE mode , along with RRC_CONNECTED mode in RAN1 and RAN2.

In section 3, we present the various aspects of random access in RRC Connected mode, and in RRC Idle/Inactive mode, and the conclusion is provided in the section 4.

# Various Aspects of SBFD random access

In this section, we discuss the RAN2 aspects of the SBFD random access procedure for RRC_CONNECTED and IDLE/INACTIVE operations.  RAN1 agreements on SBFD random access is listed in Appendix 1.

## PRACH resource configuration for RRC_CONNECTED UEs

To enable random access for SBFD symbols in SBFD-aware UEs, the gNB must set up a new set of PRACH resources which includes RO configuration, SSB-RO mapping, the random access configuration table, and PRACH repetition.

In RAN1#117, following agreements are made on the PRACH resource configuration:

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

For option 1, it seems straightforward to reinterpret that for ROs within SBFD symbols, msg1-FrequencyStart indicates the offset of the first RO with respect to the start of the UL subband, and FDMed ROs which fall outside the UL subband after applying offset are considered as invalid ROs for SBFD UEs.

Proposal 1: For option 1,  for ROs within SBFD symbols, msg1-FrequencyStart indicates the offset of the first RO with respect to the start of the UL subband, and FDMed ROs that fall outside the UL subband after applying offset are considered invalid ROs for SBFD UEs.

On the FFS part of option 2, since Option 2 necessitates support for two RACH configurations, it introduces some overhead, especially when redundant information is present in these configurations. Hence to reduce the signaling overhead, if the legacy ROs and the additional ROs have the same value for one of the RACH configuration parameters it would not be needed to include those parameters in the additional RACH configuration. In such a scenario, inclusion in the legacy RACH configuration would be enough.

Proposal 2: For Option 2, in case the legacy ROs and the additional ROs have the same value for one of the RACH configuration parameters, only the legacy RACH configuration will be configured with this parameter.

## PRACH power control for RRC_CONNECTED UEs

A UE calculates the PRACH preamble transmit power through an open-loop power control method, which relies on a downlink path loss measurement. The MAC layer performs the initial part of this calculation using the following expression:

PREAMBLE_RECEIVED TARRGET_POWER = preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAM PING COUNTER -1) x preamblePowerRampingStep

preambleReceivedTargetPower is provided to the UE within the RACH-ConfigCommon parameter structure, DEL TA_PREAMBLE represents a power offset which adjusts the PRACH Preamble received power requirement according to the PRACH sequence duration (Its value is extracted from a look-up table standardized by 3GPP within TS 38.321). A UE maintains two counters when transmitting a series of PRACH preambles: PREAMBLE_ TRANSMISSION_ COUNTER and PREAMBLE_POWER_RAMPING_COUNTER. The former is used to limit the total number of PRACH preamble transmissions within a single Random Access procedure, whereas the latter is used to control the ramping of the PRACH Preamble transmit power (visible within the equation shown above). preamblePowerRampingStep can be configured with either a 'normal' value or a 'high priority' value. The 'normal' value is provided to the UE within the RACH-ConfigCommon parameter structure. The 'high priority' value can be used to override the 'normal' value when it is necessary to minimize latency, i.e. the 'high priority' step size can be configured with a higher value to allow more rapid

power ramping.  Once, the MAC layer has calculated PREAMBLE RECEIVED TARGET POWER, the result is passed to the PHY which then calculates the PRACH preamble transmit power.

Considering that SBFD slots can be significantly different than UL slots in terms of frequency resource availability, number of available time slots, SINR, CLI conditions, etc., for RACH Configuration option 2, it is reasonable to specify separate transmit power and power ramp-up parameters for Msg1 transmissions in SBFD-ROs compared to legacy ROs.

Proposal 3: For RACH Configuration option 2,  specify separate transmit power and power ramp-up parameters for Msg1 transmissions in SBFD-ROs compared to legacy ROs.  For Option 1, there is no need to have a separate power control for SBFD-ROs.

## Random access for IDLE/INACTIVE operation

In RAN1#117, following agreement was made on the SBFD random access for RRC_IDLE/INACTIVE mode operation.

Agreement Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

When comparing the random access operations for UEs in RRC_CONNECTED mode and those in RRC_IDLE/INACTIVE mode, the primary difference is that time-frequency resources and other PRACH parameters for UEs in RRC_IDLE/INACTIVE mode are provided in the SIB, and via dedicated RRC message to the RRC_CONNECTED UEs.  Apart from this, other specification impacts are similar. To support random access for UEs in RRC_IDLE/INACTIVE mode, a unified solution can be implemented, encompassing PRACH resource configuration, PRACH resource selection, SSB-RO mapping, and related aspects.

Proposal 4: To support random access for UEs in RRC_IDLE/INACTIVE mode and RRC_CONNECTED mode, a unified solution can be implemented, encompassing PRACH resource configuration, PRACH resource selection, SSB-RO mapping, and related aspects.

## RACH Occasion selection and prioritization

For both RACH configuration Option 1 and Option 2 agreed in RAN1, SBFD-aware UEs will have the possibility of transmitting RACH on legacy ROs and SBFD ROs, while legacy UEs would only be capable of using legacy ROs. For load balancing purposes, it is beneficial that SBFD-aware UEs prioritize as much as possible RACH transmissions on SBFD ROs and avoid consuming the quite-limited capacity of legacy RACH occasions. Considering that UE-to-UE CLI may only be a problem in SBFD ROs but not in legacy ROs, and may be especially critical in cell-edge areas (where the UL transmission power is high, while the DL received signal power is low), the selection between legacy ROs and SBFD ROs may also be based UE’s coverage conditions. For instance, a cell-centre UEs transmit RACH on SBFD ROs, while a cell-edge SBFD-aware UE uses ROs belonging to the legacy RACH configuration.

Proposal 5: From the SBFD-aware UE perspective and to have better load balancing across different RACH resources, (new) SBFD ROs shall have higher priority than legacy (ROs). The priority between SBFD and legacy ROs can be based on the UE’s coverage conditions, e.g. SBFD ROs are only used by UEs in cell-centre conditions.

# Conclusion

Proposal 1: For option 1,  for ROs within SBFD symbols, msg1-FrequencyStart indicates the offset of the first RO with respect to the start of the UL subband, and FDMed ROs which falls outside the UL subband after applying offset  is considered as invalids ROs for SBFD UEs.

Proposal 2 : For Option 2, in case the legacy ROs and the additional ROs have the same value for one of the RACH configuration parameters, only the legacy RACH configuration will be configured with this parameter.

Proposal 3: For RACH Configuration option 2,  specify separate transmit power and power ramp up parameters for Msg1 transmissions in SBFD-ROs compared to legacy ROs.  For Option 1, there is no need to have a separate power control for SBFD-ROs.

Proposal 4: To support random access for UEs in RRC_IDLE/INACTIVE mode and RRC_CONNECTED mode, a unified solution can be implemented, encompassing PRACH resource configuration, PRACH resource selection, SSB-RO mapping, and related aspects.

Proposal 5: From the SBFD-aware UE perspective and in order to have better load balancing across different RACH resources, (new) SBFD ROs shall have higher priority than legacy (ROs). The priority between SBFD and legacy ROs can be based on the UE’s coverage conditions, e.g. SBFD ROs are only used by UEs in cell-centre conditions.

# Appendix 1

- RAN1#116 agreements on SBFD random access

R1-2401676	Summary#1 on SBFD random access operation	Moderator (CMCC)

Working assumption:

For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols.

Conclusion

No new PRACH format is introduced in Rel-19 duplex WI.

Agreement

For random access operation for SBFD-aware UEs in RRC CONNECTED state, at least consider the following options:

- Option 1: Use one single RACH configuration with possible enhancement

- The ROs within UL subband in SBFD symbols can be valid for SBFD-aware UE

- FFS: Further details

- Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration

- The ROs within UL subband in SBFD symbols configured by the additional RACH configuration can be valid for SBFD-aware UE

- FFS: Further details

Agreement

For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols.

- FFS Type-2 random access procedure (2-step RACH)

R1-2401677	Summary#2 on SBFD random access operation	Moderator (CMCC)

Conclusion

If PRACH is allowed in SBFD symbols for SBFD-aware UEs in RRC_IDLE/INACTIVE mode, RAN1 observed the following:

- The benefits include at least one or more of the following:

- reduced random access latency

- reduced PRACH collision probability or allowing more contiguous frequency resources for PUSCH in UL slots

- improved coverage of PRACH with sparse UL resources

- increased cell range of PRACH with sparse UL resources

- PRACH transmissions in UL subband in SBFD symbols may cause UE-to-UE CLI (similar to the case of RRC connected mode UEs) for some deployment scenarios. Initial studies based on two companies’ evaluation results, the DL performance degradation due to UE-to-UE CLI caused by PRACH transmission in SBFD symbols is not significant for indoor office scenario and Urban Macro scenario.

For future meetings

Companies to consider whether the existing random access configuration tables for unpaired spectrum (i.e., Table 6.3.3.2-3 for FR1 and Table 6.3.3.2-4 for FR2) need to be enhanced.

Agreement

For SBFD aware UEs in RRC CONNECTED state, at least PRACH without repetition is supported in SBFD symbols.

- FFS PRACH repetition in SBFD symbols.

- FFS PRACH repetition across SBFD symbols and non-SBFDs symbols.

R1-2401678	Summary#3 on SBFD random access operation	Moderator (CMCC)

Agreement

For SBFD-aware UEs in RRC CONNECTED state, further study the following two options:

- Option 1: a valid RO can only be on SBFD symbols or on non-SBFD symbols

- a configured RO across SBFD and non-SBFD symbols in the same slot or across slots is invalid

- Option 2: a valid RO can be across SBFD and non-SBFD symbols in the same slot or across slots

RAN1 to leverage the study in Rel-18 as baseline.

Agreement

For SBFD-aware UEs in RRC CONNECTED state, at least further study whether/how to enable Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols taking into account the following aspects:

- Msg2[/Msg4 PDSCH] reception in DL subband(s)

- Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] frequency resource allocation and frequency hopping

- Msg3 repetition

- Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] power control

- FFS whether/how gNB to identify whether a UE is SBFD aware UE or non-SBFD aware UE

Note: Strive to make progress in accordance to the discussion in AI 9.3.1.

For future meetings:

In RAN1#116bis meeting, at least the following issues will be discussed:

- Whether to support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode

- Details of the two options for configuring ROs for SBFD aware UEs in RRC CONNECTED mode, including RO validation rules, SSB-RO mapping rules, whether/how to allow SBFD aware UE and non-SBFD aware UE to use different PRACH preamble formats.

- Whether/how to support separate PRACH power control parameters configuration in SBFD symbols and non-SBFD symbols.

- Whether/how to enhance the existing random access configuration tables for unpaired spectrum

- Whether/how to support PRACH repetition

- RAN1#116bis agreements on random access:

R1-2403468	Summary#1 on SBFD random access operation	Moderator (CMCC)

Agreement

Confirm the working assumption:

Working assumption:

For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols.

Agreement

For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, consider the following alternatives to derive the time and frequency resources of the configured ROs in SBFD symbols.

- Alt 1-1: only based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon).

- FFS the details

- FFS: Alt 1-2: based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon) and newly introduced parameter(s).

Agreement

For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, no separate prach-ConfigurationIndex to be configured in this option.

Agreement

For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, use existing random access configurations tables for unpaired spectrum (i.e., Table 6.3.3.2-3 for FR1 and Table 6.3.3.2-4 for FR2 in TS38.211).

R1-2403469	Summary#2 on SBFD random access operation	Moderator (CMCC)

Working Assumption

For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported.

- For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc.

- For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc.

UE is not required to support both options.

Agreement

For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, down-select (in RAN1#117) from the following alternatives:

- Alt 2-3:

- The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs.

- FFS: The case where the additional-ROs partially overlap with non-SBFD symbols

- Alt 2-4:

- The additional-ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs.

For the legacy-ROs configured by legacy RACH configuration, the legacy RO validation rules and the legacy SSB-RO mapping rules are followed for SBFD aware UEs.

Proposal

Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

- Supported by 24 companies: New H3C, CMCC, ZTE, IDC, Xiaomi, HW/HiSi, Samsung, Nokia, NEC, Google, TCL, Sharp, Wilus, LGE, Fujitsu, ETRI, Sony, QC, Lenovo, SKT, MTK, CATT, Panasonic

- Objected by Ericsson

R1-2403437	SBFD Random access operation	Ericsson

R1-2403470	Summary#3 on SBFD random access operation	Moderator (CMCC)

Agreement

For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration),

- For the legacy-ROs, including the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any), the legacy SSB-RO mapping is followed.

- For the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, separate SSB-RO mapping will be used

Agreement

For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration,

- For FR2, consider from the following alternatives:

- Alt 1: use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211)

- FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols.

- Alt 3: Introduce new entries on top of existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211)

- For FR1, consider from the following alternatives:

- Alt 1: Use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211)

- FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols.

- Alt 2: Use existing random access configurations table for paired spectrum/supplementary uplink (i.e., Table 6.3.3.2-2 in TS38.211)

- Alt 3: Introduce new entries on top of existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211)

Agreement

For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state,

- no enhancements for the RO validation rule for the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any).

- FFS: the ROs in non-SBFD symbols that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs.

- FFS: It’s up to network configuration to ensure the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, which are valid for non-SBFD aware UEs based on legacy RO validation rule, are also valid for SBFD aware UEs (i.e., the configured ROs in SBFD symbols, if configured as flexible by tdd-UL-DL-ConfigurationCommon, are within the UL usable PRBs)

- the RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon is valid if at least:

- Time and frequency resource of the RO are fully within UL usable PRBs, and not overlapped with SSB

- FFS: Other condition.

Note: For the case that all the SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, there is no restriction that all the configured ROs in SBFD symbols should be within the UL usable PRBs.

- RAN1#117 agreements on random access:

Agreement

A RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is invalid by default.

A configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. This is only supported for RACH configuration option 2 and only supported for the ROs configured by the additional RACH configuration. If network configures such RO as a valid RO, UE should treat the RO as an additional-RO in SBFD symbols, and the followings are assumed by network and UE:

- The same frequency resources are used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL transmit power is used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL spatial domain filter is used for both the SBFD segment and non-SBFD segment of the PRACH.

- UE doesn’t stop PRACH transmission in the transition period/gap (if any) between SBFD and non-SBFD symbols

- There are no phase coherency requirements on the UE between the SBFD segment and non-SBFD segment of the PRACH.

- Other assumptions are not precluded.

NOTE: For FR2, network may need to ensure that the additional-RO and the legacy RO, which overlap with each other in time domain, are mapped to the same SSB.

Agreement

Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

Agreement

Confirm the following working assumption.

Working Assumption

For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported.

- For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc.

- For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc.

UE is not required to support both options.

R1-2405656	Summary#4 on SBFD random access operation	Moderator (CMCC)

Conclusion

For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1:

- The legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs.

- It is up to the network to configure ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon.

- If legacy ROs are configured in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, network ensures the ROs are within the UL usable PRBs

Agreement

For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, consider whether additional condition(s) on top of what was agreed in RAN1#116bis for RO validation are necessary. For example,

- Condition#1: A valid RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol.

- Condition#2: A valid RO starts at least X>0 (FFS the value) symbols after the SSB.

- Condition#3: A valid RO does not precede a SSB in the PRACH slot.

Agreement

Update the following agreement made in RAN1#116-bis meeting:

For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration,

- For FR2, adopt the following:

- Alt 1: use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211)

- FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols.

- For FR1, consider from the following alternatives:

- Alt 1: Use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211)

- FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols.

- Alt 2: Use existing random access configurations table for paired spectrum/supplementary uplink (i.e., Table 6.3.3.2-2 in TS38.211)