3GPP TSG-RAN WG2 Meeting #127	R2-2406824

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	NTT DOCOMO, INC.

Title:	Discussions on simulation results for RRM measurement prediction

Agenda Item:	8.3.2.1

Document for: 	Discussion and Decision

- Introduction

At RAN#102 meeting, the SID on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was approved. The SID describes the study scope of the AI/ML-aided mobility enhancements, as below,

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

At RAN2 #125bis meetings, it was agreed to start the simulations for the following items regarding the RRM measurement prediction on both FR1 and FR2 and three cases will be considered for the cell-level measurement predictions [2].

Agreements to start evaluations

- FR1-to-FR1

  - Focus on intra-frequncy in time domain prediction for the purpose of measurement reduction

  - Study inter-frequency scenario in terms of which scenarios can be studied without requiring new channel model and also resolving any simulation assumptions (if possible).

- FR2-to-FR2

  - Focus on intra-frequency

  - Perform evaluation both in time and spatial domain

Agreements

- For cell level measurement prediction model, at least consider the following cases:

  - Case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

  - Case 2: To directly predict cell level results based on cell level results.

  - Case 3: To directly predict cell level results based on beam level results.

At RAN2 #126 meetings, six scenarios and their priorities were agreed, and the detailed simulation assumptions were listed [4].

The following is agreed

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this paper, we submit our evaluation results about the 3 high-priority scenarios.

- AI/ML-based predictors for each scenario

2.1 Scenario 2, FR1-to-FR1 intra-frequency temporal domain case B

The study goal of this scenario is to reduce the measurement overhead. It is expected that the UE can lessen the actual measurement to 1/n of that without AI/ML-based predictions. The AI/ML-based predictor can output the ones not measured by the UE.

Figure 1 shows the diagrams of the AI/ML models for all 3 cases with different inputs and outputs of the model. For all scenarios evaluated, we consider a Transformer-based AI/ML model and cluster-based approaches, where the inputs to the AI/ML model consist of the measurements from multiple cells, and the outputs are the predictions of the measurements for corresponding cells. Based on the UE capabilities, we consider the cluster with 7 cells.

The significant differences between Case 1 and Case 2 are the position of the beam consolidation/selection and Layer 3 filter. In contrast, for Case 3, the behavior of the beam consolidation/selection and Layer 3 filter are implicitly included in the AI/ML models.

Figure 1 Diagram of AI/ML models for 3 cases of FR1-to-FR1 intra-frequency temporal domain Case B

2.2 Scenario 3, FR1-to-FR1 inter-frequency (frequency domain)

The study goal of this scenario is also to reduce the measurement overhead, especially the measurement gap needed for the measurements on other frequency bands. Based on the measurements on the band of the serving cell, the AI/ML-based predictor outputs the predicted measurements on the other bands.

Figure 2 shows the diagrams of the AI/ML models for all 3 cases of this scenario. We also use the Transformer-based model and follow the cluster-based approaches with a 7-cell cluster. The inputs to the model are the measurements of 7 cells on frequency band #1, and the outputs from the model are the predicted measurements of 7 co-located cells on frequency band #2 as the input ones.

Figure 2 Diagram of AI/ML models for 3 cases of FR1-to-FR1 inter-frequency

2.3 Scenario 4, FR2-to-FR2 intra-frequency domain case A

Differing from the previous scenarios, the study goal of this scenario is to improve the handover performance with AI/ML-based temporal predictions. With predictions on the temporal domain, the network or UE can report measurement events, select the target cell more intelligently, or predict the happening of some unintended events, such as the radio link failure.

The diagram of schemes is shown in Figure 3, which also follows the Transformer-based backbone and the cluster-based approach with 7 cells. The inputs to the AI/ML models are the historical measurements, and the outputs of the AI/ML model are the predictions for future slots.

Figure 3 Diagram of AI/ML models for 3 cases of FR2-to-FR2 intra-frequency temporal domain Case A

- Performance evaluations

3.1 Common simulation assumptions

Following the agreements at RAN2 #126, the common simulation assumptions are listed below in Table 1.

Table  1 Common simulation assumptions

| Parameter | FR1 Band #1 (common) | FR1 Band #2 (inter-frequency) | FR2 |
| --- | --- | --- | --- |
| Carrier frequency | 4 GHz (TDD) | 2 GHz (FDD) | 30 GHz |
| SCS | 30 kHz | 15 kHz | 120 kHz |
| Deployment | 2-tier with wrap around (7 sites, 3 sectors per site) |  | 2-tier with wrap around (7 sites, 3 sectors per site) |
| ISD | 500 m |  | 200 m |
| Channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. Inter-frequency correlation model: Section 7.6.5 in TR38.901. |  | UMi w/ distance-dependent LoS probability function defined in Table 7.4.2-1 of TR38.901. |
| System bandwidth | 20 MHz |  | 80 MHz |
| Sample period | 40 ms |  | 20 ms |
| UE speed | 30 kmph |  | 30 kmph |
| UE distribution/dropping | 100% outdoor |  | 100% outdoor |
| BS antenna configuration | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ Number of BS beams: 4 |  | (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Number of BS beams: 32 |
| BS antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi |  | TR 38.802 Table A.2.1-6 |
| UE antenna configuration | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |  | (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Number of UE beams: 4 |
| UE antenna radiation pattern | Omni-direction |  | TR 38.802 Table A.2.1-8 |
| BS Tx power | 44 dBm for 20 MHz bandwidth |  | 40 dBm |
| Maximum UE Tx power | 23 dBm |  | 23 dBm |
| BS receiver noise figure | 5 dB |  | 7 dB |
| UE receiver noise figure | 9 dB |  | 10 dB |
| BS antenna height | 25 m |  | 10 m |
| UE antenna height | 1.5 m |  | 1.5 m |
| Spatial consistency | Procedure A in TR38.901 |  | Procedure A in TR38.901 |
| UE trajectory model | Option 3 |  | Option 3 |
| UE boundary model | Option 3 |  | Option 3 |

The RRC parameters are configured following the post-RAN2 #126 email agreements in the following tables.

Table  2 RRC parameters for the Layer 3 filtering

| L3 filtering parameter | value |
| --- | --- |
| FR1 Filter Coefficient | 4 |
| FR2 Filter Coefficient | 4 |

Table  3 RRC parameters for the measurement period

| Measurement period | value |
| --- | --- |
| FR1 to FR1 intra-frequency w.o. gap | 200 ms |
| FR1 to FR1 inter-frequency with gap | 200 ms |
| FR2 to FR2 intra-frequency w.o. gap | 400 ms |

Table  4 RRC parameters for beam consolidation/selection

| Consolidation parameter | value |
| --- | --- |
| nrofSS-BlocksToAverage for FR1 | 1 |
| nrofSS-BlocksToAverage for FR2 | 3 |
| absThreshSS-BlocksConsolidation for FR1 | -110 dBm |
| absThreshSS-BlocksConsolidation for FR2 | -110 dBm |

We report our evaluation results for all 3 high-priority scenarios in the following sub-sections based on the table template agreed after Post-RAN2 #126 email discussions. All the reported values regarding the model complexity are about the models that can handle the inputs of a cluster of 7 cells and output the predictions for the corresponding cluster of 7 cells.

3.2 Scenario 2, FR1-to-FR1 intra-frequency temporal domain case B

Table  5 Evaluation results for Scenario 2

| Report parameters |  | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option | Option 3 |  |  |
|  | UE trajectory boundary processing option | Option 3 |  |  |
|  | UE speed | 30 kmph |  |  |
|  | Inter-frequency correlation assumption in general | N/A |  |  |
|  | Inter-frequency shadow fading correction | N/A |  |  |
|  | Whether LOSsoft is modeled or not | No |  |  |
|  | spatial consistency option (A or B) | Option A |  |  |
|  | Number of TX beams | 4 |  |  |
|  | Number of RX beams | 1 |  |  |
|  | Measurement reduction rate | 50% |  |  |
|  | Observation window | N/A |  |  |
|  | Prediction window | N/A |  |  |
|  | Any other parameters | Refer to Section 3.1 for common parameters. |  |  |
| Data Size | Training/validity | 212 k (UE trajectories) |  |  |
|  | Testing | 53 k (UE trajectories) |  |  |
| AI/ML model input/output | Model input | L1-RSRP on Point A1, 7 cells (incl. serving) measured | L3-RSRP on Point C 7 cells (incl. serving) measured | L1-RSRP on Point A1, 7 cells (incl. serving) measured |
|  | Model output | Predicted L1-RSRP, same cells and beams as the input | Predicted L3-RSRP, same cells as the input | Predicted L3-RSRP, same cells as the input |
| AI/ML model description | Model type | Transformer |  |  |
|  | Model complexity in a number of parameters(M) | 7.91 | 7.89 | 7.90 |
|  | Model complexity in model size (e.g. Mbyte) | 36.3 | 36.3 | 36.3 |
|  | Computational complexity [FLOPs] | 79.19 M | 79.08 M | 79.14 M |
| Metrics | Average L3 cell level RSRP difference (dB) | 0.53 | 0.45 | 0.43 |

For scenario 2, we have the following observations,

Observation 1

- For scenario 2, the average L3 cell-level RSRP of the AI/ML-based predictions is 0.43-0.53 dB. The AI/ML-based predictors can predict the measurements omitted by the overhead reduction.

- Of the three cases, Case 3 (L1 beam-level RSRP to L3 cell-level RSRP) achieves the best performance.

3.3 Scenario 3, FR1-to-FR1 inter-frequency (frequency domain)

Table  6 Evaluation results for Scenario 3

| Report parameters |  | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option | Option 3 |  |  |
|  | UE trajectory boundary processing option | Option 3 |  |  |
|  | UE speed | 30 kmph |  |  |
|  | Inter-frequency correlation assumption in general | Yes |  |  |
|  | Inter-frequency shadow fading correction | Full |  |  |
|  | Whether LOSsoft is modeled or not | No |  |  |
|  | spatial consistency option (A or B) | Option A |  |  |
|  | Number of TX beams | 4 |  |  |
|  | Number of RX beams | 1 |  |  |
|  | Measurement reduction rate | 100% for Band #2 |  |  |
|  | Observation window | N/A |  |  |
|  | Prediction window | N/A |  |  |
|  | Any other parameters | Refer to Section 3.1 for common parameters |  |  |
| Data Size | Training/validity | 1615 k (UE trajectories) |  |  |
|  | Testing | 403 k (UE trajectories) |  |  |
| AI/ML model input/output | Model input | L1-RSRP on Point A1, 7 cells (incl. serving) measured | L3-RSRP on Point C 7 cells (incl. serving) measured | L1-RSRP on Point A1, 7 cells (incl. serving) measured |
|  | Model output | Predicted L1-RSRP, co-located cells and beams as the input | Predicted L3-RSRP, co-located cells as the input | Predicted L3-RSRP, co-located cells as the input |
| AI/ML model description | Model type | Transformer |  |  |
|  | Model complexity in a number of parameters(M) | 7.89 | 7.89 | 7.89 |
|  | Model complexity in model size (e.g. Mbyte) | 30.2 | 30.2 | 30.2 |
|  | Computational complexity [FLOPs] | 446.68 M | 110.75 M | 446.68 M |
| Metrics | Average L3 cell level RSRP difference (dB) | 0.97 | 0.93 | 0.57 |

For scenario 3, we have the following observations,

Observation 2

- For scenario 3, the average L3 cell-level RSRP of the AI/ML-based predictions is 0.57-0.97 dB. The AI/ML-based predictors can predict the RSRP of co-located cells on Band #2 (4 GHz) based on the actual measurements of cells on Band #1 (2 GHz).

- Of the three cases, Case 3 (L1 beam-level RSRP to L3 cell-level RSRP) achieves the best performance.

3.4 Scenario 4, FR2-to-FR2 intra-frequency temporal domain case A

Table  7 Evaluation results for Scenario 4

| Report parameters |  | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option | Option 3 |  |  |
|  | UE trajectory boundary processing option | Option 3 |  |  |
|  | UE speed | 30 kmph |  |  |
|  | Inter-frequency correlation assumption in general | N/A |  |  |
|  | Inter-frequency shadow fading correction | N/A |  |  |
|  | Whether LOSsoft is modeled or not | No |  |  |
|  | spatial consistency option (A or B) | Option A |  |  |
|  | Number of TX beams | 32 |  |  |
|  | Number of RX beams | 8 |  |  |
|  | Measurement reduction rate | N/A |  |  |
|  | Observation window | 5 samples with 400 ms interval |  |  |
|  | Prediction window | 5 samples with 400 ms interval |  |  |
|  | Any other parameters | Refer to Section 3.1 for common parameters |  |  |
| Data Size | Training/validity | 162 k (UE trajectories) |  |  |
|  | Testing | 40 k (UE trajectories) |  |  |
| AI/ML model input/output | Model input | L1-RSRP on Point A1, 7 cells (incl. serving) measured | L3-RSRP on Point C 7 cells (incl. serving) measured | L1-RSRP on Point A1, 7 cells (incl. serving) measured |
|  | Model output | Predicted L1-RSRP, same cells and beams as the input | Predicted L3-RSRP, same cells as the input | Predicted L3-RSRP, same cells as the input |
| AI/ML model description | Model type | Transformer |  |  |
|  | Model complexity in a number of parameters(M) | 9.59 | 9.48 | 9.53 |
|  | Model complexity in model size (e.g. Mbyte) | 36.7 | 36.2 | 36.5 |
|  | Computational complexity [FLOPs] | 96.75 M | 95.08 M | 95.64 M |
| Metrics | Average L3 cell level RSRP difference (dB) | 1.62 (400 ms) 2.22 (800 ms) 2.76 (1200 ms) 3.18 (1600 ms) 3.48 (2000 ms) | 1.35 (400 ms) 1.94 (800 ms) 2.32 (1200 ms) 2.62 (1600 ms) 2.85 (2000 ms) | 0.95 (400 ms) 1.12 (800 ms) 1.25 (1200 ms) 1.39 (1600 ms) 1.55 (2000 ms) |

For scenario 4, 5 measurement samples with 400 ms intervals are predicted. We report the average L3 cell-level RSRP for these 5 predictions separately in Table 7. From the results, we have the following observations,

Observation 3

- For scenario 4, the average L3 cell-level RSRP of the AI/ML-based predictions is 0.95-1.62 dB when the prediction window is 400 ms. The best performance (0.95 dB) is achieved with Case 3. The AI/ML predictors can predict the RSRP of future measurement instances based on several historical ones.

- When the prediction distance is enlarged, the prediction accuracy is degenerated. The average RSRP difference is 1.55 dB when the prediction distance is 2 sec, which is also achieved by Case 3.

3.4 Summary of the evaluations

Our observations for all 3 high-priority scenarios are summarized as the following,

Observation 4

- For Scenario 2, Scenario 3, and Scenario 4 with a 400 ms prediction window, the accuracy of the AI/ML predictions can be less than 1 dB.

- Regarding 3 cases with different inputs and outputs, Case 3 outperforms the others for all 3 scenarios.

  - The Layer 1 measurement results are more informative as the inputs to the AI/ML models, while the Layer 3 filtering may filter some information out from the data.

  - It reduces the complexity of the learning by letting the AI/ML model learn the final Layer 3 outputs instead of the more complex intermediate Layer 1 results.

Based on our evaluation of these three scenarios, we observed that the AI/ML-based predictors generate predictions with enough accuracy to motivate further studies, which will be discussed in our contributions for Session 8.3.2.2. We have the following proposals,

Proposal 1

- Conclude that the AI/ML-based RRM measurement predictions for all three high-priority scenarios are accurate enough to promote next-stage studies.

  - Consider Case 3 (L1 beam-level RSRP to L3 cell-level RSRP) for future studies based on the evaluations.

  - FFS the details of the future studies (in Session 8.3.2.2).

- Conclusion

In this contribution, we have the following observations and proposals,

Proposal 1

- Conclude that the AI/ML-based RRM measurement predictions for all three high-priority scenarios are accurate enough to promote next-stage studies.

  - Consider Case 3 (L1 beam-level RSRP to L3 cell-level RSRP) for future studies based on the evaluations.

  - FFS the details of the future studies (in Session 8.3.2.2).

Observation 1

- For scenario 2, the average L3 cell-level RSRP of the AI/ML-based predictions is 0.43-0.53 dB. The AI/ML-based predictors can predict the measurements omitted by the overhead reduction.

- Of the three cases, Case 3 (L1 beam-level RSRP to L3 cell-level RSRP) achieves the best performance.

Observation 2

- For scenario 3, the average L3 cell-level RSRP of the AI/ML-based predictions is 0.57-0.97 dB. The AI/ML-based predictors can predict the RSRP of co-located cells on Band #2 (4 GHz) based on the actual measurements of cells on Band #1 (2 GHz).

- Of the three cases, Case 3 (L1 beam-level RSRP to L3 cell-level RSRP) achieves the best performance.

Observation 3

- For scenario 4, the average L3 cell-level RSRP of the AI/ML-based predictions is 0.95-1.62 dB when the prediction window is 400 ms. The best performance (0.95 dB) is achieved with Case 3. The AI/ML predictors can predict the RSRP of future measurement instances based on several historical ones.

- When the prediction distance is enlarged, the prediction accuracy is degenerated. The average RSRP difference is 1.55 dB when the prediction distance is 2 sec, which is also achieved by Case 3.

Observation 4

- For Scenario 2, Scenario 3, and Scenario 4 with a 400 ms prediction window, the accuracy of the AI/ML predictions can be less than 1 dB.

- Regarding 3 cases with different inputs and outputs, Case 3 outperforms the others for all 3 scenarios.

  - The Layer 1 measurement results are more informative as the inputs to the AI/ML models, while the Layer 3 filtering may filter some information out from the data.

  - It reduces the complexity of the learning by letting the AI/ML model learn the final Layer 3 outputs instead of the more complex intermediate Layer 1 results.

References

- Moderator (CMCC), RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR”, Dec. 2023.

- RAN2 Chair (InterDigital), “RAN2 #125bis Chair Notes”, Apr., 2024.

- 3GPP TR 38.901 v17.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz”, Mar., 2022.

- RAN2 Chair (InterDigital), “RAN2 #126 Chair Notes”, May, 2024.