3GPP TSG-RAN WG2 Meeting #127	R2-2406825

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on other aspects related to RRM measurement prediction

Agenda Item:	8.3.2.2

Document for: 	Discussion and Decision

- Introduction

During the RAN#102 meeting, the SID on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was approved. The SID describes the study scope of the AI/ML-aided mobility enhancements, as below,

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

At RAN2 #125bis meetings, it was agreed to start the simulations for the following items regarding the RRM measurement prediction on FR1 and FR2. Three cases will be considered for the cell-level measurement predictions [2].

Agreements to start evaluations

- FR1-to-FR1

  - Focus on intra-frequncy in time domain prediction for the purpose of measurement reduction

  - Study inter-frequency scenario in terms of which scenarios can be studied without requiring new channel model and also resolving any simulation assumptions (if possible).

- FR2-to-FR2

  - Focus on intra-frequency

  - Perform evaluation both in time and spatial domain

Agreements

- For cell level measurement prediction model, at least consider the following cases:

  - Case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

  - Case 2: To directly predict cell level results based on cell level results.

  - Case 3: To directly predict cell level results based on beam level results.

At RAN2 #126 meetings, six scenarios and their priorities were agreed, and the detailed simulation assumptions were listed [4].

The following is agreed

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Our performance evaluations of these high-priority scenarios are submitted to Session 8.3.2.1, based on which we propose to conclude that the AI/ML-based RRM measurement predictions for all three high-priority scenarios are accurate enough to promote next-stage studies. This paper discusses the remaining issues and the contents for further studies.

- Remaining issues on the evaluation methodology

2.1 Number of cells for the prediction

Multiple cells, including the serving, are naturally involved in mobility enhancements. For example, the predictions on the RRM measurements of multiple cells should be available to support the following use cases.

- For NW-sided models,

  - Target cell selection/prediction based on the RRM measurement predictions. The network should have the predictions of serving and multiple neighboring cells for such purposes.

- For UE-sided models,

  - Measurement event predictions, such as Event A3, involve both serving and neighboring cells to determine the entering and leaving.

Therefore, the RRM measurement predictions for multiple cells should be investigated for this study. There are several approaches, such as using cell-specific models to predict the measurements of serving and neighboring cells one-by-one, cell-common models to predict cell-by-cell, or a cluster-based model to predict the measurements for a cluster of cells jointly. All these approaches can satisfy the requirements of mobility enhancements, and they are only implementation issues and can be up to the company.

Based on these discussions, we have the following observations and proposals,

Observation 1

- The RRM measurements of multiple cells are involved during the mobility. The predictions for only one cell (serving or neighboring) are insufficient for mobility enhancements such as target cell selection or measurement event prediction (especially for Event A3).

Proposal 1

- Study the RRM measurement prediction for multiple cells to enhance the mobility performance.

  - The detailed AI/ML approaches, e.g., cell-common, cell-specific, or cell-cluster-based models, are up to the companies.

2.2 Prediction window length for temporal domain prediction Case A

The study goal of the temporal domain prediction Case A (FR2-to-FR2) is to enhance mobility performance by predicting future measurements. Therefore, the length of the prediction window is essential to the potential performance gain. As discussed in our contribution for Session 8.3.4.1, the radio link failure (RLF) prediction can be based on the measurement predictions. Long-term RLF prediction should be made before triggering the T310. It requires a prediction window at the second level.

Observation 2

- The RRM measurement predictions on the temporal domain can be used for the RLF prediction. Long-term RLF prediction requires a long prediction window at the second level.

Due to such requirements, our proposals are,

Proposal 2

- Study the RRM measurement prediction with a long prediction window (e.g., > 1s) for the temporal domain prediction Case A.

  - The prediction window length can be N times the sample duration.

  - Multiple predictions can be included within the prediction window.

- Further study on the impacts of RRM measurement predictions on the handover performance

From the performance evaluations of the AI/ML-based predictions, the RSRP differences are acceptable for motivating further study. However, the RSRP difference cannot directly reflect the final mobility performance since it has complex impacts on the multiple stages of the handover procedures. RAN2 should identify whether and how much the predicted measurements can improve the mobility performance for further studies. For this purpose, we discuss the potential usage of the RRM measurement predictions case-by-case and propose proper evaluation methodologies for different cases. Finally, some modelling issues are discussed to align the simulation from different companies.

3.1 Potential usage of the measurement predictions

RAN2 has agreed on two study goals for the RRM measurement predictions. The 1st study goal is overhead reduction, and the 2nd is HO performance enhancement. Different evaluation methodologies can be adopted to examine if the predictions satisfy the goals. Hence, we discuss it separately.

- For the 1st study goal (overhead reduction), where Scenario 2 and Scenario 3 are involved,

  - For the NW-sided model, the major task is to recover the omitted RRM measurements by the UE based on the UE reports of reduced measurements. The recovery can be on the temporal domain (Scenario 2) or frequency domain (Scenario 3).

  - For the UE-sided model, UE can recover the reduced measurements directly. The recovered measurements can further be used for measurement reports.

- For the 2nd study goal (HO performance), where Scenario 4 is involved,

  - For the NW-sided model, the major task is to use the predicted RRM measurements to avoid unintended events or select the target cell smartly and proactively. The handover procedures are significantly impacted by the predictions.

  - For the UE-sided model, the predictions can be used for measurement event predictions (under the scope of Session 8.3.3) or RLF predictions (under the scope of Session 8.3.4). How UE utilizes these predictions also introduces significant impacts on the handover procedures.

Observation 3

- For the 1st study goal (overhead reduction), it is essential to verify if AI/ML can fully recover the omitted measurements.

- For the 2nd study goal (HO performance), the handover procedures are significantly impacted, which performance should be verified by system-level simulations.

3.2 Evaluation methodology of the further study

According to the discussions, for the 1st study goal, the major task of the AI/ML model is to recover the omitted measurements. It can be concluded that the overhead reduction is achieved if the predicted measurements have a minor impact on the measurement report (UE-sided model) or the target cell selection (NW-sided model). Therefore, we can further evaluate the differences in measurement reports or target cell selections for concluding.

The study can be based on comparing the measurement reports/target cell selections before and after the overhead reduction with AI/ML models, e.g., the differences in measurement reports or target cells are less than a threshold such as 5%.

Proposal 3

- For the 1st study goal, further study the impacts of the RRM measurement prediction on the mobility performance by comparing the results of measurement reports (UE-sided model) or target cell selection (NW-sided model).

  - Conclude that the overhead reduction with AI/ML models can be achieved if it has a minor impact on the measurement reports or target cell selections, e.g., the difference is less than 5%.

For the 2nd study goal, which directly enhances mobility performance, system-level simulations (SLS) with a mobility model should be involved due to the complex procedures of the handover. Otherwise, we will not know whether and how much the predictions can improve the performance. For the NW-sided model, the predictions can be used for the target cell selection. For the UE-sided model, the predictions can be used for the measurement event predictions or RLF predictions, which fall into the scope of Session 8.3.3 and 8.3.4, respectively. Therefore, we have the following proposals for the 2nd study goal,

Proposal 4

- For the 2nd study goal, further study the impacts of the RRM measurement predictions on the mobility performance by the SLS.

  - For the UE-sided model, the SLS performance can be studied in corresponding sessions (8.3.3 or 8.3.4).

3.3 EVM for SLS evaluations

For the SLS regarding the handover performance, there is a 3-state HO model introduced in TR36.839 [5], which includes the following detailed models,

- Definition of the 3 states based on the A3 entering, HO command reception, and HO complete transmission.

- RLF modelling and RLF states. The Qin and Qout for RLF are generated based on L1 measurement samples with a given sample rate and L1 filtering.

- Handover/PDCCH failure modelling based on the T310 state, RLF, and the filtered average wideband CQI with a given sample rate and L1 filtering.

- Ping-pong and time-of-stay (ToS) modelling.

With these detailed models, the HOF caused by several seasons can be modelled, which includes the T310 expiry in the source cell (HO too late), HO command failure (HO too late), and HO complete failure (HO too early/wrong). The HOF caused by the RA failure (HO too early/wrong) and the T310 expiry in the target cell (HO too early/wrong) are not modelled. The state of the art of model in [5] is summarized in Table 1.

Table  1 HO Models in TR36.839

| HOF type | Main reason | Sub-reason | Model availability |
| --- | --- | --- | --- |
| Too late HO | Failure in Source | T310 running/expiry in source | Yes |
|  |  | HO command failure | Yes |
| Too early/Wrong HO | Failure in Target | RA failure | No |
|  |  | HO complete failure | Yes |
|  |  | T310 running/expiry in target | No |

Considering that TR36.839 provides a simplified simulation model, we suggest using it as much as possible and discussing whether to enhance it for the issues not addressed. Our proposals are then,

Proposal 5

- Reuse the TR36.839 model for the HOF causes modeled for the SLS regarding 2nd study goal as much as possible.

  - FFS whether and how to enhance the model for unaddressed issues, e.g., RA failure in the target cell, T310 running/expiry in the target cell.

Another issue regarding the models in TR36.839 is that they are designed for the HO evaluations for LTE networks with the HetNet deployments. Some detailed parameters should be revisited to align with our current studies, and some unmentioned parameters can be further defined to obtain comparable results from different companies. Therefore, we propose the following details about the model,

Proposal 6

- Consider the following for the evaluations with SLS,

  - Reuse the definition of the 3 HO states in TR36.839 for the HOF and Ping-pong modelling.

  - Align the L1 sample period and L3 filter with the simulation assumptions of this study.

  - Align the threshold of wideband CQI for handover/PDCCH failure modelling, and the corresponding sample period and filter.

  - Align the parameters for Qin and Qout threshold, T310, N310, and N311.

  - Align the parameters for Event A3 entering/leaving.

  - Other implementations, such as HO command/complete transmission, and HO decision/target cell selection, are up to the companies.

3.4 KPIs for the SLS

We suggest evaluating the typical handover KPIs, such as HOF rate, ping-pong rate, and time-of-stay, from the SLS. Considering that there are still many detailed implementations up to the companies, we feel that it is hard to align the benchmark performance from companies. Therefore, we suggest companies report the benchmark performance and the relative gains of AI/ML to their own benchmark performance. The relative gains will be used to conclude the studies.

Proposal 7

- Study the HOF rate, Ping-pong rate, and CDF of the time-of-stay from system-level simulations.

- Companies to implement the HO model and a rule-based HO procedure based on the models in TR36.839 and possible RAN2 agreements as a benchmark.

- Companies to report the benchmark performance and relative gain/loss of AI/ML to their benchmark for the result collections.

- Conclusion

In this contribution, we have the following observations and proposals,

Observation 1

- The RRM measurements of multiple cells are involved during the mobility. The predictions for only one cell (serving or neighboring) are insufficient for mobility enhancements such as target cell selection or measurement event prediction (especially for Event A3).

Observation 2

- The RRM measurement predictions on the temporal domain can be used for the RLF prediction. Long-term RLF prediction requires a long prediction window at the second level.

Observation 3

- For the 1st study goal (overhead reduction), it is essential to verify if AI/ML can fully recover the omitted measurements.

- For the 2nd study goal (HO performance), the handover procedures are significantly impacted, which performance should be verified by system-level simulations.

Proposal 1

- Study the RRM measurement prediction for multiple cells to enhance the mobility performance.

  - The detailed AI/ML approaches, e.g., cell-common, cell-specific, or cell-cluster-based models, are up to the companies.

Proposal 2

- Study the RRM measurement prediction with a long prediction window (e.g., > 1s) for the temporal domain prediction Case A.

  - The prediction window length can be N times the sample duration.

  - Multiple predictions can be included within the prediction window.

Proposal 3

- For the 1st study goal, further study the impacts of the RRM measurement prediction on the mobility performance by comparing the results of measurement reports (UE-sided model) or target cell selection (NW-sided model).

  - Conclude that the overhead reduction with AI/ML models can be achieved if it has a minor impact on the measurement reports or target cell selections, e.g., the difference is less than 5%.

Proposal 4

- For the 2nd study goal, further study the impacts of the RRM measurement predictions on the mobility performance by the SLS.

  - For the UE-sided model, the SLS performance can be studied in corresponding sessions (8.3.3 or 8.3.4).

Proposal 5

- Reuse the TR36.839 model for the HOF causes modeled for the SLS regarding 2nd study goal as much as possible.

  - FFS whether and how to enhance the model for unaddressed issues, e.g., RA failure in the target cell, T310 running/expiry in the target cell.

Proposal 6

- Consider the following for the evaluations with SLS,

  - Reuse the definition of the 3 HO states in TR36.839 for the HOF and Ping-pong modelling.

  - Align the L1 sample period and L3 filter with the simulation assumptions of this study.

  - Align the threshold of wideband CQI for handover/PDCCH failure modelling, and the corresponding sample period and filter.

  - Align the parameters for Qin and Qout threshold, T310, N310, and N311.

  - Align the parameters for Event A3 entering/leaving.

  - Other implementations, such as HO command/complete transmission, and HO decision/target cell selection, are up to the companies.

Proposal 7

- Study the HOF rate, Ping-pong rate, and CDF of the time-of-stay from system-level simulations.

- Companies to implement the HO model and a rule-based HO procedure based on the models in TR36.839 and possible RAN2 agreements as a benchmark.

- Companies to report the benchmark performance and relative gain/loss of AI/ML to their benchmark for the result collections.

References

- Moderator (CMCC), RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR”, Dec. 2023.

- RAN2 Chair (InterDigital), “RAN2 #125bis Chair Notes”, Apr., 2024.

- 3GPP TR 38.901 v17.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz”, Mar., 2022.

- RAN2 Chair (InterDigital), “RAN2 #126 Chair Notes”, May, 2024.

- 3GPP TR 36.839 v11.1.0, “Mobility enhancements in heterogeneous networks (Release 11)”, Dec., 2012.