3GPP TSG-RAN WG2 Meeting #127	R2-2406826

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	NTT DOCOMO, INC.

Title:	Discussions on simulation assumptions and EVM for RLF predictions

Agenda Item:	8.3.4.1

Document for: 	Discussion and Decisions

- Introduction

During the RAN#102 meeting, the SID on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was approved. The SID describes the study scope of the AI/ML-aided mobility enhancements, as below,

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

At RAN2 #126 meeting, the following were agreed about the simulation of the RLF prediction [2].

- Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR).

- Study Direct: Directly RLF prediction by AI/ML models.

- FR2 study will be prioritized for RLF prediction.

- The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction.

- RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case.  FFS on expected RLF time and indirect case.

- Simulation assumption specific to RLF will be discussed in August.  The assumption is that we will reuse RRM simulation assumptions (where possible).

In this paper, we will discuss some detailed evaluation methodology issues after discussions about the RLF cases and RLF prediction results during the last meetings.

- Sub-cases of the prediction on RLF caused by T310 expiry

The study on the prediction of RLF focuses on the T310 expiry as agreed during the last meetings, and the definitions of the RLF prediction results have been discussed for the direct and indirect cases, respectively. There were discussions on the time window or time instance of the predictions. Regarding this issue, the RLF caused by the T310 expiry can be discussed separately in two cases.

The first case is the RLF prediction made after the triggering of T310, named the short-term RLF prediction. During such a stage, there have already been radio link problems, and the UE may be unable to communicate with the network. For this case, the happening time instance of the RLF is deterministic, which is decided by the remaining duration of T310, and does not need to be predicted. Only the probability of the RLF should be predicted by the AI/ML models. The discussions about the time window or time instance make no sense.

The second case is the long-term RLF prediction made before the triggering of T310. The radio link between the UE and the network may still work during such stage. The long-term RLF prediction requires the AI/ML model to output the prediction for a long prediction window, at least longer than the duration of T310. For the long-term RLF prediction, both the probability and the happening time window/instance should be predicted.

Due to the differences in the radio link states, the functionalities of the AI/ML models are significantly different for short- and long-term RLF predictions. Therefore, we propose to discuss these two cases separately.

Proposal 1

- The study on RLF prediction considers two sub-cases, Short-term and Long-term RLF predictions.

        - The short-term RLF prediction is the prediction after T310 is triggered.

          - Only the probability is predicted for this case.

          - The RLF prediction result is the probability that the RLF happens at the time instance of the running T310 expiry. There is no need to consider the performance metrics for time accuracy.

        - The long-term RLF prediction is the one before T310 is triggered.

          - Both the probability and the happening time window/instance are predicted for this case.

          - The RLF prediction result is the probability that the RLF happens in a predefined time window or a time instance. Consider the performance metrics regarding the RLF probability and time accuracy.

- FFS: How UE utilizes the short-/long-term RLF predictions and the priority of these two sub-cases.

The long-term RLF prediction based on the RRM measurement predictions, i.e., indirect RLF prediction, also requires the measurement predictions over a long prediction window at least longer than a T310 duration. Therefore, we have a proposal related to the RRM measurement prediction to support the indirect long-term RLF predictions.

Proposal 2

- Study the RRM measurement prediction with the prediction window at least longer than a T310 duration (e.g., 1s) for temporal domain Case A.

- Evaluation methodology on the accuracy of the predictions

The second issue discussed in this paper is how to evaluate the accuracy of the RLF predictions and how to collect ground truth for the model training.

The RLF predictions intend to predict whether the RLF will happen if the UE stays in the current serving cell without the handover. Both the recovery of the channel conditions and the successful HO can stop the T310 and recover the radio link. For the second case of link recovery after HO, the states of the radio link cannot be used to evaluate whether the RLF predictions are correct since the UE has been handed over to another cell. The collected data about the radio link states can be used to train the AI/ML models neither for the same reason.

Based on the discussions, we suggest the following clarifications on the evaluation methodologies of the RLF predictions,

Proposal 3

- The performance of RLF prediction is evaluated by assuming the UE stays in the same cell until the predicted happening time of RLF.

- The ground truth for the model training is collected based on the same assumption that UE stays in the same cell until the end of the prediction window.

- Conclusion

In this contribution, we have the following observations and proposals,

Proposal 1

- The study on RLF prediction considers two sub-cases, Short-term and Long-term RLF predictions.

        - The short-term RLF prediction is the prediction after T310 is triggered.

          - Only the probability is predicted for this case.

          - The RLF prediction result is the probability that the RLF happens at the time instance of the running T310 expiry. There is no need to consider the performance metrics for time accuracy.

        - The long-term RLF prediction is the one before T310 is triggered.

          - Both the probability and the happening time window/instance are predicted for this case.

          - The RLF prediction result is the probability that the RLF happens in a predefined time window or a time instance. Consider the performance metrics regarding the RLF probability and time accuracy.

- FFS: How UE utilizes the short-/long-term RLF predictions and the priority of these two sub-cases.

Proposal 2

- Study the RRM measurement prediction with the prediction window at least longer than a T310 duration (e.g., 1s) for temporal domain Case A.

Proposal 3

- The performance of RLF prediction is evaluated by assuming the UE stays in the same cell until the predicted happening time of RLF.

- The ground truth for the model training is collected based on the same assumption that UE stays in the same cell until the end of the prediction window.

References

- Moderator (CMCC), RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR,” Dec. 2023.

- RAN2 Chair (InterDigital), “RAN2 #126 Chair Notes”, May, 2024.