3GPP TSG RAN WG2 #127                                                                                  R2-2406827

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	CATT

Title:	Discussion on LCM for UE-sided model for Beam Management use case

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

- Introduction

In RAN2#126 meeting, RAN2 discussed the LCM for UE-sided model for beam management use case and achieved the agreements as follows [1].

Agreements

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

2	We will work offline on the definitions for functionality types and define what is availability.

3	The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable.   For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

4	For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning

5	For BM use case, As a baseline the UE determines whether a functionality is applicable.  Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive

In this contribution, we will continue to discuss these general aspects for functionality based LCM and also specific aspects for Beam Management use case.

- Discussion

  - General aspects for functionality based LCM

- Functionality granularity and capability

In R19 AI/ML for air interface WID, one objective for RAN2 is to design the signaling and protocol aspects of functionality based LCM. In RAN2#125bis meeting, functionality granularity and capability aspect was discussed and agreed that the supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability. Which AI/ML-enabled Features/FGs and functionalities are supported should be up to RAN1’s progress. However, according to the R19 AI/ML for air interface WID, the general framework part is out of RAN1 scope and no related discussion is ongoing in RAN1. We understand RAN1 will not discuss the AI/ML general framework unless triggered by other WGs.  Therefore, we suggest RAN2 to take over the discussion and confirm with RAN1 when the consensus is achieved in RAN2.

Observation 1: The AI/ML general framework is out of RAN1 scope and no related discussion for functionality granularity is ongoing in RAN1.

In RAN2#125bis meeting, a FFS is about what functionality refers to. “Functionality” was defined by RAN1 during R18 SI stage, and RAN2 has never discussed and achieved the common understanding for “functionality”. In R18 SI stage, RAN1 has given the definition of “functionality” and captured it to the TR 38.843 as follow.

For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG.

From RAN2's perspective, it can be understood that the functionality is network configuration(s) enabled AI/ML-enabled Feature/FG, and which configuration is supported indicated by UE. However, it is still unclear for the granularity of functionality specific to (sub) use case, e.g., whether it should be configuration correlated (e.g., for BM-Case1 sub use case, it refers to 4-to-32 beam prediction or 8-to-32 beam prediction), or it should be sub use case or use case level. In our understanding, at least it should be sub use case level or more detailed level (configuration correlated), we prefer configuration correlated, i.e., for BM-Case1 sub use case (i.e., spatial domain), the functionality granularity can be 4 beams-to-32 beams prediction or 8 beams-to-32 beams prediction, etc. For BM-Case2 sub use case (i.e., temporal domain), the functionality can be 4 samples to 4 samples prediction, 16 samples to 4 samples prediction, etc.

Proposal 1: For beam management use case, RAN2 to discuss which option is adopted for functionality granularity:

Option 1: Sub use case level;

Option 2: Inference configuration based level (e.g., 4 beams-to-32 beams prediction for spatial domain prediction, 16 samples to 4 samples prediction for temporal domain prediction).

For positioning accuracy enhancements use case, the similar options can also be considered. However, Option 2 is not clear for positioning accuracy enhancements use case at the current discussion stage, and we understand the functionality could be in sub use case level or more detailed level. Therefore, we suggest RAN2 to assume the granularity of functionality is sub use case level (case 1/2a/2b/3a/3b), and this can be revisited if more RAN1’s input is obtained.

Proposal 2: For positioning accuracy enhancements use case, RAN2 assumes the sub use case level (case 1/2a/2b/3a/3b) as the granularity of functionality, and this can be revisited if more RAN1’s input is available.

- Functionality types and applicability functionality reporting

In RAN2#126 meeting, functionality types and applicability functionality reporting related issues were discussed and some agreements were achieved. After RAN2#126 meeting, an email discussion [2] [3] was organized by Intel, Samsung to discuss the functionality types and applicable functionality reporting, and some consensus was achieved. However, there are still some issues which were left for further discussion.

- Proactive/reactive applicable functionality reporting

In [3], rapporteur summarized signaling procedure for proactive/reactive reporting as below.

Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

Step 3:  Network configures UE that it is allowed to provide its applicable functionalities.

Step 4: UE sends applicable functionalities to network upon change of applicable functionality/condition.

Step 5: Network sends inference configuration for the applicable functionalities to the UE.

Step 6: Start inference/monitoring based on network/UE activation/deactivation.

Figure 1 Proactive reporting

Step 1: Network sends UECapabilityEnquiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

Step 3:  Network provides network configurations and initiates UE to report its applicable functionalities.

Step 4: UE sends applicable functionalities to network.

Step 5: Network sends updated inference configuration for applicable functionalities reported in Step 4 to the UE. (see Q2-6)

Step 6: Start inference/monitoring based on network/UE activation/deactivation.

Figure 2 Reactive reporting

For step 3, whether the inference configuration of supported functionalities can be included, i.e., whether the inference configuration can be configured before applicable functionality reporting has no consensus. If the inference configuration is supported to be configured in step3 which is based on the UE reported supported functionalities, the network can configure the inference configuration for functionalities of all or sub of the supported functionalities based on the network implement, which means the network can blindly provide the inference configuration which even never be used as no available model in the functionality. Such blindly manner seems not necessary and will increase the signaling overhead.

Proposal 3: In functionality based LCM, the inference configuration will only be provided after applicable functionality reporting (i.e., in step 5).

In [2], the functionality types were discussed and a FFS is whether definition on configured functionalities is needed.

Configured functionalities: this refers to functionalities that gNB/LMF configured to UE. UE for model inference and performing measurements for training purposes? Depending on proactive/reactive approach, configured functionalities may or may not be applicable upon configuration.

Based on Proposal 3, the inference configuration should only be provided to UE from network after applicable functionality reporting, the network configures inference configuration for one of the applicable functionalities to active the functionality to allow UE to perform inference and monitoring procedure, which means the configured functionalities is the activated functionalities. Therefore, it’s not necessary to define the configured functionalities.

Proposal 4: No definition on configured functionalities is needed.

In our view, the applicable functionality reporting signaling procedure should be the network configure to UE that UE is allowed to do UAI reporting via OtherConfig in RRCReconfiguration message via step 3, and also the network provides the NW-side additional condition to UE by dedicated signaling or broadcast signaling when the network first indicates the NW-side additional condition to UE or when the NW-side additional condition has changed (if any). The UE then determines the applicable functionalities based UE-side additional condition (internally known by UE), NW-side additional condition (received from network), and model availability in device etc. The UE reports the applicable functionalities in UAI when it reports the applicable functionalities at the first time or when the applicable functionalities have changed. Based on the applicable functionalities, the network provides the inference configuration for one of the applicable functionalities, the UE considers the functionality which the inference configuration is received in step 5 is the initial active functionality, and starts to perform inference and monitoring. The overall signaling procedure is descripted in Figure 3 below.

Step 1: Network sends UECapabilityEnquiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

Step 3:  Network configures UE that it is allowed to provide its applicable functionalities.

Step 4: Network provides to UE the NW-side additional condition (by dedicated signaling or broadcast signaling).

Step 5: UE sends applicable functionalities to network upon first or change of applicable functionality.

Step 6: Network sends inference configuration for the selected applicable functionality to the UE.

Step 7: Start inference/monitoring based on network/UE activation/deactivation.

Figure 3 Signaling procedure for applicable functionality reporting

For step 4 in Figure 3, the network can provide the NW-side additional condition before step 3 (e.g., send by broadcast signaling), or in the same RRCReconfiguration message with the OtherConfig which the network configures UE it is allowed to provide its applicable functionalities, or after the step 3, which needs to be further discussed.

Proposal 5: RAN2 to take the Figure 3 signaling procedure as baseline and further to discuss the signaling details of step 4 (i.e., network provides the NW-side additional condition to UE).

- Non-applicable functionalities reporting

In [2], an issue that whether to define available functionalities separately from applicable functionalities is needed, but still is FFS. In [3], the related issue was discussed that whether UE can include non-applicable functionalities in Step 4 (in Figure 1 or Figure 2 above) after the basic procedure of applicable functionality reporting is clear, i.e., whether UE can report the functionalities which model is available in device, but not be applicable functionality. From our perspective, the intention of reporting the non-applicable functionalities is not clear, if the non-applicable functionalities are reported in step 4 is for the network to configure them in step 5 as mentioned in [3], we understand it is not necessary the network to provide the inference configuration for a non-applicable functionality, and this will introduce some extra complexity if supported. Also, no definition on available functionalities separately from applicable functionalities is needed.

Proposal 6: Non-applicable functionalities reporting by UE to network is not supported, also no definition of available functionalities separately from applicable functionalities is needed.

- Neighbouring cell’s applicable functionality reporting

In [3], an issue was proposed by companies that whether neighbouring cell’s applicability needs to be considered in applicable functionality reporting to serving cell. In the current stage, the intention of neighbouring cell’s applicable functionality reporting is not clear, based on the discussion in [3], the NW-side additional condition needs to provide to UE from network for determining the applicable functionalities in UE side, it can be e.g., associated ID is used to indicate the NW-side additional condition, and in RAN1#117 meeting, the agreement was achieved as follow.

Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

Based on the highlighted in yellow part, the associated ID is cell level, and ffs whether can applicable for other cells, RAN1 will continue to discuss the ffs issue which has the impact on neighbouring cell’s applicable functionality reporting. Therefore, we suggest that RAN2 to postpone the discussion on neighbouring cell’s applicable functionality reporting after more progress is made in RAN1 on NW-side additional condition.

Proposal 7: RAN2 to postpone the discussion on neighbouring cell’s applicable functionality reporting after more progress is made in RAN1 on NW-side additional condition.

  - Functionality-based LCM for UE-sided model for BM use case

- Functionality management

In R18 SI stage, performance monitoring was discussed per sub use case in RAN1 and some general aspects were achieved and captured in TR38.843. For beam use case, for UE-sided model, the following contents about performance monitoring were achieved.

For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Type 1 performance monitoring: - Configuration/Signalling from gNB to UE for measurement and/or reporting - UE may have different operations - Option 1 (NW-side performance monitoring): UE sends reporting to NW (e.g., for the calculation of performance metric at NW) - Option 2 (UE-assisted performance monitoring): UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s) - Indication from NW for UE to do LCM operations - Note: At least the performance and reporting overhead of model monitoring mechanism should be considered - Type 2 performance monitoring: - Indication/request/report from UE to gNB for performance monitoring - Note: The indication/request/report may be not needed in some case(s) - Configuration/Signalling from gNB to UE for performance monitoring measurement and/or reporting - If it is for UE side model monitoring, UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Mechanism that facilitates the UE to detect whether the functionality/model is suitable or no longer suitable

In Rel-19 WID stage, in RAN1#117 meeting, for BM-Case1 and BM-Case2 sub use cases, for UE-side model, the following agreement was achieved.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

It can be seen that, for BM-Case1 and BM-Case2 sub use cases, the general mechanisms for performance monitoring have been made in RAN1 during R18 SI discussion, and in Rel-19 WID stage, it is still under the discussion in RAN1.

In Rel-19 WID stage, RAN2 discussed the general mechanism for functionality management in RAN2#125bis meeting, and agreed as follow.

Agreements: - For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). - “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

However, which performance monitoring mechanism will be used for BM-Case1 and BM-Case2 sub use cases needs further discussion. We understand that RAN1 will continue the discussion for performance monitoring on the details for Type 1 and/or Type 2. Therefore, from RAN2’s perspective, RAN2 can wait for more progress on performing monitoring from RAN1 before discussing and deciding the performance monitoring mechanism and signaling details for beam management use case.

Proposal 8: For beam management use case for UE-sided model, RAN2 postpones the discussion of performance monitoring mechanism and signaling details until more progress is made in RAN1.

- Model inference

During Rel-19 WID stage, RAN1 discussed the details (report content of inference results, the configuration of inference results reporting etc.) for model inference for UE-sided model for beam management use case.

Based on the discussion in RAN1, for UE-sided model, the inference results need to report to the network, and according to the reply LS [4] from RAN1 in R18 SI stage, it indicated the beam prediction results for UE-sided model is time-critical for reporting, then, L1 signaling should be used for the reporting. For the configuration of inference results reporting for UE-sided model at least for BM-Case1, RAN1 agreed CSI-ReportConfig is used, the details discussion is still ongoing in RAN1, RAN2 can wait for RAN1’s progress. Therefore, RAN2 can start the discussion for model inference for UE-sided model for beam management use case until more progress made in RAN1.

Proposal 9: For beam management use case, RAN2 postpones the discussion for UE-sided model inference until more progress made in RAN1.

- Conclusion

- General aspects for functionality based LCM

Functionality granularity and capability

Observation 1: The AI/ML general framework is out of RAN1 scope and no related discussion for functionality granularity is ongoing in RAN1.

Proposal 1: For beam management use case, RAN2 to discuss which option is adopted for functionality granularity:

Option 1: Sub use case level;

Option 2: Inference configuration based level (e.g., 4 beams-to-32 beams prediction for spatial domain prediction, 16 samples to 4 samples prediction for temporal domain prediction).

Proposal 2: For positioning accuracy enhancements use case, RAN2 assumes the sub use case level (case 1/2a/2b/3a/3b) as the granularity of functionality, and this can be revisited if more RAN1’s input is available.

Functionality types and applicability functionality reporting

Proposal 3: In functionality based LCM, the inference configuration will only be provided after applicable functionality reporting (i.e., in step 5).

Proposal 4: No definition on configured functionalities is needed.

Proposal 5: RAN2 to take the Figure 3 signaling procedure as baseline and further to discuss the signaling details of step 4 (i.e., network provides the NW-side additional condition to UE).

Proposal 6: Non-applicable functionalities reporting by UE to network is not supported, also no definition of available functionalities separately from applicable functionalities is needed.

Proposal 7: RAN2 to postpone the discussion on neighbouring cell’s applicable functionality reporting after more progress is made in RAN1 on NW-side additional condition.

- Functionality-based LCM for UE-sided model for BM use case

Functionality management

Proposal 8: For beam management use case for UE-sided model, RAN2 postpones the discussion of performance monitoring mechanism and signaling details until more progress is made in RAN1.

Model inference

Proposal 9: For beam management use case, RAN2 postpones the discussion for UE-sided model inference until more progress made in RAN1.

- References

- Chairman’s Notes RAN2#126, Fukuoka, Japan, May 20th – 24th, 2024

- [POST126][032][AIML PHY] LCM (Intel, Samsung) – phase1

- [POST126][032][AIML PHY] LCM (Intel, Samsung) – phase2

- R1-2310681 Reply LS on Data Collection Requirements and Assumptions RAN1