3GPP TSG-RAN WG2 Meeting #127	R2-2406830

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	CATT, Turkcell

Title:	Simulation results of intra-frequency RRM Measurement Prediction

Agenda Item:	8.3.2.1

Document for:	Discussion and Decision

Introduction

In RAN2#126 meeting, 3 scenarios have been agreed as high priority [1]:

Table 1: Prioritization of evaluation scenarios

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

And the following 3 sub cases were agreed in RAN2#125bis [2]:

1 For cell level measurement prediction model, at least consider the following cases: Case 1: To predict beam level results, then generate cell level results based on the predicted beam results; Case 2: To directly predict cell level results based on cell level results. Case 3: To directly predict cell level results based on beam level results

In this document, we focus on the 2 intra-frequency scenarios with high priority, i.e., Case 2 and Case 4, and the simulation results of cell level results prediction based on the cell level results are provided.

Discussion

Before the discussion on the simulation results, some clarification on the definition of Average RSRP Difference in Case 2/4 simulation is provided. In the simulation, one universal AI model addressing all UEs/cells is used, and the cell level RSRP (per UE per cell at each sample time) is used as AI inputs to predict the cell level RSRP (per UE per cell in the future time sample). Then, the predicated RSRP RSRP_Predicated (per UE per cell at the predicted sample time) and RSRP obtained from system simulation RSRP_Simulated (per UE per cell corresponding to the predicted sample time) are compared to derive the RSRP Difference. Finally, Average RSRP difference is calculated as the average absolute RSRP Difference. The formula is illustrated in Formula (1):

Average RSRP Difference=∑Total_Predicated_Sample(|RSRP_Predicted-RSRP_Simulated|)Total_Predicated_Sample

(1)

2.1	Case 4: FR2 to FR2 intra-frequency temporal domain case A

Temporal domain case A is mainly used to predict measurement result in the future of cell. Based on the predicted measurement result, some procedures, e.g., measurement event or RLF can also be predicated. One of the benefits of the predication is to improve the HO performance. For example, the target cell can be predicated in advance based on the predicated measurement result, which helps to facilitate network mobility control.

For AI model, long short-term memory (LSTM) based neural networks have played an important role in the field of Natural Language Processing. In addition, they have been used widely for sequence modeling. The reason why LSTMs have been used widely for this is because the model connects back to itself during a forward pass of the samples, and thus benefits from context generated by previous predications when predication for any new sample.

For Case 4, system level simulations are performed and we use LSTM model to predict the measurement result. The simulation results are summarized based the table agreed in RAN2#126 meeting which is shown in Table 2.

Table 2: Simulation report table for FR2 to FR2 intra-frequency temporal domain case A

| Report parameters |  | CATT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |  |  |  |  |  |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |  |  |  |  |  |  |  |
|  | UE speed (30,60,90,120 Km/h) | 60Km/h |  |  |  |  |  |  |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A |  |  |  |  |  |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A |  |  |  |  |  |  |  |
|  | Whether LOSsoft is modeled or not | Yes |  |  |  |  |  |  |  |
|  | spatial consistency option (A or B) | Procedure A |  |  |  |  |  |  |  |
|  | Number of TX beams | 64 |  |  |  |  |  |  |  |
|  | Number of RX beams | 8 |  |  |  |  |  |  |  |
|  | Measurement reduction rate(50%,…Note2) | N/A |  |  |  |  |  |  |  |
|  | Observation window(Note3) | 400ms | 800ms |  | 1200ms |  | 1600ms |  | 2000ms |  |
|  | Prediction window (Note3) | 400ms |  |  |  |  |  |  |  |
|  | Any other parameters (Note 4) | N/A |  |  |  |  |  |  |  |
| Data Size (Number of Samples) | Training/validity | ≈209000/ ≈23200 |  |  |  |  |  |  |  |
|  | Testing | ≈33800 |  |  |  |  |  |  |  |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP of all cells |  |  |  |  |  |  |  |
|  | Model output(Note 6) | Cell level RSRP of all cells |  |  |  |  |  |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |  |  |  |  |  |  |  |
|  | Model complexity in a number of parameters(M) | ≈1.26M |  |  |  |  |  |  |  |
|  | Model complexity in model size (e.g. Mbyte) | ≈15Mbytes |  |  |  |  |  |  |  |
|  | Computational complexity [FLOPs] | ≈890K |  |  |  |  |  |  |  |
| Metrics | Average L3 cell level RSRP difference (dBm) | 2.368 | 1.556 | 1.540 |  | 1.574 |  | 1.546 |  |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | N/A |  |  |  |  |  |  |  |

In order to show the trend of RSRP difference with different observation window /predication window length, the average L3 cell level RSRP difference (dB) is illustrated in Figure 1. In the following figures, one observation window unit (400ms) vs. one prediction window unit (400ms) is denoted as 1:1 and the same principle is applied to other observation window lengths. It can be seen that the average RSRP difference is around 1.5dB.

Observation 1: The average RSRP difference for FR2 to FR2 intra-frequency temporal domain case A prediction is around 1.5 dB.

Figure 1: RSRP difference with different observation/predication window length for AI prediction

In light of the Observation 1, we analyze the impacts of RSRP difference on the predication accuracy of best cell. And the accuracy of the predicted best cell is simulated and shown in Figure 2. It can be seen that:

- the trend of accuracy rate for best cells for different observation/predication window length grows as the observations window increases;

- the average accuracy rate for best cell is about 90% for predication window length 2, 3, 4, and 5;

- the trend of accuracy of the predicated best cell aligns with the trend of RSRP difference.

Observation 2: The trend of accuracy of the predicated best cell aligns with the trend of RSRP difference for FR2 to FR2 intra-frequency temporal domain case A.

Figure 2: Accuracy of the predicated best cell with different observation/predication window length for AI prediction

Another observation is that the RSRP difference does not decrease as the observation window length grows, but remains almost the same when the observation window length is equal or larger than 2. This shows that the input samples in the previous windows (i.e. window 3/4/5 in this simulation) contribute limited benefits to predication. One of the reasons may be the correlation between these samples decreases as the window length increases, considering the UE may move approximately 20 meters length during the 3 window units (3*400ms). Another reason may be the information of the nearest 2 window is sufficient for AI predication. Even though the prediction window length is enlarged, there is no obvious improvement on the gain of RSRP difference predication.

Observation 3: There is no obvious improvement on RSRP difference when the predication window length is larger than the some specific observation window length for FR2 to FR2 intra-frequency temporal domain case A.

Proposal 1: Capture Table 2 for FR2 to FR2 intra-frequency temporal domain case A into the TR.

2.1	Case 2: FR1 to FR1 intra-frequency temporal domain case B

Temporal domain case B can be used to predict the measurement result in the future time instance to reduce the measurement.

For Case B, the input are a series of measurement results (with a given measurement periodicity), whereas the output is the prediction of RSRP values at a point of time which does not and will not coincide with any measurement. LSTM seems not suitable for this task. ResNet is one residual learning framework to ease the training of the networks which can be used to reformulate the layers. For temporal domain case B, ResNet shows its good feature on prediction with different window lengths for predication window and observation window.

Table 3: Simulation report table for FR1 to FR1 intra-frequency temporal domain case B

| Report parameters |  | CATT |  |
| --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |  |
|  | UE speed (30,60,90,120 Km/h) | 90Km/h |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A |  |
|  | Whether LOSsoft is modeled or not | Yes |  |
|  | spatial consistency option (A or B) | Procedure A |  |
|  | Number of TX beams | 8 |  |
|  | Number of RX beams | 2 |  |
|  | Measurement reduction rate(50%,…Note2) | 1/2 | 2/3 |
|  | Observation window(Note3) | N/A |  |
|  | Prediction window (Note3) | N/A |  |
|  | Any other parameters (Note 4) | N/A |  |
| Data Size (Number of Samples) | Training/validity | ≈16000/ ≈2000 |  |
|  | Testing | ≈2000 |  |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP of all cells |  |
|  | Model output(Note 6) | Cell level RSRP of all cells |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | ResNet |  |
|  | Model complexity in a number of parameters(M) | 0.024M | 0.027M |
|  | Model complexity in model size (e.g. Mbyte) | 0.093Mbytes | 0.106Mbytes |
|  | Computational complexity [FLOPs] | 292.66K | 578.26K |
| Metrics | Average L3 cell level RSRP difference (dBm) | 0.7781 | 1.3343 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | N/A |  |

Based on the simulation results, it can be seen that when MRRT is 1/2, the RSRP difference is 0.7781 dB. When the MRRT increases to 2/3, the RSRP difference deteriorates to 1.3343 dB.

Observation 4: RSRP difference can be smaller than 1dB when MRRT = 1/2 for FR1 to FR1 intra-frequency temporal domain case B.

Observation 5: RSRP difference deteriorates when MRRT increases for FR1 to FR1 intra-frequency temporal domain case B.

Based on the simulation results and observations, it is proposed that:

Proposal 2: Capture Table 3 for FR1 to FR1 intra-frequency temporal domain case B into the TR.

Conclusion

In this document, we simulate Case2 and Case4 and provide these simulation results, and we have the following observations and proposls:

FR2 to FR2 intra-frequency temporal domain case A:

Observation 1: The average RSRP difference for FR2 to FR2 intra-frequency temporal domain case A prediction is around 1.5 dB.

Observation 2: The trend of accuracy of the predicated best cell aligns with the trend of RSRP difference for FR2 to FR2 intra-frequency temporal domain case A.

Observation 3: There is no obvious improvement on RSRP difference when the predication window length is larger than the some specific observation window length for FR2 to FR2 intra-frequency temporal domain case A.

Proposal 1: Capture Table 2 for FR2 to FR2 intra-frequency temporal domain case A into the TR.

FR1 to FR1 intra-frequency temporal domain case B:

Observation 4: RSRP difference can be smaller than 1dB when MRRT = 1/2 for FR1 to FR1 intra-frequency temporal domain case B.

Observation 5: RSRP difference deteriorates when MRRT increases for FR1 to FR1 intra-frequency temporal domain case B.

Proposal 2: Capture Table 3 for FR1 to FR1 intra-frequency temporal domain case B into the TR.

Reference

- R2-2406202, Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan;

- R2-2404102, Report of 3GPP TSG RAN WG2 meeting #125bis, Changsha, China;