3GPP TSG RAN WG2 #127                                                                                  R2-2406831

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	CATT, Turkcell

Title:	Simulation results of inter-frequency RRM Measurement Prediction

Agenda Item:	8.3.2.1

Document for:	Discussion and Decision

- Introduction

In R19 AI mobility, FR1 to FR1 frequency domain prediction is one of the high priority scenarios as agreed in 3GPP RAN2#126 meeting [1]:

Table 1: Prioritization of evaluation scenarios

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In the table above, the 1st goal of evaluation is to reduce measurement overhead, and the 2nd goal of evaluation is to enhance handover performance. And the scenario case 3 of frequency domain prediction is only used for saving measurement.

3 sub cases have been agreed in RAN2#125bis to consider for AI mobility study item:

1 For cell level measurement prediction model, at least consider the following cases: Case 1: To predict beam level results, then generate cell level results based on the predicted beam results; Case 2: To directly predict cell level results based on cell level results. Case 3: To directly predict cell level results based on beam level results

For FR1 to FR1 inter-frequency prediction, considering the sub cases, the frequency domain measurement prediction applies to both beam level and cell level. In this paper, we focus on the sub case 2 of cell level inter-frequency RRM measurement prediction. The corresponding simulation results are provided.

- Simulation evaluation

  - Frequency domain prediction w/t UE mobility

We simulated the FR1 to FR1 cell-level inter-frequency prediction with the second sub case, i.e., based on RSRP samples of multiple cells in one frequency to predict RSRP of the co-located cells in another frequency. System level simulations are performed first, and then we use ResNet model to predict the measurement result. One universal AI model for all the UEs/cells in the simulation is used. The simulation results are summarized in the table agreed in RAN2#126 meeting.

Since UE trajectory and speed are the important parameters in the general simulation assumption template, first we simulate with moving UE to fit the general simulation assumption.

Table 2: Simulation report table for FR1 to FR1 cell-level inter-frequency prediction

| Report parameters |  | CATT |  |
| --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |  |
|  | UE speed (30,60,90,120 Km/h) | 90Km/h |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | No |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | Full |  |
|  | Whether LOSsoft is modeled or not | Yes |  |
|  | spatial consistency option (A or B) | Procedure A |  |
|  | Number of TX beams | 8 |  |
|  | Number of RX beams | 2 |  |
|  | Measurement reduction rate(50%,…Note2) | N/A |  |
|  | Observation window(Note3) | N/A |  |
|  | Prediction window (Note3) | N/A |  |
|  | Any other parameters (Note 4) | N/A |  |
| Data Size (Number of Samples) | Training/validity | 13000/1625 |  |
|  | Testing | 1625 |  |
| AI/ML model input/output | Model input (Note 5) | Cell-level measured RSRP of all cells on 2G | Cell-level measured RSRP of all cells on 4G |
|  | Model output(Note 6) | Cell-level predicted RSRP of all cells on 4G | Cell-level predicted RSRP of all cells on 2G |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | ResNet |  |
|  | Model complexity in a number of parameters(M) | 0.017M |  |
|  | Model complexity in model size (e.g. Mbyte) | 0.065MB |  |
|  | Computational complexity [FLOPs] | 285.60K |  |
| Metrics | Average L3 cell level RSRP difference (dB) | 0.2056 | 0.1997 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | N/A |  |

The following figure shows the simulation results of inter-frequency AI/ML inference. We perform RSRP simulations of measured and predicted cells between 2G/4G. Aver-RSRP-Diff refers to the average value of the absolute RSRP difference of all cells corresponding to the UE (where the RSRP difference refers to the difference between the predicted RSRP value and the RSRP value obtained based on system simulation), and it’s formulated in (1):

Average RSRP Difference=∑Total_Predicated_Sample(|RSRP_Predicted-RSRP_Simulated|)Total_Predicated_Sample

(1)

Figure 1: CDF of RSRP-Diff for cell-level inter-frequency prediction

From the simulation results, we observe that similar performance (RSRP difference) can be achieved by RSRP prediction from 2G to 4G and from 4G to 2G.

Based on the simulation results in Table 2 and Figure 1, it can be seen that:

- the Average L3 cell level RSRP difference is around 0.2 dB;

- the proportion of sample points for RSRP difference below 1dB is higher than 97.8%.

This shows that the cell-level inter-frequency RRM measurement prediction has excellent performance, which proves that the AI/ML could achieve high prediction accuracy for cell-level inter-frequency RRM measurement prediction.

Observation 1: AI/ML could achieve high prediction accuracy regarding the cell-level inter-frequency RRM measurement prediction based on the simulation result.

Proposal 1: Capture Table 2 for the cell-level inter-frequency RRM measurement prediction with UE mobility into the TR.

  - Frequency domain prediction w/o UE mobility

Besides inter-frequency prediction with UE mobility, the prediction simulation results without UE mobility are also provided.

In this simulation assumption, the spatial factor is not considered, i.e. all the predicted results are assumed to be the measurement results in the same time instance.

Table 3: Simulation report table for FR1 to FR1 cell-level inter-frequency prediction without UE mobility

| Report parameters |  | CATT |  |
| --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | N/A |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | N/A |  |
|  | UE speed (30,60,90,120 Km/h) | 0 |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | No |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | Full |  |
|  | Whether LOSsoft is modeled or not | Yes |  |
|  | spatial consistency option (A or B) | N/A |  |
|  | Number of TX beams | 8 |  |
|  | Number of RX beams | 2 |  |
|  | Measurement reduction rate(50%,…Note2) | N/A |  |
|  | Observation window(Note3) | N/A |  |
|  | Prediction window (Note3) | N/A |  |
|  | Any other parameters (Note 4) | N/A |  |
| Data Size (Number of Samples) | Training/validity | 13000/1625 |  |
|  | Testing | 1625 |  |
| AI/ML model input/output | Model input (Note 5) | Cell-level measured RSRP of all cells on 2G | Cell-level measured RSRP of all cells on 4G |
|  | Model output(Note 6) | Cell-level predicted RSRP of all cells on 4G | Cell-level predicted RSRP of all cells on 2G |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | ResNet |  |
|  | Model complexity in a number of parameters(M) | 0.017M |  |
|  | Model complexity in model size (e.g. Mbyte) | 0.065MB |  |
|  | Computational complexity [FLOPs] | 285.60K |  |
| Metrics | Average L3 cell level RSRP difference (dB) | 0.2083 | 0.1963 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | N/A |  |

We list the comparison between simulation results with and without UE mobility in the table below and draw the CDF for the simulation results in Figure 2.

Table 4: Comparison between inter-frequency prediction results w or w/o UE mobility

| Set No. | Measured frequency | Prediction frequency | Moving UE | Aver-RSRP-Diff (dB) | Relative difference |
| --- | --- | --- | --- | --- | --- |
| 1 | 2G | 4G | yes | 0.2056 | 1.3% |
|  |  |  | no | 0.2083 |  |
| 2 | 4G | 2G | yes | 0.1997 | 1.7% |
|  |  |  | no | 0.1963 |  |

Figure 2: CDF of RSRP-Diff for cell-level inter-frequency prediction for UEs with and without mobility

Based on Aver-RSRP-Diff in Table 2, it can be seen that the relative difference of Aver-RSRP-Diff with and without UE mobility for inter-frequency prediction is below 2%, which is within the reasonable margin of error. And the CDF of RSRP-Diff for cell-level inter-frequency prediction for UEs with and without mobility almost overlaps.

Observation 2: The simulation result of inter-frequency prediction is almost the same between the cases with and without moving UE.

Proposal 2: Capture Table 3 for the cell-level inter-frequency RRM measurement prediction without UE mobility (i.e. stationary UE) into TR.

- Conclusion

In this contribution, we provide our simulation results on the FR1 to FR1 cell-level frequency domain prediction, and the observations and proposals are summarized as follows:

Observation 1: AI/ML could achieve high prediction accuracy regarding the cell-level inter-frequency RRM measurement prediction based on the simulation result.

Proposal 1: Capture Table 2 for the cell-level inter-frequency RRM measurement prediction with UE mobility into the TR.

Observation 2: The simulation result of inter-frequency prediction is almost the same between the cases with and without moving UE.

Proposal 2: Capture Table 3 for the cell-level inter-frequency RRM measurement prediction without UE mobility (i.e. stationary UE) into TR.

- References

- Chairman’s Notes RAN2#126, Fukuoka, Japan, May 20th – 24th, 2024