3GPP TSG-RAN WG2 Meeting #127	R2-2406832

Maastricht, Netherlands, Aug 19th – 23th, 2024

Source:	CATT, Turkcell

Title:	Simulation assumptions and evaluation methodology for RLF prediction

Agenda Item:	8.3.4.1

Document for:	Discussion and Decision

Introduction

In RAN2#126 meeting, the following agreements were made for RLF predication in AI mobility:

Agreements

1	Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR).

2	Study Direct: Directly RLF prediction by AI/ML models.

3	FR2 study will be prioritized for RLF prediction

4	The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction.

5	HOF prediction is downprioritized in our study.  NO simulations/evaluations should be done/submitted

6	RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case.  FFS on expected RLF time and indirect case.

7	No evaluation/simulations are expected for August meeting for RLF

8	Simulation assumption specific to RLF will be discussed in August.  The assumption is that we will reuse RRM simulation assumptions (where possible).

In this document, we continue to discuss the simulation assumptions and evaluation methodology for RLF prediction. Although it has been agreed both indirect and direct RLF prediction can be studied, we only focus on indirect prediction in this paper.

Discussion

2.1	Simulation methodology

- 2.1.1	RLF procedure

It has been agreed that the study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction. And based on the current specification, when the UE RRC receives N310 consecutive "out-of-sync" indications for the PCell from lower layers, the timer T310 for the corresponding PCell will be started. Once T310 expires, the UE RRC will declare RLF. On the other hand, if the UE RRC receives N311 consecutive "in-sync" indications for the PCell from lower layers while T310 is running, the UE shall stop T310. For indirect RLF prediction, it’s based on the serving cell measurement predictions in temporal domain. E.g., based on the predicted SINR, it can be evaluated whether T310 expires or T310 can be stopped when consecutive “in-sync” is received.

This procedure is illustrated in Figure 1a and Figure 1b:

Figure 1a: AI model prediction for “out-of-sync”

Figure 1b: AI model prediction for “in-sync”

Therefore, the whole RLF procedure can be simulated as following:

Proposal 1: The evaluation procedure for RLF can be summarized as:

- The UE evaluates whether N310 consecutive “out-of-sync” indications are received from physical layer;

- The UE starts T310 when N310 consecutive “out-of-sync” indications are received;

- If the UE receives N311 consecutive “in-sync” during T310, the UE stops T310; else if T310 expires, the UE declares RLF;

- The measurement result, e.g. SINR, can be predicted by AI model.

When T310 expires in MCG, the UE behaviors are specified as following which is highlighted in yellow:

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T310 | Upon detecting physical layer problems for the SpCell i.e. upon receiving N310 consecutive out-of-sync indications from lower layers. | Upon receiving N311 consecutive in-sync indications from lower layers for the SpCell, upon receiving RRCReconfiguration with reconfigurationWithSync for that cell group, upon reception of MobilityFromNRCommand, upon the reconfiguration of rlf-TimersAndConstant, upon initiating the connection re-establishment procedure, upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message including reconfigurationWithSync for that cell group, and upon initiating the MCG failure information procedure. Upon SCG release, if the T310 is kept in SCG. | If the T310 is kept in MCG: If AS security is not activated: go to RRC_IDLE else: initiate the MCG failure information procedure as specified in 5.7.3b or the connection re-establishment procedure as specified in 5.3.7 or the procedure as specified in 5.3.10.3 if any DAPS bearer is configured. If the T310 is kept in SCG, Inform E-UTRAN/NR about the SCG radio link failure by initiating the SCG failure information procedure as specified in 5.7.3. |

Regarding the simulation of RLF prediction, it is not necessary to follow these “after RLF” procedures. We can simply re-drop the UE when RLF happens.

Proposal 2: The UE will be re-dropped during simulation when RLF occurs.

- 2.1.2	HO procedure

Normally, when the UE moves to the edge of the cell, the signal strength of the neighbor cells may be stronger than the serving cell. Hence, HO can be performed to guarantee continuous data transmission. When the UE moves to the edge of the cell, RLF may also happen if late HO is performed. Thus, HO procedure affects whether RLF man happen in simulation. In our view, the following options can be considered for HO procedure.

[Option 1]: Simple HO procedure

This option is to simply simulate the legacy HO procedure to avoid the unnecessary RLF.

Event A3 can be used as start point for the HO procedure. I.e., the UE measures the neighbor cells and evaluates whether entry condition of A3 event is satisfied. If entry condition of A3 is fulfilled, the TTT can be started. When TTT expires, the UE can be switched to the target cell directly. There is no HOF, as RAN2 agreed “HOF prediction is downprioritized in our study”. In this option, RLF rate may be quite low as when UE moves to the cell edge HO may happen rather than RLF, which will bring enormous simulation work to “collect” sufficient RLFs.

[Option 2]: No HO procedure simulation

The RLF rate may be quite low when handover procedure is simulated. Considering huge amount of data (at least 10 thousands samples) is required for AI model training/verify/testing, this brings significant impact on simulation efficiency. Considering the goal of AI simulation for RLF is to check whether AI model can be used to predict the RLF accurately, mobility simulation without HO procedure can still be used.

[Option 3]: Adjusting RLF parameters with HO procedure

This is one compromised solution which is used to adjust the RLF parameters to enable “easy” RLF with HO procedure. For instance, the T310 is set one smaller value or the Qout/Qin is set to be a higher value. In this way, RLF can happen more easily and HO procedure is also modeled in mobility simulation. However, the RLF procedure in this option does not reflect the real RLF status either, since the values are intentional to trigger RLF more easily.

According to the analysis above, RAN2 can discuss which option is adopted for RLF simulation in AI mobility.

Proposal 3: RAN2 to discuss how to handle HO procedure in RLF simulation:

-	Option 1: Simple HO procedure;

-	Option 2: No HO procedure simulation

-	Option 3: Adjusting RLF parameters with HO procedure

2.2	Simulation assumptions

In Hetnet simulation captured in TR 36.839, the following values were assumed for RLF simulation:

Table 5.2.1.3.1: The parameters for determine the RLFs and the PDCCH failures. Items; Description. Qout; -8 dB. Qin; -6 dB. T310; 1s (the default value in 36.331). N310; 1. T311; Not used for calibration (since RLF recovery is not simulated in the calibration). N311; 1.

For simplicity, these values for RLF modeling can be used with small modification for RLF predication in AI mobility as staring point.

Proposal 4: The RLF parameter values from Hetnet mobility simulation can be used with small modification for RLF predication in AI mobility as starting point:

| Items | Description |
| --- | --- |
| Qout | -8 dB |
| Qin | -6 dB |
| T310 | 1s (the default value in 38.331) |
| N310 | 1 |
| T311 | Not used for calibration (since RLF recovery is not simulated in the calibration) |
| N311 | 1 |

It has been agreed that FR2 will be prioritized for RLF simulation. According to TS 38.133, it is specified that:

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB [ms] period becomes worse than the threshold Qout_SSB within TEvaluate_out_SSB [ms] evaluation period. UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB [ms] period becomes better than the threshold Qin_SSB within TEvaluate_in_SSB [ms] evaluation period.

Generally, the evaluation period can be derived according to Table 8.1.2.2-2 in TS 38.133.

For simplicity, we assume Evaluate_out_SSB = Max(200, Ceil(10  P  N)  TSSB) = 200ms and Evaluate_in_SSB = Max(100, Ceil(5  P  N)  TSSB) = 100ms. Therefore, it is proposed that :

Proposal 5: Evaluate_out_SSB is assumed to be 200ms and Evaluate_in_SSB is assumed to be 100ms for RLF procedure in AI mobility.

2.3	Parameters reported by company

Regarding the modeling inputs, SINR evaluated based on reference signals can be considered. But whether N310/T310 can be considered as AI model input highly depends on the AI model. From our perspective, these parameters can be left to company’s simulation implementation.

Proposal 6: SINR can be used as the input of the AI model for RLF simulation. Regarding N310/T310, it is up to company to decide whether to use these parameters as AI model input.

It has been agreed that the representative RLF case for direct and indirect predication both can be studied. Companies can choose whether direct or indirect case is applied for RLF simulation. But company can report which case is used.

Proposal 7: Company can report whether the RLF simulation is direct or indirect case for AI mobility.

The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction.

For indirect study, RLF prediction is based on the temporal domain serving cell measurement predictions. Similar mechanism like RRM measurement can be reused for KPI in indirect RLF predication as starting point, for example, the average SINR difference can be reported as outputs of the AI model by the companies.

Regarding the next step for indirect study, the expected RLF time and/or whether RLF happens within a time window or at a time instance can also be simulated. Companies can also report the accuracy of the AI model, e.g., the rate whether actual RLF happens within the predicted window or at the same time instance of predicted RLF.

Proposal 8: For indirect RLF predication, the following outputs can be reported by company:

- The average difference between actual SINR and predicated SINR;

- The rate whether real RLF happens within the predicted window or at the same time instance of predicted RLF.

It has been agreed that some parameters can be reported by companies, which has been concluded in Post email [POST126][031][AIMob] Simulations (OPPO). We think the similar table can be reused for RLF predication in AI mobility, but the metric part could be updated, it is proposed that:

Proposal 9: The table concluded in Post email [[POST126][031][AIMob] Simulations (OPPO)]with changes on metrics can be reported by companies for RLF prediction:

| Report parameters |  | Company A |
| --- | --- | --- |
| Metrics | Average SINR difference (dB) Or The rate whether real RLF happens within the predicted window or at the same time instance of predicted RLF. |  |

Conclusion

In this document, we further discuss the simulation assumptions and evaluation methodology for RLF prediction. And we propose:

Proposal 1: The evaluation procedure for RLF can be summarized as:

- The UE evaluates whether N310 consecutive “out-of-sync” indications are received from physical layer;

- The UE starts T310 when N310 consecutive “out-of-sync” indications are received;

- If the UE receives N311 consecutive “in-sync” during T310, the UE stops T310; else if T310 expires, the UE declares RLF;

- The measurement result, e.g. SINR, can be predicted by AI model.

Proposal 2: The UE will be re-dropped during simulation when RLF occurs.

Proposal 3: RAN2 to discuss how to handle HO procedure in RLF simulation:

-	Option 1: Simple HO procedure;

-	Option 2: No HO procedure simulation

-	Option 3: Adjusting RLF parameters with HO procedure

Proposal 4: The RLF parameter values from Hetnet mobility simulation can be used with small modification for RLF predication in AI mobility as starting point:

| Items | Description |
| --- | --- |
| Qout | -8 dB |
| Qin | -6 dB |
| T310 | 1s (the default value in 38.331) |
| N310 | 1 |
| T311 | Not used for calibration (since RLF recovery is not simulated in the calibration) |
| N311 | 1 |

Proposal 5: Evaluate_out_SSB is assumed to be 200ms and Evaluate_in_SSB is assumed to be 100ms for RLF procedure in AI mobility.

Proposal 6: SINR can be used as the input of the AI model for RLF simulation. Regarding N310/T310, it is up to company to decide whether to use these parameters as AI model input.

Proposal 7: Company can report whether the RLF simulation is direct or indirect case for AI mobility.

Proposal 8: For indirect RLF predication, the following outputs can be reported by company:

- The average difference between actual SINR and predicated SINR;

- The rate whether real RLF happens within the predicted window or at the same time instance of predicted RLF.

Proposal 9: The table concluded in Post email [[POST126][031][AIMob] Simulations (OPPO)]with changes on metrics can be reported by companies for RLF prediction:

| Report parameters |  | Company A |
| --- | --- | --- |
| Metrics | Average SINR difference (dB) Or The rate whether real RLF happens within the predicted window or at the same time instance of predicted RLF. |  |