3GPP TSG-RAN WG2 Meeting #127                                                        R2-2406833

Maastricht, NL, Aug 19th – 23rd, 2024

Source:              CATT

Title:	             Discussion on LS on intra-band EN-DC channel spacing

Agenda Item:	7.24.1

Document for:	Discussion and Decision

Introduction

According to RAN4 LS [1], “RAN4 respectfully asks RAN2 to introduce the capability to support intra-band non-contiguous EN-DC with nominal channel spacing”, as the capability of intraBandENDC-Support doesn’t “reflect the implementation UE can support”. In this paper, we further discuss whether a corresponding new UE capability is needed. And the companion CR and draft LS are also provided in [2] and [3].

Discussion

The content of RAN4 LS [1] is as follows:

In Rel-15, RAN4 sent the LS R4-1813862 to RAN2 for intra-band MR-DC band combination signaling, with the assumptions that if a UE supports intra-band non-contiguous EN-DC, it also supports contiguous in the same band. And then the capability of intraBandENDC-Support was introduced in TS 38.306 as follows, with three states, including: contiguous (not include), non-contiguous and ‘both’. However, from RAN4’s perspective, UE implementing ‘non-contiguous’ needs more stringent capability. In this case, if a UE supports ‘non-contiguous’, it should not preclude the support of ‘contiguous’ in the same band. The capability UE reports should reflect the implementation UE can support, rather than just for configurations. Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. intraBandENDC-Support Indicates whether the UE supports intra-band (NG)EN-DC with only non-contiguous spectrum, or with both contiguous and non-contiguous spectrum for the (NG)EN-DC combination as specified in TS 38.101-3 [4]. If the UE does not include this field for an intra-band (NG)EN-DC combination the UE only supports the contiguous spectrum for the intra-band (NG)EN-DC combination. If intrabandENDC-Support-UL is absent and the band combination supports intra-band (NG)EN-DC only in DL, this field indicates the DL capability. If intrabandENDC-Support-UL is absent and the band combination supports intra-band (NG)EN-DC in DL and UL, this field indicates the common capability for both DL and UL. If intrabandENDC-Support-UL is included, intraBandENDC-Support indicates the DL capability.; BC; No; N/A; N/A. With the UE capability, it is noticed that in the field, some UEs indicating ‘non-contiguous’ via intraBandENDC-Support cannot access to the NW configuring intra-band contiguous EN-DC. The UE can only access to single carrier rather than intra-band EN-DC, which degrades the UE performance. Considering the capability of intraBandENDC-Support has been implemented for several releases, to make up for the problem, RAN4 agrees to introduce a new capability to support intra-band non-contiguous with nominal channel spacing. In RAN4 spec, nominal channel spacing between E-UTRA carrier and an adjacent NR carrier is defined for intra-band contiguous EN-DC in TS 38.101-3.

According to R15 RAN4 LS [4], the most original requirement/assumption from RNA4 is if a UE supports intra-band non-contiguous EN-DC, it also supports contiguous spectrum within the same band.

2. Intra-band MR-DC band combination signalling RAN4 specifications have different requirements for case 1 and case2: Case 1: MR-DC band combination where the spectrum is contiguous across LTE and NR within the same band; Case 2: MR-DC band combination where both LTE and NR are in the same band, but not in one contiguous spectrum. UE architectures and capabilities vary based on the various case 1 and case 2 requirements. The network needs to be informed of the UE capability differences between case 1 and case2. The default configuration should be case 1 with the UE capability indicating that the UE supports case 2. RAN4 assumes that if a UE supports case 2 that it also supports case 1.

However, it’s specified/allowed currently that UE supports intra-band (NG)EN-DC with only non-contiguous spectrum by UE capability intraBandENDC-Support, which means that “some UEs indicating ‘non-contiguous’ via intraBandENDC-Support cannot access to the NW configuring intra-band contiguous EN-DC. The UE can only access to single carrier rather than intra-band EN-DC, which degrades UE performance.”

Observation 1: Legacy UE capability, such as intraBandENDC-Support, defines a case that UE supports intra-band (NG)EN-DC with only non-contiguous spectrum, which does not reflect the implementation UE can support. As the original RAN4 assumption is that if a UE supports case 2 (non-contiguous spectrum) it also supports case 1 (contiguous spectrum).

In order to solve this problem, two options could be discussed:

- Option 1, based on [1], RAN4 suggests to introduce new UE capability to support intra-band non-contiguous with nominal channel spacing. Therefore, the combination of the new UE capability and intraBandENDC-Support (with “non-contiguous” only) means to support both contiguous and non-contiguous spectrum for the intra-band EN-DC scenario. And UE still supports only non-contiguous spectrum for intra-band EN-DC if this new capability is absent.

- Option 2, update the field description of intraBandENDC-Support from R18, i.e., add the restriction that the UE only indicates ‘both’ if this field is indicated. In this way, it still aligns with RAN4 intention since R18 and future UEs don’t indicate ‘non-contiguous only’. And in the meantime, if a legacy UE (pre-R18) indicates ‘non-contiguous only’ for intraBandENDC-Support, the network can still treat it as if it “only” supports non-contiguous spectrum for intra-band EN-DC.

Considering the spec impact introduced by these two options, we notice that this “new UE capability” actually needs to involve multiple relevant UE capabilities as below, i.e., intrabandENDC-Support-UL, intraBandENDC-Support-v1790 and intraBandENDC-Support-UL-v1790, which are all associated to this UE feature.

SupportedIntraENDC-BandCombination-r17 ::= SEQUENCE { supportedBandwidthCombinationSetIntraENDC-v1790 BIT STRING (SIZE (1..32)) OPTIONAL, mrdc-Parameters-v1790 MRDC-Parameters-v1790 OPTIONAL } MRDC-Parameters-v1790 ::= SEQUENCE { intraBandENDC-Support-v1790 ENUMERATED {non-contiguous, both} OPTIONAL, intraBandENDC-Support-UL-v1790 ENUMERATED {non-contiguous, both} OPTIONAL } MRDC-Parameters-v15n0 ::= SEQUENCE { intraBandENDC-Support-UL ENUMERATED {non-contiguous, both} OPTIONAL }

If option 1 is selected, four new capabilities need to be introduced, while the new capabilities mapping to intraBandENDC-Support-v1790 and intraBandENDC-Support-UL-v1790 also need to follow the IE structure for inter-band (NG)EN-DC band combination with multiple intra-band (NG)EN-DC components. It can be seen that the signaling design of option 1 is more complex. And if we go for option 2, as long as UE vendors can confirm this understanding is correct, no new UE capabilities need to be introduced. In this case the signaling of option 2 is also more concise.

Furthermore, there is no NBC issue with option 2, as the network could understand the “both” capability of R18 UE and the “non-contiguous only” capability of legacy UEs (if any) correctly. So it is preferable to adopt option 2 to align with RAN4 LS’s intention. A draft CR is also provided in [x].

Observation 2: there is no NBC issue with option 2, as the network could understand the “both” capability of R18 UE and the “non-contiguous only” capability of legacy UEs (if any) correctly.

Proposal 1: Instead of introducing new UE capabilities, update the field descriptions of the following UE capabilities from R18, including intraBandENDC-Support, intrabandENDC-Support-UL, intraBandENDC-Support-v1790 and intraBandENDC-Support-UL-v1790, to add restriction that a UE only indicates ‘both’ if the corresponding UE capability is supported.

If the proposal above can be agreed, a reply LS needs to be sent back to RAN4 to explain RAN2 approach. The draft LS reply is also provided in [y].

Proposal 2: Send a reply LS to R4 and explain RAN2 approach.

Conclusion

According to the analysis in section 2, it is observed that:

Observation 1: Legacy UE capability, such as intraBandENDC-Support, defines a case that UE supports intra-band (NG)EN-DC with only non-contiguous spectrum, which does not reflect the implementation UE can support. As the original RAN4 assumption is that if a UE supports case 2 (non-contiguous spectrum) it also supports case 1 (contiguous spectrum).

Observation 2: there is no NBC issue with option 2, as the network could understand the “both” capability of R18 UE and the “non-contiguous only” capability of legacy UEs (if any) correctly.

And we propose:

Proposal 1: Instead of introducing new UE capabilities, update the field descriptions of the following UE capabilities from R18, including intraBandENDC-Support, intrabandENDC-Support-UL, intraBandENDC-Support-v1790 and intraBandENDC-Support-UL-v1790, to add restriction that a UE only indicates ‘both’ if the corresponding UE capability is supported.

Proposal 2: Send a reply LS to R4 and explain RAN2 approach.

Reference

- R4-2410693, LS on intra-band EN-DC channel spacing, To: RAN2

- R2-2406834, Capability update for intraBandENDC-Support, CATT

- R2-2406835, DRAFT Reply LS on intra-band EN-DC channel spacing, To: RAN4

- R4-1813862, Reply LS on intra-band combination for NR CA and MR-DC, To: RAN2