3GPP RAN WG2 Meeting #127	R2-2406838

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	CATT

Title:	Discussion on missing PRACH SCS configuration

Agenda Item:	6.1.3.1

Document for:	Discussion

- Introduction

This document analyses an error in the presence conditions of RA preamble SCS (i.e. PRACH SCS, orΔfRA in RAN1 specs).

- Discussion

  - Rel-15/16 background

The SCS of RA preambles can often be different from the SCS for other UL channels, e.g. PUCCH/PUSCH SCS.

In Rel-15 NR (and in E-UTRA), there are two types of RA preambles: one with sequence length L=839, and the other one with L=139. There are multiple possible values of RA preamble SCS for both preamble types, as specified in (then-version of) TS 38.211:

Table 6.3.3.1-1: PRACH preamble formats for and ΔfRA∈1.25, 5 kHz. Format; Support for restricted sets. 0; 839; 1.25 kHz; 24576κ; 3168κ; Type A, Type B. 1; 839; 1.25 kHz; 2⋅24576κ; 21024κ; Type A, Type B. 2; 839; 1.25 kHz; 4⋅24576κ; 4688κ; Type A, Type B. 3; 839; 5 kHz; 4⋅6144κ; 3168κ; Type A, Type B. Table 6.3.3.1-2: Preamble formats for and ΔfRA=15⋅2μ kHz where . Format; Support for restricted sets. A1; 139; -. A2; 139; -. A3; 139; -. B1; 139; -. B2; 139; -. B3; 139; -. B4; 139; -. C0; 139; -. C2; 139.

Rel-15 version of TS 38.211

But how to deduce the preamble SCS value was different:

- For L=839 the preamble SCS can be deduced from the PRACH format, which is in turn indicated by the PRACH configuration index signalled by RRC.

- For L=139 the preamble SCS cannot be deduced so and thus has to be indicated by RRC explicitly.

As the result, RAN2 introduced an IE with condition “L139”, specifying that it must be present if L=139 and must be absent if L=839:

msg1-SubcarrierSpacing SubcarrierSpacing OPTIONAL, -- Cond L139 msg1-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2). Only the values 15 or 30 kHz (FR1), and 60 or 120 kHz (FR2) are applicable. If absent, the UE applies the SCS as derived from the prach-ConfigurationIndex in RACH-ConfigGeneric (see tables Table 6.3.3.1-1 and Table 6.3.3.2-2, TS 38.211 [16]). The value also applies to contention free random access (RACH-ConfigDedicated), to SI-request and to contention-based beam failure recovery (CB-BFR). But it does not apply for contention free beam failure recovery (CF-BFR) (see BeamFailureRecoveryConfig).. L139; The field is mandatory present if prach-RootSequenceIndex L=139, otherwise the field is absent, Need S..

Rel-15 version of TS 38.331

In Rel-16 NR, two more sequence lengths were introduced: L=571 and L=1151. Their use was limited for NR-Unlicensed (which can only occur in FR1) and the preamble SCS were fixed:

Table 6.3.3.1-2: Preamble formats for LRA∈139, 571, 1151 and ΔfRA=15⋅2μ kHz where μ∈0,1,2,3. Format; ΔfRA; Support for restricted sets. μ∈0,1,2,3; μ=0; μ=1. A1; 139; 1151; 571; -. A2; 139; 1151; 571; -. A3; 139; 1151; 571; -. B1; 139; 1151; 571; -. B2; 139; 1151; 571; -. B3; 139; 1151; 571; -. B4; 139; 1151; 571; -. C0; 139; 1151; 571; -. C2; 139; 1151; 571.

Rel-16 version of TS 38.211

Since the SCS for the two new lengths were fixed as 30kHz and 15kHz respectively, the presence conditions of the fields indicating preamble SCS were kept unchanged in TS 38.331. (The field description was changed anyway, adding Table 6.3.3.1-2 as reference.)

  - How the problem occurred

In Rel-17 NR, the highest available frequency was extended to 71GHz, and more available SCS values were introduced with L=139, 571 and 1151 for FR2-2:

msg1-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2). Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz If absent, the UE applies the SCS as derived from the prach-ConfigurationIndex in RACH-ConfigGeneric (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]). The value also applies to contention free random access (RACH-ConfigDedicated), to SI-request and to contention-based beam failure recovery (CB-BFR). But it does not apply for contention free beam failure recovery (CF-BFR) (see BeamFailureRecoveryConfig).. prach-RootSequenceIndex PRACH root sequence index (see TS 38.211 [16], clause 6.3.3.1). The value range depends on whether L=839 or L=139 or L=571 or L=1151. The length of the root sequence corresponding with the index indicated in this IE should be consistent with the one indicated in prach-ConfigurationIndex in the RACH-ConfigDedicated (if configured). If prach-RootSequenceIndex-r16 is signalled, UE shall ignore the prach-RootSequenceIndex (without suffix). For FR2-2, only the following values are applicable depending on the used subcarrier spacing: 120 kHz: L=139, L=571, and L=1151 480 kHz: L=139, and L=571 960 kHz: L=139.

Rel-17 version of TS 38.331

Table 6.3.3.1-2: Preamble formats for LRA∈139, 571, 1151 and ΔfRA=15⋅2μ kHz where μ∈0,1,2,3,5,6. Format; ΔfRA; Support for restricted sets. μ∈0,1,2,3,5,6; μ∈0,3; μ∈1,3, 5. A1; 139; 1151; 571; -. A2; 139; 1151; 571; -. A3; 139; 1151; 571; -. B1; 139; 1151; 571; -. B2; 139; 1151; 571; -. B3; 139; 1151; 571; -. B4; 139; 1151; 571; -. C0; 139; 1151; 571; -. C2; 139; 1151; 571.

Rel-17 version of TS 38.211

For convenience, available values for L=139, 571 and 1151 in each FR are listed below (mainly summarised from the field descriptions shown above):

|  | FR1 | FR2-1 | FR2-2 |
| --- | --- | --- | --- |
| L=139 | 15kHz, 30kHz | 60kHz, 120kHz | 120kHz, 480kHz, 960kHz |
| L=571 | 30kHz | N/A (see in Section 5.3.2) | 120kHz, 480kHz |
| L=1151 | 15kHz | N/A (see in Section 5.3.2) | 120kHz |

It can be clearly seen that there are two available values of RA preamble SCS for L=571 in FR2-2. Technically speaking, it needs to be indicated explicitly. But the current presence condition of RA preamble SCS configuration blocks this explicit configuration, i.e., “The field is mandatory present if prach-RootSequenceIndex L=139, otherwise the field is absent, Need S”.

Observation 1: There are two possible values of RA preamble SCS for L=571 in FR2-2, which can’t be derived by UE, but needs to be indicated explicitly by network.

Observation 2: The current presence condition of RA preamble SCS configuration blocks this explicit configuration for L=571 in FR2-2, i.e., “The field is mandatory present if prach-RootSequenceIndex L=139, otherwise the field is absent, Need S”.

Therefore, the presence conditions of the RA preamble SCS configuration should be corrected for L=571 in FR2-2.

Proposal 1: Update the presence conditions of the RA preamble SCS configuration to make them mandatory present for L=571 in FR2-2.

  - What fields to revise

The problem only occurs if L=571 in FR2-2 is applicable. There are two such configuration fields in Rel-17:

- msg1-SubcarrierSpacing in RACH-ConfigCommon;

- msgA-SubcarrierSpacing in RACH-ConfigCommonTwoStepRA.

There is one more configuration field in Rel-18:

- msg1-SubcarrierSpacing in RACH-ConfigTwoTA.

So both the Rel-17 CR and the Rel-18 CR are Cat-F ones.

- Conclusion

Observation 1: There are two possible values of RA preamble SCS for L=571 in FR2-2, which can’t be derived by UE, but needs to be indicated explicitly by network.

Observation 2: The current presence condition of RA preamble SCS configuration blocks this explicit configuration for L=571 in FR2-2, i.e., “The field is mandatory present if prach-RootSequenceIndex L=139, otherwise the field is absent, Need S”.

Proposal 1: Update the presence conditions of the RA preamble SCS configuration to make them mandatory present for L=571 in FR2-2.

Two companion CRs are provided according to the proposals above [1][2].

- Reference

[1] R2-2406839, Correction on missing PRACH SCS configuration, CATT

[2] R2-2406840, Correction on missing PRACH SCS configuration, CATT