3GPP TSG-RAN WG2 Meeting #127	R2-2406845

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	7.25.1

Source:	Nokia

Title:	On the Inconsistency between extended k1 range and DL-DataToUL-ACK-v1700 (RAN4 LS)

WID/SID:	NR_ATG-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN4 is diligently correcting the Rel-18 Air-To-Ground (ATG) related specification. As a result, the LS to RAN1 and RAN2 has been sent in [1]. In this paper we are discussing the actions required in RAN2 considering the feedback shared in [1].

# 2	Discussion

RAN4 colleagues in their LS [1] are concerned about the range defined in NR RRC, corresponding to k1 parameter. More specifically, it is claimed that k1-RangeExtensionATG-r18 and k1-RangeExtension-r17 should have the same range which is indicated by NR’s parameter DL-DataToUL-ACK-v1700 [2]. The range of DL-DataToUL-ACK-v1700 is currently just from 16 to 31 (see below) while RAN4 in [1] indicates it should be from 0 to 31.

Observation 1: RAN4 in R2-2409974 indicates the range for DL-DataToUL-ACK should be defined as (0..31), not as 16..31).

However, when inspecting the latest version of NR RRC [2] the following can be found:

PUCCH-ResourceGroupId-r16 ::= INTEGER (0..maxNrofPUCCH-ResourceGroups-1-r16) DL-DataToUL-ACK-r16 ::= SEQUENCE (SIZE (1..8)) OF INTEGER (-1..15) DL-DataToUL-ACK-r17 ::= SEQUENCE (SIZE (1..8)) OF INTEGER (-1..127) DL-DataToUL-ACK-v1700 ::= SEQUENCE (SIZE (1..8)) OF INTEGER (16..31) DL-DataToUL-ACK-r18 ::= SEQUENCE (SIZE (1..8)) OF INTEGER (0..31) DL-DataToUL-ACK-DCI-1-2-r16 ::= SEQUENCE (SIZE (1..8)) OF INTEGER (0..15) DL-DataToUL-ACK-DCI-1-2-r17 ::= SEQUENCE (SIZE (1..8)) OF INTEGER (0..127) DL-DataToUL-ACK-DCI-1-2-r18 ::= SEQUENCE (SIZE (1..8)) OF INTEGER (0..31) UL-AccessConfigListDCI-1-1-r16 ::= SEQUENCE (SIZE (1..16)) OF INTEGER (0..15) UL-AccessConfigListDCI-1-2-r17 ::= SEQUENCE (SIZE (1..16)) OF INTEGER (0..15) UL-AccessConfigListDCI-1-1-r17 ::= SEQUENCE (SIZE (1..3)) OF INTEGER (0..2) DL-DataToUL-ACK-MulticastDCI-Format4-1-r17 ::= SEQUENCE (SIZE (1..8)) OF INTEGER (0..15) -- TAG-PUCCH-CONFIG-STOP -- ASN1STOP

PUCCH-Config field descriptions

dl-DataToUL-ACK, dl-DataToUL-ACK-DCI-1-2 List of timing for given PDSCH to the DL ACK (see TS 38.213 [13], clause 9.1.2). The field dl-DataToUL-ACK applies to DCI format 1_1 and the field dl-DataToUL-ACK-DCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.2.3). The dl-DataToUL-ACK-v1700 is applicable for NTN and dl-DataToUL-ACK-r17 is applicable for up to 71 GHz. The dl-DataToUL-ACK-r18 is applicable for ATG. If dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-r17 or dl-DataToUL-ACK-v1700 or dl-DataToUL-ACK-r18 is signalled, UE shall ignore the dl-DataToUL-ACK (without suffix). The value -1 corresponds to "inapplicable value" for the case where the A/N feedback timing is not explicitly included at the time of scheduling PDSCH. The fields dl-DataToUL-ACK-r17 and dl-DataToUL-ACK-DCI-1-2-r17 are only applicable for SCS of 480 kHz or 960 kHz. The field dl-DataToUL-ACK-r18 applies to DCI format 1_1 and the field dl-DataToUL-ACK-DCI-1-2-r18 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.2.3).

As can be noticed above, RAN2 has already taken into account the conclusions reached in RAN4 and defined dl-DataToUL-ACK-r18 in the following way [2]:

DL-DataToUL-ACK-r18 ::=                  SEQUENCE (SIZE (1..8)) OF INTEGER (0..31)

Thus, the range of k1 for ATG seems to be already correct. Nevertheless, RAN4 in [1] indicated that the inappropriate range is applicable to both ATG (Release 18) and NTN (Release 17). As a consequence, the correction introduced in [2] might be insufficient.

Observation 2: According to the field description, dl-DataToUL-ACK-r18 with extended range (0..31) applies just to Release 18 ATG while the RAN4 LS (R4-2409974) indicates the range should be also extended for Release 17 NTN.

Assuming the Observation 2 is correct, RAN2 needs to make a Release 17 correction and e.g. add another v17xy instance of DL-DataToUL-ACK to cover the missing range of (0..15).

Proposal 1: RAN2 discusses how to increase the k1 range for Release 17 NTN, e.g. via introducing the v17xy instance of DL-DataToUL-ACK to cover the missing range of (0..15).

Once the conclusion on the above is reached, RAN2 responds to RAN4 LS. The exemplary LS response can be found in Annex A.

Proposal 2: Take the draft LS response from Annex A as a starting point for responding to R4-2409974.

# 3	Conclusion

This document has made the following observations:

Observation 1: RAN4 in R2-2409974 indicates the range for DL-DataToUL-ACK should be defined as (0..31), not as 16..31).

Observation 2: According to the field description, dl-DataToUL-ACK-r18 with extended range (0..31) applies just to Release 18 ATG while the RAN4 LS (R4-2409974) indicates the range should be also extended for Release 17 NTN.

And proposed the following:

Proposal 1: RAN2 discusses how to increase the k1 range for Release 17 NTN, e.g. via introducing the v17xy instance of DL-DataToUL-ACK to cover the missing range of (0..15).

Proposal 2: Take the draft LS response from Annex A as a starting point for responding to R4-2409974.

# References

- R4-2409974	LS on inconsistent issue between extended k1 range and RRC parameter DL-DataToUL-ACK-v1700 for R17 NTN 3GPP TSG-RAN WG4 Meeting # 111 Fukuoka , Japan, 20th – 24th May 2024

- 3GPP TS 38.331 	NR RRC v18.2.0, July 2024

# Annex A – Draft LS Response

3GPP TSG-RAN WG2 Meeting #127	R2-240xxxx

Maastricht, Netherlands, 19 – 23 August 2024

Title:	[DRAFT] Response LS on inconsistent issue between extended k1 range and RRC parameter DL-DataToUL-ACK-v1700 for R17 NTN

Response to:	R4-2409974

Release:	Release 18

Work Item:	NR_ATG

Source:	TSG RAN WG2

To:	TSG RAN WG4

Cc:	TSG RAN WG1

Contact Person:

#### Name:	Jedrzej Stanczak

E-mail Address:	jedrzej.stanczak@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 would like to thank RAN4 for their LS in R4-2409974. RAN2 would like to inform RAN4 that k1 range for Release 18 ATG has been already extended in V18.2.0 of TS 38.331. However, RAN2 is still discussing how to ensure similar k1 range is supported for Release 17 NTN.

2. Actions:

To RAN4 group.

ACTION: 	RAN2 respectfully asks RAN4 to take the aforementioned feedback into account.

3. Date of Next TSG-RAN WG2 Meeting:

RAN2#127bis	from 2024-10-14	to 2024-10-18		Hefei, China

RAN2#128	from 2024-11-18	to 2024-11-22		Orlando, USA