3GPP TSG-RAN WG2 Meeting #127	R2-2406849

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.8.4

Source:	Nokia, Nokia Shanghai Bell

Title:	On How To Support MBS in Rel-19 NR NTN

WID/SID:	NR_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In December 2023, RAN approved a Rel-19 work item (WI) aimed at defining Phase 3 enhancements to Non-Terrestrial Networks (NTN). There was a minor revision of the WID at RAN#103. Among the objectives captured in the WID [1] the following can be found:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

In RAN2#125bis meeting, the first agreements on the topic were made and these were as follows [2]:

Agreements: - For MBS broadcast service we don’t restrict the work to any satellite constellation type - We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS - We will cover at least the case where the indicated intended service area covers a portion of a NTN cell - The intended service area can cover the area of more than one NTN cells (or portions thereof) - Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended serive area) - At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered): - Circles (like for TN coverage) - Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

The topic was subsequently discussed in RAN2#126 with the following decisions made [3]:

Agreements: 1. For MBS broadcast service, both EFC and EMC are supported. 2. RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area. 3. MBS broadcast intended service area is provided via system information 4. For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS 5. When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

Additionally, the following two aspects have been labelled as “come back next meeting” [3]:

- Come back next meeting to check whether a UE may not (or should not) establish MRB(s) associated with a service limited to an intended service area when it is not located within the area (up to UE implementation how the UE detects it’s not in the intended service area). - Come back next meeting to check whether a UE may (or should) release its established MRB(s) associated with a service limited to an intended service area when exiting the area (up to UE implementation how the UE detects it’s not in the intended service area).

In this paper we present our further view on what is to be addressed for this Rel-19 objective in RAN2.

# 2	Discussion

## 2.1	Support for emergency broadcasting services

For emergency broadcasting services, the following was initially agreed [2]:

RAN2#125bis agreements - We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS

NTN can support much greater coverage area compared to terrestrial network (TN) due to much larger beam footprint (even with LEO satellite constellation, not to mention GEO). Capability to support multi-beam layout enhances the geographical coverage zone of the tracking area of the satellite. As a result, NTN can provide connectivity to large number of UEs with minimal terrestrial network coverage. Therefore, dissemination of public warning system like ETWS to larger number of UEs can be an attractive use case for broadcasting over NTN.

However, ETWS message contains two parts, the primary and the secondary warning messages, the primary notification message and the secondary notification message. As per TS 22.268, which provides the requirements for the public warning system, the following points about ETWS can be found in the specification:

Duration of the delivery time for PLMN operators is the time from the receipt of the Warning Notification by the PLMN operator, i.e. the edge of the 3GPP network, to the time that the Warning Notification is successfully delivered to the UEs.

Provisioning of delivery of Primary and Secondary Notification may be required:

-	Primary Notification shall be delivered within 4 seconds to the UE in the Notification Area even under congestion situation.

-	Secondary Notification is delivered to the users in the Notification Area even under congestion situation.

Note 1: 	UEs that are out of coverage or switched off are not considered in the requirements.

Note 2: 	Secondary Notification may not always be generated as it depends on the Warning Notification Provider’s Primary Notification shall:

-	convey data which is small enough to be sent quickly on the network.

-	convey small amount of data to indicate the imminent occurrence of Earthquake and Tsunami, etc.

Secondary Notification may:

-	convey a large amount of data in order to deliver text, audio to instruct what to do / where to get help, graphical data such as a map indicating the route from present position to evacuation site, time table of food distribution.

policy.

Note 3: 	Primary Notification may not always be generated (i.e. the warning may start with a Secondary Notification).

Since the primary notification message is the critical part of the ETWS message, broadcast of ETWS over NTN should focus predominantly on the primary notification message of ETWS, if ETWS is to be supported.

Proposal 1:  RAN2 to confirm that for ETWS message dissemination through broadcast over NTN (if supported), initially will focus on primary notification message.

## 2.2	Area contention of the broadcasting services

The 3GPP contribution in [5] contained the following proposal on the potential scenarios to be supported:

R2-2402355 Proposal 1: RAN2 discusses and confirms the intended scenarios to be supported for MBS broadcast service via NR NTN in Rel-19: - S1: The service area is a portion of an NTN cell, and the Content of the service can only be received within the service area (e.g. broadcast via a dedicated beam(s) ). - S2: The service area is a portion of an NTN cell, but the content of the service can be received within the whole cell. - S3: The intended service area consists of a list of NTN cell(s)/tracking area(s).

The proposal was discussed during RAN2#125bis meeting resulting with the following outcome [2]:

RAN2#125bis agreements: - We will cover at least the case where the indicated intended service area covers a portion of a NTN cell - The intended service area can cover the area of more than one NTN cells (or portions thereof) - Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended serive area)

During RAN2#126 it was additionally agreed that:

2. RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

Thus, scenario 1 from [5] has been excluded.

Observation 1: The decision taken at RAN2#126 excludes scenario 1 (The service area is a portion of an NTN cell, and the Content of the service can only be received within the service area) from further considerations in Rel-19.

However, it remains open what shall be the UE’s behaviour when it is located outside of the service area, but the MBS can be received there. Namely:

- Can the UE establish the MRBs?

- Shall the UE discard the existing MRBs when it leaves the service area?

In addition, if the UE shall perform the actions listed above, it needs to be clarified how the UE determines it has left the service area.

Proposal 2: RAN2 needs to discuss the following:

- Whether the UE can establish MRBs when it is outside of the service area, but the MBS can be received there

- Whether the UE shall discard MRBs when it leaves the service area, but the MBS can be received there

- How the UE determines it has left the service area

Regarding a) Whether the UE can establish MRBs when it is outside of the service area, but the MBS can be received there, we believe it is not the best idea to leave the decision entirely to the UE’s implementation (as argued during the RAN2#126 by some companies) while the NW has no control or knowledge on whether the UE has done it. In general, we think the UE should be allowed to establish the MRBs based on  not just if it can receive the MBS, but also if the UE’s location is within  the intended service area.

Proposal 3: The UE is allowed to establish MRBs if it can receive broadcast service and if its location is within the MBS service area.

Regarding b) Whether the UE shall discard MRBs when it leaves the service area, but the MBS can be received there, we think here the approach might be a bit more flexible. The UE does not have to discard everything immediately when it is at the MBS service area border, but there might be conditions which are checked to determine if the UE discards those MRBs or may maintain them. The condition can be related to the time or distance that has elapsed since the leaving service area. Alternatively, it might be associated with the next MBS service area which might be providing the same set of services. In such circumstances, deleting the MRBs might be unnecessary.

Proposal 4: Discarding MRBs might be subject to additional conditions, such as the time or distance that has elapsed since leaving the MBS area. Another option is to consider what services are provided in the neighbouring MBS service areas.

Regarding c) How the UE determines it has left the service area, we acknowledge that it requires new means when compared to MBS in terrestrial networks. Nevertheless, it is expected the NTN-capable UE will monitor its geo-position, so it should be able to realize it has left or entered the MBS service area.

Proposal 5: NTN-capable UE realizes it is within or outside of MBS service area based on the GNSS it is using for other NTN purposes.

In MBS services, the content is usually limited to specific UEs at application layer (i.e., not all UEs in the service area are authorized to receive the MBS data). There can be also a core network level encryption, i.e., if MBSF is deployed in the core network. The UE’s capability to receive broadcast services depend on two factors. The first one is whether the UE is present in the service area and the second one is whether the UE has the required subscription to receive the broadcast data (e.g., TV service). In case of MBSF deployments, the UE should run a separate client to be able to communicate with the MBSF. Also, the service content is encrypted, which means that a security functionality is required to provision a session security context. The subscription management and the security provisioning are handled at application layer and both are transparent to RAN and radio protocols to avoid any impact on RAN specifications and facilitate a higher deployment flexibility to network operators.

Proposal 6: RAN2 to further explore whether scenario S2 requires specification changes.

Finally, the third scenario refers to the simplest approach which is to consider a single cell as the smallest possible area which maps to broadcast service area. This approach may require minimal specification efforts. However, especially in case of GEO (GSO) satellites the footprint could be very large, e.g. spanning over 200 kilometres so hard to imagine there could be an MBS service area which needs to be that large.

Observation 2: The footprint of GEO satellites could stretch over hundreds of kilometres which is likely much larger than any MBS service area needs to be.

RAN2 needs to decide if the issue described in Observation x is a valid concern. Namely, if the fact that MBS service area is larger than intended area where MBS should be available should be addressed somehow.

Proposal 7: RAN2 is asked to decide if it is problematic when MBS service area is larger than intended area where MBS should be available.

## 2.3	Supporting MBS in Earth-moving deployments

During RAN2#126 the following has been agreed [3]:

1. For MBS broadcast service, both EFC and EMC are supported.

This is in line with the objective captured in the WID [1], wherein the aim is to provide broadcast services despite the satellite constellation and the NTN cell deployment. However, this raises a new problem for MBS signalling when satellites in NTN system are in continuous motion. For example in EMC the footprint will be continuously moving, and a particular satellite will be able to provide MBS just temporarily in a certain area.

Observation 3: MBS service will be provided by certain satellite only during limited period (i.e. when satellite’s footprint overlaps with the MBS area).

Thus, cell identifiers associated with the MBS service area will change over time.

Observation 4: MBS service area provided by NTN will be covered by cells whose identifiers change over time.

On the other hand, current MBS framework was designed considering earth-fixed cells without moving gNBs. For example, a broadcast session that is initiated from an application function is forwarded to the gNBs in the MBS service area. More specifically, AMF picks the gNBs that reside in the service area of the MBS session and sends Broadcast Setup Request only to those gNBs.

Observation 5: In legacy MBS design AMF selects gNBs that are in the service area corresponding to MBS session.

However, currently there is no mechanism for moving cells, e.g., a gNB coming into an MBS service area cannot dynamically receive/release Broadcast Session. If RAN2 agrees there is a need to support such scenario, we shall also let RAN3 know there is a need to define a corresponding network signalling.

Proposal 8: As RAN2 decided to support MBS via EMC in NTN, RAN2 sends the LS to RAN3 asking this WG to define corresponding NW signalling.

## 2.4	SIB or MCCH messages for signalling

At RAN2#126 (May 2024) the following has been agreed [3]:

For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

According to the MBS description in [4], the MBS configuration information is carried on MCCH logical channel. It comprises the information on the MBS broadcast sessions that are provided, related scheduling information and it can also contain the list of neighbour cells where the same broadcast MBS services are provided.

Observation 6: MCCH provides the list of MBS broadcast sessions, scheduling information, and the list of neighbours with the same MBS services.

In order to receive MCCH, the UE needs to acquire appropriate information from SIB1 and SIB20 [4].

Observation 7: The information from SIB1 and SIB20 is essential for the UE to be able to received MCCH.

One of the Rel-19 NTN questions that remain is where this service area related information is to be provided for NTN cells supporting MBS. For example, if the intended service area information is provided in SIB (e.g. SIB20 or SIB21), then we need a separate list of provided broadcast sessions in the cell, although the list already exists in the MCCH. Another approach could be providing the intended service area for each broadcast sessions in the MCCH in the same order.

Proposal 9: RAN2 follows the legacy approach and provides MBS related information (such as list of MBS sessions) in MCCH (within MBSBroadcastConfiguration).

## 2.5	Format of the service area

The following agreements were made during the RAN2#125bis meeting that are relevant for the format of the intended service area [2]:

RAN2#125bis agreements: - We will cover at least the case where the indicated intended service area covers a portion of a NTN cell - The intended service area can cover the area of more than one NTN cells (or portions thereof) - At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered): - Circles (like for TN coverage) - Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

If the conclusion in RAN2 is that a smaller than cell-level MBS service area needs to be supported, then RAN2 has a plethora of solutions already available. For example, as a part of NTN enhancements in Release 18, 3GPP defined the support for terrestrial network (TN) coverage area signalling. A new (i.e. Rel-18) system information block (SIB25) conveys the following information elements (IEs) [4]:

CoverageAreaInfo-r18 ::= SEQUENCE { tn-AreaId-r18 TN-AreaId-r18, tn-ReferenceLocation-r18 ReferenceLocation-r17, tn-DistanceRadius-r18 INTEGER(0..65536) }

TN coverage area is described using a reference location and the radius. Together they constitute a circular area with a centre defined by the referenceLocation IE. In our understanding, if MBS service areas do not require a signalling that would reflect their irregular shapes (i.e. other than a circle) then a similar signalling framework could be used in NTN cells.

Proposal 10: RAN2 adopts a similar signalling framework as defined for CoverageAreaInfo in SIB25 for reflecting the MBS service area.

One related aspect agreed in RAN2#126 concerns the association between the MBS service area coordinates and the MBS session [3]:

When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

It implies there has to be an identifier which allows linking the MBS area information (e.g. provided in SIB) with the MBSBroadcastConfiguration in MCCH. One obvious approach is to include in SIB the mbs-SessionId which is available in MBS-SessionInfoList (i.e. as a part of MBSBroadcastConfiguration).

Proposal 11: For the association of MBS session and service area geographical information, RAN2 considers including mbs-SessionId in the SIB providing the service area geographical coordinates.

# 3	Conclusion

This document has made the following observations:

Observation 1: The decision taken at RAN2#126 excludes scenario 1 (The service area is a portion of an NTN cell, and the Content of the service can only be received within the service area) from further considerations in Rel-19.

Observation 2: The footprint of GEO satellites could stretch over hundreds of kilometres which is likely much larger than any MBS service area needs to be.

Observation 3: MBS service will be provided by certain satellite only during limited period (i.e. when satellite’s footprint overlaps with the MBS area).

Observation 4: MBS service area provided by NTN will be covered by cells whose identifiers change over time.

Observation 5: In legacy MBS design AMF selects gNBs that are in the service area corresponding to MBS session.

Observation 6: MCCH provides the list of MBS broadcast sessions, scheduling information, and the list of neighbours with the same MBS services.

Observation 7: The information from SIB1 and SIB20 is essential for the UE to be able to received MCCH.

And proposed the following:

Proposal 1:  RAN2 to confirm that for ETWS message dissemination through broadcast over NTN (if supported), initially will focus on primary notification message.

Proposal 2: RAN2 needs to discuss the following:

- Whether the UE can establish MRBs when it is outside of the service area, but the MBS can be received there

- Whether the UE shall discard MRBs when it leaves the service area, but the MBS can be received there

- How the UE determines it has left the service area

Proposal 3: The UE is allowed to establish MRBs if it can receive broadcast service and if its location is within the MBS service area.

Proposal 4: Discarding MRBs might be subject to additional conditions, such as the time or distance that has elapsed since leaving the MBS area. Another option is to consider what services are provided in the neighbouring MBS service areas.

Proposal 5: NTN-capable UE realizes it is within or outside of MBS service area based on the GNSS it is using for other NTN purposes.

Proposal 6: RAN2 to further explore whether scenario S2 requires specification changes.

Proposal 7: RAN2 is asked to decide if it is problematic when MBS service area is larger than intended area where MBS should be available.

Proposal 8: As RAN2 decided to support MBS via EMC in NTN, RAN2 sends the LS to RAN3 asking this WG to define corresponding NW signalling.

Proposal 9: RAN2 follows the legacy approach and provides MBS related information (such as list of MBS sessions) in MCCH (within MBSBroadcastConfiguration).

Proposal 10: RAN2 adopts a similar signalling framework as defined for CoverageAreaInfo in SIB25 for reflecting the MBS service area.

Proposal 11: For the association of MBS session and service area geographical information, RAN2 considers including mbs-SessionId in the SIB providing the service area geographical coordinates.

# References

- RP-240775 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3 3GPP TSG RAN Meeting #103 Maastricht, The Netherlands, March 18-22, 2024

- R2-2403733 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #125bis Changsha, China, Apr 15th – 19th, 2024

- R2-2405703 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #126 Fukuoka, Japan, May 20th – 24th, 2024

- 3GPP TS 38.331 NR RRC, v18.2.0, July 2024.

- R2-2402355, Discussion on support of broadcast service via NR NTN, CATT, China Broadnet, 3GPP TSG RAN2#125bis meeting, Rel-19, April 2024.