3GPP TSG-RAN WG2 Meeting #127	R2-2406851

Maastricht, The Netherlands, 19th – 23th August 2024	revision of R2-2405149

Agenda item:	8.6.3

Source:	Nokia

Title:	Further View on Measurement-related Enhancements for Rel-19 LTM

WID/SID:	NR_Mob_Ph4 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The phase 4 mobility enhancements work item description captures the measurement enhancements in the [1] as follows:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

    - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

Following agreement was made in RAN2#125bis [2]:

Agreements on measurements:

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

Following agreements were made in RAN2#126 [3]:

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In this paper we present our further view on the Release 19 measurement framework for LTM.

# 2	Discussion

## 2.1 Event triggered Reporting for LTM

### 2.1.1 Beam level and Cell level measurements

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

Agreements on measurements:

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

Serving cell beam event evaluation:

For event triggered L1 measurement the use of beam level measurements were agreed. The open point is now that which serving cell beam measurements are used for event evaluation for events such as LTM2/3/5. The TCI state framework has the following configuration path:

RRC configured list of (unified) TCI States -> MAC CE activation of subset of TCI states -> DCI based indication of one of the activated TCI States that is the ‘indicated’ TCI state.

Additionally, if only one TCI state is activated, it is the ‘indicated’ TCI State.

The UE does not typically measure the TCI states directly, but indirectly via the CSI measurement configuration. Since the LTM cell switch operates on beam level i.e. the UE is indicated with a candidate cell TCI state in the cell switch command (CSC), it makes sense to use the measurement derived from the indicated TCI State of the serving cell. Furthermore MIMO UEIBM made an agreement that (indicated) TCI State can be used as a measurement resource and such approach could be adopted also for LTM. In case of the serving cell supports Rel-15 TCI framework, the TCI state for the measurement resource is derived based on the activated TCI state for PDCCH.

Observation 1: the LTM cell switch operates on beam level so it makes sense to use the indicated beam of the serving cell as reference for beam level event evaluation.

Proposal 1: For beam level event evaluation serving cell reference resource, for LTM 2/3/5 the measurement derived on indicated DL or joint TCI state or the activated TCI state for PDCCH of the serving cell is used.

The serving cell and candidate cell measurement RS should be of the same type (at least for the event evaluation). During Rel-18 LTM work item it was agreed the SSB is used as measurement RS for L1 reporting on candidate cells (and serving).  Thus the at least SSB measurement should be used for serving cell reference. As the TRS cannot be currently used directly as measurement RS, the SSB associated with the TCI state should be used.

The Rel-19 work specifies support for CSI-RS based LTM, thus it should be also considered as measurement RS. This would be up to RAN1 to discuss details i.e. how to derive CSI-RS measurement RS for serving cell based on a TCI state.

Proposal 2: Use at least the SSB associated with the indicated/activated TCI state of the serving cell for deriving the beam level measurement for the serving cell. Further details (and whether/how to use CSI-RS) up to RAN1.

Candidate cell beam(s) for event evaluation:

For the candidate cell measurements, the measurement resources (e.g. SSBs) listed in the LTM-CSI-Resource config can be used to determine whether certain beam of a candidate cell fulfills the reporting criteria. the LTM CSI resource config may comprise of measurement resources of one more candidate cells.

Figure 1. CSI resources and reporting configuration for event LTM3/5

Additionally, if the CSI resource config includes the RS of one candidate cell (or subset of candidate cell) the network could configure multiple LTM CSI resource configs with respective report configurations.

Same resource configuration approach could be used also for the LTM4 event (Figure 2). Also the same approach could be used for the LTM2 event, but without candidate cell resources (for reporting) associated with the event.

Figure 2. CSI resources and reporting configuration for event LTM4.

In the LTM 3/4/5, the UE would determine for each resource listed in the resource set to determine whether it fulfils the reporting criteria. If the CSI- resource config includes RS associated with the serving cell for which the event is configured, the UE may ignore the serving cell RS for event evaluation / measurements (i.e. the UE is not configured to evaluate serving cell RS listed in the resource set used for beam level evaluation since the indicated TCI state should provision the reference resource).

Proposal 3: For the beam level event evaluation for LTM 3/4/5 the measurement resources listed in the R18 resource configuration are used for evaluating beam level events.

Cell level event evaluation:

For the cell level evaluation (which is still FFS) the UE may need to evaluate one or more measurement resources to derive cell quality. For legacy L3 cell quality the UE averages up to N beams (maxNrofSS-BlocksToAverage) to form the cell quality result. If no beam is above the threshold, the UE uses the highest quality beam (N=1). Similar averaging could be used for LTM cell quality derivation. However the filtering aspects should be then considered as well.

If the cell level events are agreed the following approach could be taken (see Figure 3). The evaluated cells for the event triggered reporting (for the certain reporting configuration) are the cells for which the RSs are included in the CSI resource set associated with the reporting configuration.  Also, the cell quality is derived based on the RS listed in the resource set for each respective cell.

Figure 3. Possible approach for cell-level events.

In alternative approach, each of the candidate cell could have a dedicated CSI resource set configuration that are compared with each other. FFS on details.

Figure 4. Alternative approach for cell level events.

Proposal 4: for the cell level event evaluation, the UE derives the cell quality for the serving cell and the candidate cell(s) based on the measurement RS listed in the CSI resource set associated with the event triggered reporting config. FFS if the derivation is similar to L3 ( i.e. thresholds, number of beams to average etc.).

### 2.1.2	Event types

In RAN2#126 the following was noted [3]:

FFS on the need of Event LTM1.

As this is meant to be a beam-level equivalent of Event A1, it is expected LTM1 would be defined as “Beam of serving cell becomes better than absolute threshold”. However, it remains unclear what would be the use of the event defined in such way in Rel-19 LTM. Thus, we suggest the following:

Proposal 5: Do not support LTM1 event in Release 19 LTM work.

### 2.1.3	Reporting configuration framework for LTM

In Release 18 LTM, the CSI resource configurations for L1 measurements were included in LTM configuration (LTM-config) whereas the reporting configurations were part of the respective candidate cell configurations (i.e. inside the serving cell configuration). For LTM R19 enhancements same approach should be taken i.e. the event triggered reporting configuration is seen as new form of CSI reporting type and it should be  part of the candidate cell configuration.

Proposal 6: The event/reporting configuration is part of the serving cell configuration.

### 2.1.4		Filtering for event triggering

To prevent too frequent event triggered reporting a mechanism to perform at least time domain filtering would be needed. Here we discuss two potential approaches.

Approach 1:

One example of an event triggering procedure with time domain filtering is the RLM/BFR procedure. In BFR, the L1 provides indication when the criteria is fulfilled ( i.e. beam failure instance is indicated) and L2 counts the failure instances. When the counter reaches maximum value, the event is triggered (BFR). The time domain filtering is configured using the counter parameter i.e. the network configures the maximum counter value. In a special case, the max value of the counter can be configured to be 1 which means that single occurrence of the indication triggers BFR. This type of approach was selected for MIMO UEIBM i.e. the events are counted within a time window.

In such procedure adopted for LTM, the event evaluation could be performed at L1 and L1 indicates to higher layer (L2) when the event criterion is fulfilled. L2 may trigger the reporting of the measurement associated with the event triggered reporting configuration. This may require counter per configured event.

Approach 2:

In the second approach the LTM could adopt the TTT concept similar to L3 reporting. In the TTT approach, the UE is configured to observe whether the evaluated criteria is fulfilled for the duration of the TTT (e.g. LTM3 event). In a special case the TTT duration could be configured to be ‘0’ which means that that if the event criteria is fulfilled, the event is triggered immediately.

Since the L2 is typically responsible for triggering of signals or procedures (RACH/BFR/MPE/SR) the TTT should run at L2. L1 would be responsible for triggering the indication that event criteria is fulfilled. Also it would need to considered whether the L1 should be providing periodical indications or just indication that event criteria is not fulfilled.

For both approaches some form of hysteresis and/or may be needed to avoid dropping the event triggering based on one slightly lower quality sample.

Both options are valid approached, however the MIMO UEIBM track selected the approach 1, thus we slightly prefer approach 1 to harmonize.

Proposal 7: Support defining configurable time domain filtering based counting lower layer indications at L2 (approach 1)

The measurement framework for NR has numerous components which were defined over several releases, starting from Rel-15. One such configurable aspect is reportOnLeave. It is a flag indicating if the UE should report also when one of the cells that previously met a reporting criteria has now left the cellsTriggeredList. A similar mechanism could be defined for LTM L1 measurements events.

Proposal 8: Consider a similar mechanism as reportOnLeave for LTM L1 measurement reporting framework.

### 2.1.5	Reporting Framework

The reporting framework for LTM should consider the pros and cons of different frameworks that are most suitable for LTM use cases/reporting.

UCI based reporting

The CSI reporting framework does not support event triggered reporting mechanism until R19 and the currently supported reporting types for LTM reporting are the same as for legacy CSI reporting: aperiodic, semi persistent and periodic (which are all network scheduled/triggered). The UCI reporting container is fixed based on the reporting configuration and offers in practise very little to no flexibility for the reporting format. Also, the CSI framework has no mechanism currently to trigger any reporting indication or does not have indication to request resources or indicate the reporting event. Thus for UCI a new procedure would need to be defined. For such operation an aperiodic CSI reporting framework could be used/enhanced.

As a benefit, the UCI reporting has low overhead reporting format and e.g. for LTM L1-RSRP reporting such format has been specified already in Release 18. In MIMO UEIBM track the UCI based mechanism was selected.

Although it may be tempting to consider reusing the MIMO UEIBM framework LTM, the LTM should still consider the pros and cons of different frameworks from the LTM use cases point of view.

Observation 2: MIMO UEIBM enhancement selected the UCI based framework for event-triggered reporting. However, the reporting framework for LTM should be selected based on LTM use cases and needs.

MAC CE based reporting

For MAC CE based reporting, the NR has event triggered reporting framework that could be considered or even partly reused: PHR reporting and Beam Failure Recovery (and LBT failure). As an example, the SCell BFR (of mTRP BFR) procedure can use SR + MAC CE type of event based reporting where the SR can be dedicated for specific indication i.e. SCell beam failure (or a TRP failure). Alternatively, the MAC CE can be provided in an available uplink grant.

MAC CE container has the design flexibility and can carry variable size payloads. The MAC header/payload itself could indicate the reported information/triggered event. Thus, configuration of an existing UL signal (SR) could be used.

Observation 3: For SR + MAC CE type event reporting framework for LTM, SR ID can be associated with event triggered reporting. MAC CE can carry identification of reported event and content.

Observation 4: Defining the SR + (aperiodic) UCI type event reporting framework could allow similar  operation as LTM SR+MAC CE and could reuse at least in some cases the L1 reporting formats that were defined in R18.

Alternatively, the uplink signal for indicating the event could have a design that supports indication of different event types or IDs or indication of the reporting content. Instead of being (single bit) SR or configuring multiple SR, each dedicated for an event, a new uplink signal could carry more information.

For LTM, it could be beneficial to study further both reporting container alternatives in a common framework i.e. define an uplink signal (that may be a new signal or existing one) that indicates a reporting event and also serves as request for uplink reporting resource. The reporting container can be UCI or MAC CE. The MAC CE based solution could be seen more suitable for LTM use case since it supports dynamic payload size. However, in our view the selection of reporting container and uplink signal design is a RAN1 task.

Observation 5: The MAC CE based solution could be seen more suitable for LTM use case since it supports dynamic payload size.

Observation 6: The MAC CE based solution enables the provision of beam report on available uplink grant without first indicating network.

In case the UCI based solution is selected, the container and uplink signal design (to indicate event and request UL resources) is a RAN1 task.

In case of MAC CE based solution, the RAN2 should take lead in design.

Observation 7: For UCI based solution RAN1 is responsible for the design of UL signal and the UCI reporting container.

Observation 8: For MAC CE based solution RAN2 is responsible for the design of UL signal and the MAC CE reporting container.

Proposal 9: Select MAC CE as reporting container and define MAC CE based solution for event triggered LTM reporting.

### 2.1.6. Mobility Simulation for Event Triggered Reporting

To quantify the benefits of event triggered reporting, a system-level simulation was conducted.

Figure 5-Simulation results comparing the Rel. 18 periodic reporting and Rel.19 event-triggered reporting in terms of reported L1 measurements. Different reporting periods of 20ms, 40ms, 80ms for Rel.18 and LTM3- and LTM4 configurations with different thresholds are simulated. The number of reported measurements with proper configuration can decrease with event triggered reporting.

In Figure 5, the Rel.18 periodic reporting and the Rel.19 event triggered reporting are compared in terms of the average number L1 measurement reports, per user per second. The Rel.18 scenarios have the different periodic reporting periods of [20ms, 40ms, 80ms] and the Rel.19 scenarios have different configurations of the proposed event triggered reporting events. LTM3 and LTM4 events are defined with higher and lower threshold, i.e., LTM3 thresholds [-5 dB, +3 dB] and LTM4 thresholds [-70 dBm, -100 dBm]. In Figure 6, the ratio of mobility events (RLFs + HOFs) over all mobility events (HOs + RLFs + HOFs) is illustrated for the same scenarios to observe the overhead and performance trade-off.

Figure 6: Ratio of Mobility Events (RLFs + HOFs) over all Mobility Events (HOs + RLFs + HOFs) in percentage. With less frequent reporting in Rel.18, the ratio increases as the LTM cell switch is delayed. However, with proper configuration the event-triggered reporting, the ratio is comparable with Rel.18 with periodic reporting.

With increasing the reporting period for the Rel.18 scenarios, the number of reported L1 measurements decrease as expected and the mobility performance also decrease, as the mobility failures increase with less frequent available measurement. On the other hand, with the proposed Rel.19 event triggered reporting with LTM3 configuration, the overhead will decrease significantly, without heavily impacting the mobility performance. At the same time, the reporting event is not showing very much sensitivity to the threshold values. However, for the LTM4 configuration, since the measurements of the candidates are reported independent of the source cell’s, the reports are not necessarily arrived at the network near the time when a handover is necessary. Also, the LTM4 exhibits a sensitivity to the threshold values.

Observation 9: Achieving the same mobility performance metrics (such as the number of handovers, radio link failures, and handover failures) with event-triggered reporting using LTM3 event requires significantly fewer reported measurements compared to periodic reporting.

Observation 10: The LTM4 event is sensitive to the threshold value and the mobility performance could be impacted as the measurements are not transmitted in time for handover.

# 3	Conclusion

This paper discussed RAN2 aspects of LTM L1 measurement framework which shall be enhanced as a part of Release 19. The following observations have been made:

Observation 1: the LTM cell switch operates on beam level so it makes sense to use the indicated beam of the serving cell as reference for beam level event evaluation.

Observation 2: MIMO UEIBM enhancement selected the UCI based framework for event-triggered reporting. However, the reporting framework for LTM should be selected based on LTM use cases and needs.

Observation 3: For SR + MAC CE type event reporting framework for LTM, SR ID can be associated with event triggered reporting. MAC CE can carry identification of reported event and content.

Observation 4: Defining the SR + (aperiodic) UCI type event reporting framework could allow similar  operation as LTM SR+MAC CE and could reuse at least in some cases the L1 reporting formats that were defined in R18.

Observation 5: The MAC CE based solution could be seen more suitable for LTM use case since it supports dynamic payload size.

Observation 6: The MAC CE based solution enables the provision of beam report on available uplink grant without first indicating network.

Observation 7: For UCI based solution RAN1 is responsible for the design of UL signal and the UCI reporting container.

Observation 8: For MAC CE based solution RAN2 is responsible for the design of UL signal and the MAC CE reporting container.

Observation 9: Achieving the same mobility performance metrics (such as the number of handovers, radio link failures, and handover failures) with event-triggered reporting using LTM3 event requires significantly fewer reported measurements compared to periodic reporting.

Observation 10: The LTM4 event is sensitive to the threshold value and the mobility performance could be impacted as the measurements are not transmitted in time for handover.

And the following has been proposed:

Proposal 1: For beam level event evaluation serving cell reference resource, for LTM 2/3/5 the measurement derived on indicated DL or joint TCI state or the activated TCI state for PDCCH of the serving cell is used.

Proposal 2: Use at least the SSB associated with the indicated/activated TCI state of the serving cell for deriving the beam level measurement for the serving cell. Further details (and whether/how to use CSI-RS) up to RAN1.

Proposal 3: For the beam level event evaluation for LTM 3/4/5 the measurement resources listed in the R18 resource configuration are used for evaluating beam level events.

Proposal 4: for the cell level event evaluation, the UE derives the cell quality for the serving cell and the candidate cell(s) based on the measurement RS listed in the CSI resource set associated with the event triggered reporting config. FFS if the derivation is similar to L3 ( i.e. thresholds, number of beams to average etc.).

Proposal 5: Do not support LTM1 event in Release 19 LTM work.

Proposal 6: The event/reporting configuration is part of the serving cell configuration.

Proposal 7: Support defining configurable time domain filtering based counting lower layer indications at L2 (approach 1)

Proposal 8: Consider a similar mechanism as reportOnLeave for LTM L1 measurement reporting framework.

Proposal 9: Select MAC CE as reporting container and define MAC CE based solution for event triggered LTM reporting.

# 4	References

[1]	RP-240299	Revised WID: NR mobility enhancements Phase 4

[2] R2-2403731 	Report from session on V2X/SL, R19 NES and MOB 3GPP TSG-RAN WG2 Meeting #125bis Changsha, China, April 15 – 19, 2024

[3] RAN2#126 minutes

[4] 3GPP TS 38.331 NR RRC