3GPP TSG-RAN2 Meeting #127	R2-2406852

# Maastricht, Netherlands, 19th – 23rd August 2024

Agenda item:	7.4.2

Source:	Google

Title:					Discussion on LTM fast processing capabilities

Document for:	Discussion & Decision

- Introduction

As shown below, the ltm-FastProcessingConfig-r18 defined in TS 38.306 is based on feature 39-6 in the RAN4 feature list. This paper identifies and proposes clarifications for ambiguities in the field description of ltm-FastProcessingConfig-r18.

| ltm-FastProcessingConfig-r18 Indicates whether the UE supports fast processing of LTM candidate cell RRC configuration. This capability signalling comprises the following parameters: - maxNumberStoredConfigCells-r18 indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTMCandidateConfig(s) and Scell(s) in LTMCandidateConfig(s) for MCG and SCG, that UE can store the configurations. - maxNumberConfigs-r18 indicates the maximum number of LTMCandidateConfigs that UE can support fast processing. A UE supporting this capability shall also indicate support of ltm-MAC-CE-JointTCI-r18 or ltm-MAC-CE-SeparateTCI-r18. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. NOTE: The conditions for fast processing of an LTM candidate cell RRC configuration is defined in clause 6.3 in TS 38.133 [5]. | Band | No | N/A | No |
| --- | --- | --- | --- | --- |

- Discussion

## 2.1 	maxNumberConfigs-r18

The “NOTE” in the field description of ltm-FastProcessingConfig-r18 describes “The conditions for fast processing of an LTM candidate cell RRC configuration is defined in clause 6.3 in TS 38.133 [5]”.  Clause 6.3 of TS 38.133 is excerpted in Annex. In our understanding, the capability maxNumberConfigs-r18 should correspond to the number of candidate cells for early ASN.1 decoding and validity check as defined in TS 38.133 as shown below. The current description of maxNumberConfigs-r18 in TS 38.306 lacks clarity.

## 6.3 L1/L2-Triggered Mobility <unrelated part omitted> TLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command. TLTM-RRC-processing = 0, if the UE supports [earlyDecodingAndValidityCheck] capability, and at least one of the following conditions is met: - The number of candidate cells in the LTM candidate cell configuration does not exceed [number of candidate cells for early ASN.1 decoding and validity check], - UE has received LTM candidate cell TCI state activation command for the target cell at least THARQ + 13 ms before the LTM cell switch command, and the number of candidate cells with TCI state(s) in LTM candidate cell active TCI state list does not exceed [number of candidate cells for early ASN.1 decoding and validity check], - [FFS: SCell is not part of the cell switch.] Otherwise TLTM-RRC-processing = 10 ms.

Observation 1: maxNumberConfigs-r18 represents the number of candidate cells for which the UE can perform early ASN.1 decoding and validity check as described in TS 38.133.

Proposal 1: Clarify in the field description that maxNumberConfigs-r18 indicates the maximum number of LTMCandidateConfigs for which the UE can perform early ASN.1 decoding and validity check as specified clause 6.3 in TS 38.133.

In addition, LTM candidate configurations may reference an LTM reference configuration, which has the same ASN.1 structure as a LTM candidate configuration. The UE needs to be capable of early decoding and validating both the LTM candidate configuration and the LTM reference configuration. However, the current field description and TS 38.133 do not account for the LTM reference configuration.

Observation 2: The field description of maxNumberConfigs-r18 and TS 38.133 do not consider cases where an LTM reference configuration is present.

Early decoding and validation of the LTM reference configuration demands UE memory and processing resources. If an LTM reference configuration is configured, the maximum number of LTM candidate configurations the UE can early decode and validate might be reduced by one. To address this, we propose two options for RAN2's consideration:

Proposal 2: Clarify the meaning of maxNumberConfigs-r18 in TS 38.306 using one of the following options:

Option 1: maxNumberConfigs-r18 represents the total number of LTMCandidateConfigs and ltm-ReferenceConfiguration for which the UE can perform early ASN.1 decoding and validity check.

Option 2: maxNumberConfigs-r18 represents the maximum number of LTMCandidateConfigs for which the UE can perform early ASN.1 decoding and validity check, as described in TS 38.133. The UE indicating the maxNumberConfigs-r18 shall perform early ASN.1 decoding and validity check for the ltm-ReferenceConfiguration if received.

Compared to option 1, option 2 requires the UE to reserve extra memory to store a decoded LTM reference configuration. In case no LTM reference configuration is configured, the extram memory is wasted. Therefore, we prefer option 1.

## 2.2 	maxNumberStoredConfigCells-r18

The maxNumberStoredConfigCells-r18 specifies the UE's memory capacity for storing serving and candidate cell configurations, as defined in the field description and RAN4 feature list. However, LTM reference configuration is not described in the field description. With similar reasoning as described above, we propose to clarify maxNumberStoredConfigCells-r18 as below:

Proposal 3: maxNumberStoredConfigCells-r18 indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTMCandidateConfig(s) and ltm-ReferenceConfiguration(s), and Scell(s) in LTMCandidateConfig(s) and ltm-ReferenceConfiguration(s) for MCG and SCG, that UE can store the configurations.

Furthermore, the size of LTM candidate and reference configurations should be generally propotional to the number of configured cells. From a UE implementation perspective, the UE allocates a fixed memory size for storing the LTM candidate configurations, the LTM reference configuration(s) and serving cell configurations, regardless of whether it supports LTM fast processing.

Proposal 4: RAN2 should discuss and clarify whether UEs not supporting LTM fast processing should indicate maxNumberStoredConfigCells-r18.

- Conclusion

We propose the following proposals for RAN2 discussion and decision.

Proposal 1: Clarify in the field description that maxNumberConfigs-r18 indicates the maximum number of LTMCandidateConfigs for which the UE can perform early ASN.1 decoding and validity check as specified clause 6.3 in TS 38.133.

Proposal 2: Clarify the meaning of maxNumberConfigs-r18 in TS 38.306 using one of the following options:

Option 1: maxNumberConfigs-r18 represents the total number of LTMCandidateConfigs and ltm-ReferenceConfiguration for which the UE can perform early ASN.1 decoding and validity check.

Option 2: maxNumberConfigs-r18 represents the maximum number of LTMCandidateConfigs for which the UE can perform early ASN.1 decoding and validity check, as described in TS 38.133. The UE indicating the maxNumberConfigs-r18 shall perform early ASN.1 decoding and validity check for the ltm-ReferenceConfiguration if received.

Proposal 3: maxNumberStoredConfigCells-r18 indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTMCandidateConfig(s) and ltm-ReferenceConfiguration(s), and Scell(s) in LTMCandidateConfig(s) and ltm-ReferenceConfiguration(s) for MCG and SCG, that UE can store the configurations.

Proposal 4: RAN2 should discuss and clarify whether UEs not supporting LTM fast processing should indicate maxNumberStoredConfigCells-r18.

- Reference

[1] 3GPP TS 38.306 v18.2.0

[2] 3GPP TS 38.133 v18.2.0

[3] R4-2410748 RAN4 feature list

- Text proposal for 38.306

| ltm-FastProcessingConfig-r18 Indicates whether the UE supports fast processing of LTM candidate cell RRC configuration. This capability signalling comprises the following parameters: - maxNumberStoredConfigCells-r18 indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTMCandidateConfig(s) and ltm-ReferenceConfiguration(s), and Scell(s) in LTMCandidateConfig(s) and ltm-ReferenceConfiguration(s) for MCG and SCG, that UE can store the configurations. - maxNumberConfigs-r18 indicates the maximum number of LTMCandidateConfigs and ltm-ReferenceConfiguration for which the UE can perform early ASN.1 decoding and validity check as specified in clause 6.3 in TS 38.133 [5]. A UE supporting this capability shall also indicate support of ltm-MAC-CE-JointTCI-r18 or ltm-MAC-CE-SeparateTCI-r18. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. NOTE: The conditions for fast processing of an LTM candidate cell RRC configuration is defined in clause 6.3 in TS 38.133 [5]. | Band | No | N/A | No |
| --- | --- | --- | --- | --- |

# Annex: LTM RRM requirement in 38.133 v18.6.0

## 6.3	L1/L2-Triggered Mobility

### 6.3.1	LTM PCell Cell Switch

#### 6.3.1.1	Introduction

The purpose of LTM cell switch is to switch the PCell or PSCell to a target cell indicated in LTM cell switch command. The requirements in this section are applicable to LTM PCell switch.

The requirements in this clause are applicable to SA and NR-DC, and to both intra-frequency and inter-frequency LTM cell switch.

The requirements for inter-frequency cell switch to an FR2 target cell in this clause are only applicable, when

-	network has configured UE to perform L3 measurement with SSB index or L1 measurement for the target cell before the cell switch command, or

-	the SFN of the serving cell from which cell switch command is received and the SFN of the target cell are the same.

The requirements in this clause are applicable to SA for the following scenarios:

PCell switch to a neighboring LTM candidate cell

-	FR1 cell to FR1 cell

-	FR1 cell to FR2 cell

-	FR2 cell to FR2 cell

-	FR2 cell to FR1 cell

PCell switch to an LTM candidate cell that is a serving SCell in MCG

-	FR1 cell to FR1 cell

-	FR2 cell to FR2 cell

The requirements in this clause are applicable to NR-DC for the following scenarios:

PCell switch to a neighboring LTM candidate cell

-	FR1 cell to FR1 cell

PCell switch to an LTM candidate cell that is a serving SCell in MCG

-	FR1 cell to FR1 cell

#### 6.3.1.2	LTM Cell Switch delay

LTM cell switch delay DLTM is the delay from the end of the last TTI containing the MAC-CE command for cell switch until the time the UE transmits the first UL message on the target cell.

When the target cell and the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, the LTM cell switch delay is defined as:

DLTM = Tcmd  + TLTM-interrupt

Where:

Tcmd equals to THARQ + 3ms, where THARQ is the timing between cell switch command and acknowledgement as specified in TS 38.213.

TLTM-interrupt is as stated in section 6.3.1.3.

The target cell in the LTM cell switch command is known if the following conditions are met:

-	During the last 5 seconds before the reception of the cell switch command:

-	The UE has sent a valid L1 or L3 measurement report for the target cell, and

-	One of the SSBs measured from the NR target cell configured for measurement remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell,

-	One of the SSBs measured from the NR target cell configured for measurement remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell,

Otherwise, the cell is unknown.

The target joint DL/UL TCI state or separate DL and UL TCI states in the LTM cell switch command are known if the following conditions are met:

[-	The target TCI state in the cell switch command is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state ≥ -3dB; or]

[-	The target TCI state in cell switch command is activated before receiving the cell switch command and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or]

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target DL/UL TCI state to the completion of LTM cell switch, where the RS resource for L1-RSRP measurement is the RS in target DL/UL TCI state or QCLed to the target DL/UL TCI state

-	LTM cell switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target DL/UL TCI state before the LTM cell switch command

-	The target DL/UL TCI state remains detectable during the LTM cell switching period

-	The SSB associated with the target DL/UL TCI state remain detectable during the cell switching period

-	SNR of the TCI state ≥ -3dB

Otherwise, the target joint DL/UL TCI state or separate DL and UL TCI state is unknown.

#### 6.3.1.3	Interruption time

The interruption time TLTM-interrupt is the time between the end of the last TTI containing the MAC-CE command for LTM cell switch until the time the UE transmits the first UL message on the target cell, excluding Tcmd stated in section 6.3.1.2.

TLTM-interrupt  = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms,

Where:

TLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command.

TLTM-RRC-processing = 0, if the UE supports [earlyDecodingAndValidityCheck] capability, and at least one of the following conditions is met:

-	The number of candidate cells in the LTM candidate cell configuration does not exceed [number of candidate cells for early ASN.1 decoding and validity check],

-	UE has received LTM candidate cell TCI state activation command for the target cell at least THARQ + 13 ms before the LTM cell switch command, and the number of candidate cells with TCI state(s) in LTM candidate cell active TCI state list does not exceed [number of candidate cells for early ASN.1 decoding and validity check],

-	UE has received PDCCH order for early RACH for the target cell at least NT,2+10 ms before the LTM cell switch command, where NT,2 is defined in section 8.1 of TS 38.213 [3]. [FFS further conditions]

-	[FFS: SCell is not part of the cell switch.]

Otherwise TLTM-RRC-processing = 10 ms.

TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

If the UE supports [faster LTM processing] capability, the value of TLTM-processing equals to

[faster intra-FR processing delay (10 ms or 15 ms)] for FR1 to FR1 and FR2 to FR2 LTM cell switch.

[faster inter-FR processing delay (20 ms or 30 ms)] for FR1 to FR2 and FR2 to FR1 LTM cell switch.

Otherwise, the value of TLTM-processing equals to

-	20 ms for FR1 to FR1 and FR2 to FR2 LTM cell switch.

-	40 ms for FR1 to FR2 and FR2 to FR1 LTM cell switch.

Editor’s note: FFS whether a smaller value can be considered in some scenarios or under certain conditions, when the UE does not support [faster LTM processing] capability.

Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

TRS-proc is the time for SSB processing.

Tfirst-RS = 0 and TRS-proc= 0 under the following conditions:

-	The target TCI state indicated in the LTM cell switch command is in the serving cell active TCI state list, or

-	The UE is configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and

-	The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and

-	the time gap between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in section 8.x, and

- 	the time gap between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is not more than [160 ms], or

-	the measurement period of the SSB associated to target TCI state is not larger than 160 ms after the LTM candidate cell TCI state activation MAC-CE is received, or

-	The target cell is an FR1 cell, and the UE is not configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and

-	The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and

-	the time gap between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in section 8.x, and not more than TCI state activation delay stated in section 8.x + [160 ms], or

-	The time gap between the latest PDCCH ordered RACH preamble transmission on the target cell and the cell switch command is not more than [160 ms].

Otherwise,

Tfirst-RS is the time to the first SSB transmission on the target cell [after Tcmd].

Editor’s note: FFS whether TRS transmission is also considered.

TRS-proc = 2 ms.

TLTM-IU is the interruption uncertainty during LTM cell switch.

For RACH-based LTM cell switch, TLTM-IU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TLTM-IU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

For RACH-less LTM cell switch, TLTM-IU_is the uncertainty on transmitting the first uplink transmission on the target cell.