3GPP TSG-RAN WG2 #127	R2-2406854

Maastricht, Netherlands, 19th-23th August 2024

Agenda item:		8.6.2	Inter-CU LTM

Title:		Discussion on inter-CU LTM

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

During the previous RAN2 meeting, some agreements for inter-CU LTM have been reached [1][2].

In this contribution, we further discuss the inter-CU LTM issues and provide our views, including the reference configuration, LTM execution and candidate cells release.

# 2. Discussion

2.1 Reference configuration

One issue that needs to be discussed is whether single reference or multiple reference should be configured for inter-CU LTM. Given that only single reference configuration for SCPAC, similarly, we think that supporting only one reference for inter-CU LTM is not a problem for signaling overhead. In addition, RAN2 agreed that the maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations, in this case, introducing multiple reference configuration has very limited benefits for signalling overhead but it will brings additional configuration complexity.

Therefore, we think that a single reference configuration should be enough to avoid additional signalling overherad as well as additional complexity.

Proposal 1: Only single reference configuration is used per UE, i.e., common for inter-CU LTM and intra-CU LTM.

2.2	Inter-CU LTM execution

RAN2 agreed that the UE can be configued with a mixture of intra-CU and inter-CU candidate LTM cells, and Rel-19 inter-CU LTM support mixture of subsequent intra-CU LTM and subsequent inter-CU LTM after a first/previous LTM switch. The UE behaviour for different type (i.e., inter-CU or intra-CU) of LTM execution is different, especially for L2 behaovor.

Considering that the PDCP is located in CU, and the CU is not changed during the intra-CU LTM procedure, the PDCP reestablishment is not needed in the Rel-18 intra-CU LTM. For intra-CU intra-DU LTM, only the MAC reset is performed. For intra-CU inter-DU LTM, the RLC reestablish and PDCP data recovery should be also performed besides the MAC reset. Therefore, L2 no reset identity (ltm-ServingCellNoResetID, ltm-NoResetID) configured by RRC is introduced to determine if the RLC reestablish and PDCP data recovery should be performed during intra-CU LTM.

For inter-CU LTM, the PDCP reestablish should be performed due to PDCP anchor change and security key update. The detailed method for UE to determine whether to perform PDCP reestablish should be discussed when there is some progress on security issues by SA3.

Therefore, the UE should decide whether the inter-CU or intra-CU LTM cell switch is triggered to perform the corresponding L2 reset behavior. In addition, the L2 no reset concept (i.e., ltm-NoResetID) can be reused to further determine the L2 behavior when it is determined whether PDCP reestablish is performed upon LTM cell switch.

Proposal 2: Upon triggering cell switch, the UE needs to determine whether the inter-CU or intra-CU LTM is performed.

Proposal 3: RAN2 to assume the L2 no reset concept (i.e., ltm-NoResetID) can be reused to further determine the L2 behavior when it is determined whether PDCP reestablish is performed upon LTM cell switch.

In addition, when the LTM cell switch is triggered by the source gNB, the target gNB needs to know about the initiation of the cell switch command to the UE. Especially for DG based RACH-less LTM, the target gNB starts the dynamic scheduling based on the notification. In addition, the selected TCI state also need to be transferred from source gNB to target gNB. In Rel-18 intra-CU LTM, only the CU-DU cell switch notification in F1 interface is needed because the CU is not changed. However, for Rel-19 inter-CU LTM, the CU is changed, cell change notification from source CU to target CU is also needed in Xn interface. On the detailed Xn signaling for cell switch notification, whether it can extend the current F1 message or introduce a new message can up to RAN3.

Proposal 4: The source CU should inform the target CU about the cell switch. The detailed signaling up to RAN3.

2.3	LTM candidate cells release

In Rel-18 intra-CU LTM, subsequent LTM cell switch between candidates without RRC reconfiguration can be supported. That is, one RRC Reconfiguration can be used for multiple LTM cell switch until the network modifies or releases the candidate cell configurations, and UE autonomous release of the candidate cells is not supported.

We think that the same principle can be used for inter-CU LTM since the subsequent intra-CU and inter-CU LTM also can be supported by Rel-19 inter-CU LTM.

Proposal 5: For inter-CU LTM, the candidate cell configuration can only be released by network.

# 3. Conclusion

In this contribution we discussed some issues related to inter-CU LTM and made the following proposals.

Proposal 1: Only single reference configuration is used per UE, i.e., common for inter-CU LTM and intra-CU LTM.

Proposal 2: Upon triggering cell switch, the UE needs to determine whether the inter-CU or intra-CU LTM is performed.

Proposal 3: RAN2 to assume the L2 no reset concept (i.e., ltm-NoResetID) can be reused to further determine the L2 behavior when it is determined whether PDCP reestablish is performed upon LTM cell switch.

Proposal 4: The source CU should inform the target CU about the cell switch. The detailed signaling up to RAN3.

Proposal 5: For inter-CU LTM, the candidate cell configuration can only be released by network.

# 4. References

[1] R2-2402102, Report of 3GPP TSG RAN WG2 meeting #125bis

[2] Draft Report of 3GPP TSG RAN WG2 meeting #126