3GPP TSG-RAN WG2#127												    R2-2406856

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.2.3

Source: 	NEC

Title:	        Ambient-IoT Paging

Document for:	 Discussion

- Introduction

In TSG RAN Meeting #102, a new SID of studying on solutions for Ambient IoT in NR has been approved [1]. Paging will be supported as a basic function to enable DO-DTT and DT data transmission. In RAN2#126, the following agreements were made:

Paging 1 RAN2 will study the following cases for AIoT paging message: • a message containing an ID of a single A-IoT device. • a message containing a group ID that maps to multiple A-IoT devices. • a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. • a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

This contribution provides our views on the paging of ambient-IoT.

- Discussion

  - Paging contains group ID

Support of identifying group of devices leads to how to group devices. We think at least following aspects can be considered:

- A static grouping information. This may base on particular provider or operator, or particular product type;

- Group information decided by A-IoT NAS layer (if supported);

- Group information decided by A-IoT AS layer. Initial trigger message(s) may be used to set group information. Device may be identified to be in different groups and further access the reader according to the group information. For example, a device, whose group information is set to “group 1”, should only respond to further commands for “group 1”.

Consider how to group devices from following aspects:

- Group information based on particular provider or operator, or particular product type, which may be a static grouping information;

- Group information decided by NAS layer;

- Group information based on paging: initial trigger message(s) may be used to set group information. Device may be identified to be in different groups and further access the reader according to the group information.

Paging message may include group ID that maps to multiple devices. For a group of devices which are selected by the same group ID, at least part of device ID may be common with each other, for example, some initial bits. During random access, it may not very optimized because common bits are not useful for contention resolution.

For devices in a single device group, at least part of device ID may be common with each other because they can be masked by a same group ID.

For devices masked by the same group ID, avoid use common bits for contention resolution.

  - Paging contains multiple IDs

For paging message contains multiple device IDs, the reader has knowledge of the number of intended devices. When the intended multiple devices are known to be within the reader’s coverage, then it is nature to configure contention free RA for this case. On the other hand, when the reader does not know whether the intended multiple devices are within the coverage or not, it may configure contention-based RA.

For A-IoT paging message contains multiple device IDs of A-IoT devices, the reader can configure either contention free RA or contention-based RA.

Assuming a device ID list is used to indicate multiple device IDs, the list can also implicitly indicate the allocated resource for further random access. For example, each entry of the device ID list may map to a random access resource. By using the mapping relationship, A-IoT devices can be scattering to different random access resources and achieve contention free random access.

For contention free RA configuration when A-IoT paging message contains multiple IDs of A-IoT devices, mapping relationship between the IDs of A-IoT device and the random access resources is implicitly indicated by the device ID list.

Figure 2.2-1 mapping relationship between the entry of device ID list and random access resources

  - Respond based on multiple initial trigger messages

In RFID technology, selecting tag populations is implemented by successive SELECT commands which brings much more flexibilities for device identifying. Similarly, we think it is also beneficial to support multiple initial trigger messages for one paging procedure. For example, an initial trigger message set consists of multiple initial trigger messages, device(s) to determine whether to respond based on more than one initial trigger messages (i.e., intersections or unions of device/group identifiers).

Support multiple initial trigger messages for one paging procedure, device(s) to determine whether to respond based on more than one initial trigger messages.

  - Respond based on data to report

Considering use cases of reaching devices for collecting data, i.e., sensor data, it is possible that only part of devices have available data. If all the devices respond upon being paged (especially respond to the A-IoT paging message triggering an inventory and command procedure), some of them performing a successful random access-like procedure may  be found with empty senor data buffer.

In some use cases it is possible that not all of the devices being paged need to respond.

Large device populations will bring more collisions in the access stage, causing more delay or access failures. Therefore, for devices without available data to report, it is better not to respond the paging even if it is being paged. In such a case we think allowing intended devices to determine whether to respond to the A-IoT paging message triggering an inventory and command procedure is beneficial. In other words, device(s) can determine whether to respond the paging procedure not only based on the indicated device/group identifier, but also based on whether there’s available data to be reported.

Based on configuration of the A-IoT paging message triggering, device(s) can determine whether to respond the paging procedure based on whether there’s available data to be reported.

  - Determination of transmission/access occasion

In RFID, within an access round triggered by “Query” command, determination of transmission/access occasion is based on reception of “QueryRep” command which instructs devices to decrement their slot counters and, if slot count=0 after decrementing, to backscatter an RN16 to the reader immediately. Like “QueryRep” command, a transmission/access occasion start message is supported for determination of transmission/access occasion.

Like “QueryRep” command in RFID, a transmission/access occasion start message is supported for determination of transmission/access occasion.

The transmission/access occasion start message instructs devices to decrement their slot counters and, if slot count=0 after decrementing, to backscatter an RN16 to the reader immediately.

RAN1#116bis agreed to “Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS.” If RAN1 agrees to support frequency-domain multiple access of D2R transmissions, then whether to support individual transmission/access occasion start message for multiple frequency-domain accesses of D2R transmission needs further discussions.

Moreover, RAN1#117 agreed to “Study whether/how an A-IoT device can count the time with sufficient accuracy (with a certain timing error due to SFO) at least for the purposes related to TDM(A) (if needed), and if so for how long after receiving an R2D transmission.” The time count purposes listed in [3] are

•	e.g., device can sleep or skip monitoring PRDCH for a time duration based on reader’s indication or based on the scheduling/timing relations, TDMA of multiple D2R transmissions from different devices by the same R2D transmission etc.

•	Clock assumptions, accuracy

•	Device power consumption, complexity

If RAN1 agrees to support device behaviour to sleep or skip monitoring PRDCH (e.g., the transmission/access occasion start message) for a time duration, then whether to include indicate the cumulative number of past transmission/access occasion in the transmission/access occasion start message needs further discussions.

Regarding the detailed content of transmission/access occasion start message, RAN2 to wait for more RAN1 progresses on frequency-domain multiple access and device behaviour to sleep or skip monitoring PDSCH during a time duration.

  - Multiple readers issue

As per the general scope of SID [1], the following coverage requirement is considered:

General Scope - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”.

Given such a small coverage and the need for seamless coverage, a region may be covered or served by multiple readers.

An A-IoT operating area may be covered or served by multiple readers.

In RFID technology, inventory process comprising an interrogator and an associated tag population. An interrogator chooses one of four sessions and inventories tags within that session. From the tag perspective, each session may associate to different interrogators or the same interrogator with different inventory service. In this way sessions allow tags to keep track of their inventoried status separately for each of four possible time-interleaved inventory processes, using an independent inventoried flag for each process. The interrogator and associated tag population operate in one and only one session for the duration of an inventory round.

RFID sessions allow tags to keep track of their inventoried status separately for each of four possible time-interleaved inventory processes.

When we come to A-IoT device, whether or how do A-IoT devices handle multiple processes with the same or different readers may need to be considered.

RAN2 to study whether or how do A-IoT devices handle multiple processes with the same or different readers.

- Conclusion

Observation 1:	For devices in a single device group, at least part of device ID may be common with each other because they can be masked by a same group ID.

Observation 2:	In some use cases it is possible that not all of the devices being paged need to respond.

Observation 3:	An A-IoT operating area may be covered or served by multiple readers.

Observation 4:	RFID sessions allow tags to keep track of their inventoried status separately for each of four possible time-interleaved inventory processes.

Proposal 1:	Consider how to group devices from following aspects:

a)	Group information based on particular provider or operator, or particular product type, which may be a static grouping information;

b)	Group information decided by NAS layer;

c)	Group information based on paging: initial trigger message(s) may be used to set group information. Device may be identified to be in different groups and further access the reader according to the group information.

Proposal 2:	For devices masked by the same group ID, avoid use common bits for contention resolution.

Proposal 3:	For A-IoT paging message contains multiple device IDs of A-IoT devices, the reader can configure either contention free RA or contention-based RA.

Proposal 4:	For contention free RA configuration when A-IoT paging message contains multiple IDs of A-IoT devices, mapping relationship between the IDs of A-IoT device and the random access resources is implicitly indicated by the device ID list.

Proposal 5:	Support multiple initial trigger messages for one paging procedure, device(s) to determine whether to respond based on more than one initial trigger messages.

Proposal 6:	Based on configuration of the A-IoT paging message triggering, device(s) can determine whether to respond the paging procedure based on whether there’s available data to be reported.

Proposal 7:	Like “QueryRep” command in RFID, a transmission/access occasion start message is supported for determination of transmission/access occasion.

Proposal 8:	The transmission/access occasion start message instructs devices to decrement their slot counters and, if slot count=0 after decrementing, to backscatter an RN16 to the reader immediately.

Proposal 9:	Regarding the detailed content of transmission/access occasion start message, RAN2 to wait for more RAN1 progresses on frequency-domain multiple access and device behaviour to sleep or skip monitoring PDSCH during a time duration.

Proposal 10:	RAN2 to study whether or how do A-IoT devices handle multiple processes with the same or different readers.

- References

[1] RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, Huawei (moderator, RAN1 Vice-Chair).

[2] Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan, ETSI MCC

[3] R1-2405381, FL summary #3 on frame structure and timing aspects for Rel-19 Ambient IoT