3GPP TSG-RAN WG2 Meeting #127 	   R2-2406858

# Maastricht, Netherlands, Aug 19th – 23th, 2024

Source:	TCL

Title:	Solutions for DSR enhancement

Agenda item:	8.7.4

Document for:	Discussion and decision

- Introduction

In RAN2 #126 meeting, we have discussed the DSR enhancement extensively and reached the following agreements:

Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. FFS whether DSR triggering is impacted FFS whether PDU set importance needs to be included

In this contribution we will discuss the FFSs related to DSR enhancement.

- Discussion

  - How to report multiple pairs of remaining time and buffer size in DSR

Based on the DSR MAC CE in Release 18, which includes the remaining time and the corresponding buffer size, including multiple pairs of remaining time and buffer size in DSR is inefficient. A potentially more efficient solution is to predefine multiple remaining time levels with maximum limits. Then, include a one-bit indicator for each remaining time level to signify whether there is buffer size information for the corresponding remaining time level for each LCG whose delay information is reported in the DSR. Figure 1 is an example for the DSR format for multiple pairs of remaining time and buffer size (the maximum level is 7).

Figure 1

Regarding the real remaining time range for each remaining time level, two options can be considered:

- Defining multiple remaining time levels and the corresponding rang in a table. The following table is an example in which the level 6 is used to report the non-delay critical data.

| Remaining time level | Remaining time range (ms) |
| --- | --- |
| 0 | Remaining time < 10 |
| 1 | 10 < Remaining time < 20 |
| 2 | 20 < Remaining time < 30 |
| 3 | 30 < Remaining time < 40 |
| 4 | 40 < Remaining time < 50 |
| 5 | 50 < Remaining time < 60 |
| 6 | non-delay critical data |

- Configuring multiple remaining time levels and the corresponding rang for DSR by RRC message. The number of pairs of the remaining time levels and the corresponding rang is configurable but not exceeding the the maximum limits. And the configuration could be applicable for a LCG or UE.

Proposal 1: It is suggested to define remaining time level and only include a indicator for the remaining time level in the DSR to simplify the MAC CE for multiple pairs of remaining time and buffer size.

- LCP is an essential mechanism on the UE side for further allocating resources based on logical channel priorities. Providing associated logical channel priority information for buffer size information in the DSR can help the gNB better allocate uplink resources. In addition to the remaining time and buffer size in the DSR, including the highest LCH priority information for an LCG in the DSR is worth considering.

Proposal 2: It is suggested to enhance the DSR with highest LCH priority information for a LCG.

  - How to trigger a DSR with multiple pairs of remaining time and buffer size

- Basically the legacy principle of triggering a DSR for a logical channel can be reused. However, for the enhanced DSR with multiple pairs of remaining time and buffer size, following additional issues should be further considered:

- For enhanced DSR, should the threshold for triggering DSR be consistent with the configured remaining time? If they are consistent, does each remaining time need to trigger a DSR?

- In principle, the threshold for triggering enhanced DSR does not need to be associated with any configured remaining time. If not associated, one or more thresholds corresponding to enhanced DSR can be configured to trigger the enhanced DSR. However, to simplify the configuration process, it is reasonable to associate the threshold with the remaining time. Each configured remaining time range can correspond to a threshold, and one or more thresholds corresponding to the remaining time can be selectively enabled to trigger the enhanced DSR.

Proposal 3: It is suggested to selectively enable one or more thresholds corresponding to the configured remaining time to trigger the enhanced DSR.

- The conflict between legacy DSR and the enhanced DSR when both are configured simultaneously

- If both legacy DSR and enhanced DSR are configured simultaneously, and the enhanced DSR is triggered but pending, the subsequent legacy DSR will not be triggered. If the legacy DSR is triggered, the enhanced DSR can be triggered, but the buffer size information corresponding to the remaining time already included in the legacy DSR should be excluded from the enhanced DSR.

Proposal 4: It is suggested to further study the conflict between legacy DSR and enhanced DSR.

  - PSI in DSR or not

- We believe that PSI information should not be included in the DSR for the following reasons:

- PSI is only useful during congestion, which rarely occurs. Therefore, carrying PSI information under normal conditions would result in unnecessary resource wastage.

- When congestion does occur, PDUs with low PSI may be discarded due to the discard timer expiring on the UE side, while the gNB remains unaware of when or if the discarding happens.

- Including PSI information in the DSR will increase the format complexity of the DSR MAC CE, as it requires reporting multiple buffer sizes corresponding to different PSIs.

Proposal 5: It is suggested not to include PSI information in DSR.

- Conclusion

In this contribution, we discussed the DSR enhancement for XR service and provided our proposals as follows:

Proposal 1: It is suggested to define remaining time level and only include a indicator for the remaining time level in the DSR to simplify the MAC CE for multiple pairs of remaining time and buffer size.

Proposal 2: It is suggested to enhance the DSR with highest LCH priority information for a LCG.

Proposal 3: It is suggested to selectively enable one or more thresholds corresponding to the configured remaining time to trigger the enhanced DSR.

Proposal 4: It is suggested to further study the conflict between legacy DSR and enhanced DSR.

Proposal 5: It is suggested not to include PSI information in DSR.

- References

- RP-234057: "New WID: XR (eXtended Reality) for NR Phase 3".