3GPP TSG-RAN WG2 Meeting #127                                   R2-2406859

Maastricht, Netherlands, 19th August - 23th August, 2024

Agenda item:		7.24.2.2

Source:		Samsung

Title:			Discussion on TEI18 enhancement to measurement report

Document for:	Discussion & Decision

# 1	Introduction

In RAN2#126, RAN2 made the following agreement to enhance the measurement report as TEI 18 topic.

Agreements: 1 After performing measurements according to the existing RAN4 requirements, if a measurement report is triggered the UE reports a new indication within MeasResults IE to indicate if an event entering condition for the cell is satisfied for the first time. 2 After performing measurements according to the existing RAN4 requirements, if a measurement report is triggered the UE reports a new indication within MeasResults IE to indicate which cell(s) fulfilled the event leaving condition. 3 For reporting best cell change, to introduce an indication (e.g. reportOnlyIfBestCellChange) in ReportConfigNR, used to indicate the UE to only send periodical event MR if the best neighbour cell has changed. The field is optionally configured when reportAmount >1. 4 as a compromise we will limit N=2 and new capability. Send a report if either first best cell has changed or the second best cell has change 5 RAN2 to introduce new UE capabilities in case these new informations to be reported are agreed.

The motivation of the enhancement is for UE 1) to provide gNB with additional information about the leaving and entering condition for a cell to help HO decision and 2) to only send periodical event MR if the best neighbour cell has changed. The enhancement can be summarized as below.

- A new indication within MeasResults IE to indicate if an event entering condition for the cell is satisfied for the first time.

- A new indication within MeasResults IE to indicate which cells(s) fulfilled the event leaving condition.

- A new indication in ReportConfigNR IE to configure UE to only send periodical event MR if the best neighbour cell has changed.

For the enhancement, the RRC CR is agreed in R2-2406021 and UE capability CR is agreed in R2-2405994.

In this contribution, we discuss some issues that need corrections related to the TEI18 enhancement.

# 2	Discussion

## 2.1	Report of the leaving event

According to the latest TS 38.331 v18.2.0 [1], the UE can store/report the cells for which the event leaving condition via cellsMetLeavingCond/cellsMetReportOnLeaveList. For that, the UE checks if the leaving condition is fulfilled for the cells included in the cellsTriggeredList and stores the concerned cells in the cellsMetLeavingCond as in Table 1.

Table 1

### 5.5.4 Measurement report triggering #### 5.5.4.1 General If AS security has been activated successfully, the UE shall: <omitted> 2> if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event: 3> store the concerned cell(s) in the cellsMetLeavingCond defined within the VarMeasReportList for this measId; 3> remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId; 3> if reportOnLeave is set to true for the corresponding reporting configuration: 4> if the corresponding reportConfig does not include numberOfTriggeringCells; or 4> if the corresponding reportConfig includes numberOfTriggeringCells and a measurement report was previously sent to the network for at least one of the concerned cell(s): 5> initiate the measurement reporting procedure, as specified in 5.5.5; 3> remove all the concerned cell(s) from cellsMetLeavingCond defined within the VarMeasReportList for this measId, if any;

Issue 1. UE behaviour for leaving condition:

According to green part in Table 1, the UE  stores the concerned (leaving) cell(s) in the cellsMetLeavingCond and remove them again even when it is not actually necessary as in the following cases.

First, if the UE does not support enterinAndLeaveCellReport, the UE can not report the leaving event anyway and thus does not need to store the concerned cell(s) in cellsMetLeavingCond.

Second, if enteringLeavingReport is not configured, the UE does not include cellsMetReportOnLeaveList (i.e., the cells in cellsMetLeavingCond ) in the measurement report. Also in this case, there is no need for UE to store the concerned cell(s) in cellsMetLeavingCond.

Observation 1: If enteringLeavingReport is not configured or not supported by UE, there is no need for UE to store the concerned cells for which the leaving conditions is fulfilled in cellsMetLeavingCond.

To address the issue, we think the green part should be executed only if enteringLeavingReport is configured in the corresponding reportConfig. The gNB should not configure enteringLeavingReport based on UE capability information if the UE does not support it. No additional condition is needed to check the UE capability.

Proposal 1: When the event leaving condition is fulfilled, the UE stores the concerned cell(s) in cellsMetLeavingCond only if enteringLeavingReport is configured in the corresponding reportConfig. The TP1 in annex can be considered for the correction.

Issue 2. Maximum number of leaving cells:

According to yellow part in Table 1, the UE checks if the leaving condition is fulfilled for the cells included in the cellsTriggeredList and stores the concerned cells in the cellsMetLeavingCond. However, there is the discrepancy between the max cell number in cellsTriggeredList (32) and that in cellsMetLeavingCond/cellsMetReportOnLeaveList (8). Let’s assume that the number of cells in cellsTriggeredList is 15 and more than 8 cells (i.e., 10 cells) are met the leaving condition for TTT. In this case, the number of concerned cells exceeds the maximum length of cellsMetLeavingCond and it is unclear how the UE can report the concerned cells properly.

Observation 2: There is the discrepancy between the max cell number in cellsTriggeredList (32) and that in cellsMetLeavingCond/cellsMetReportOnLeaveList (8).

Observation 3: When there are more than 8 cells for which the leaving condition is fulfilled together, it is unclear how the UE can report the concerned cells via cellsMetReportOnLeaveList properly.

To address the issue, we can consider the following two options.

- Option1: Change the maximum length of cellsMetLeavingCond/cellsMetReportOnLeaveList from maxCellReport (8) to maxNrofCellMeas (32)

- Option2: Specify how to limit the number of cells to be stored in cellsMetLeavingCond within the current max number, maxCellReport (8)

For the option 1, we think it is too late to extend the maximum length since the correction is NBC after the ASN.1 freeze for R18. Moreover, the motivation to include up to 32 cells in cellsMetReportOnLeaveList is unclear considering that the UE can report the measurement results only for up to 8 neighbouring cells via measResultListNR. Here, it should be reminded that the enhancement for the explicit report of leaving event mainly comes from the motivation to help gNB cancel/update the HO configuration for the concerned cells [2]. Thus, in our view, the UE does not need to report the leaving event for the cells that have not been reported to the gNB before. From gNB’s perspective, there is no need to check the leaving event for the unknown (or not reported) cells. Therefore, it seems reasonable to use the common maximum number maxCellReport (8) for both measResultListNR and cellsMetReportOnLeaveList.

Observation 4: It is reasonable to use the common maximum number maxCellReport (8) for both measResultListNR and cellsMetReportOnLeaveList. Also, extending the maximum length of cellsMetReportOnLeaveList from 8 to 32 is NBC correction after R18 ASN.1 freeze.

Observation 5: The UE does not need to report the leaving event for the cells that have not been reported to the gNB before. The gNB would not be interested in the event for the unknown cells.

For the option 2, based on the observation above, we can simply think of the limitation to allow UE to include each of the concerned cells only if the cell has been reported to the network before. This can limit the number of cells to be stored in cellsMetLeavingCond within the current max number, maxCellReport (8). Please, note that UE can report the measurement results only for up to 8 cells via measResultListNR according to the current RRC specification.

Proposal 2: The UE stores each of the concerned cell(s) in cellsMetLeavingCond only if the cell has been reported to the gNB before. The TP2 in annex can be considered for the correction.

## 2.2	Best cell change reporting

According to the latest TS 38.331 v18.2.0 [1], NW can configure UE to send periodical event MR only if either first best cell or the second best cell has changed. When the UE is configured to do that, it should store the previously reported first/second best cell(s) to check if the current best cells are the same as before or not. For that, the UE stores the first/second best cells in reportedBestNeighbourCell as specified in Table 2.

Table 2

### 5.5.5 Measurement reporting #### 5.5.5.1 General <omitted> 6> if reportType is set to eventTriggered and reportOnBestCellChange is configured: 7> if reportOnBestCellChange is set to n1: 8> set the reportedBestNeighbourCell defined within the VarMeasReportList for this measId to the first cell listed in the measResultNeighCells; 7> if reportOnBestCellChange is set to n2: 8> set the first cell in reportedBestNeighbourCell defined within the VarMeasReportList for this measId to the first cell listed in the measResultNeighCells and the second cell in reportedBestNeighbourCell defined within the VarMeasReportList for this measId to the second cell listed in the measResultNeighCells;

Issue 3. Exceptional cases of no first/second best:

According to yellow part, when reportOnBestCellChange is set to n1 or n2, the UE sets the first cell in reportedBestNeighbourCell to the first cell listed in measResultNeighCells. However, there can be the case where there is no cell in measResultNeighCells. For example, let’s assume that there is only one cell A in the cellsTriggeredList and it can be removed after leaving condition is met. In this case, the UE can not set the first cell in reportedBestNeighbourCell.

For the green part, when reportOnBestCellChange is set to n2, the UE sets the second cell in reportedBestNeighbourCell to the second cell listed in measResultNeighCells. However, when only one single cell triggers the event, there is only one cell in measResultNeighCells. At this time, UE is not able to set the second best cell in reportedBestNeighbourCell.

Observation 6: When there is no cell or only one cell in measResultNeighCells, the UE can not set the first/second best cell in reportedBestNeighbourCell properly.

To address the issue, we think the procedure text should be updated to allow UE to set the the first/second best cell in reportedBestNeighbourCell only when it is available.

Proposal 3: The UE can set the first cell in reportedBestNeighbourCell only if measResultNeighCells is not empty and the UE can set the second cell in reportedBestNeighbourCell only if there is more than one entry in measResultNeighCells. The TP3 in annex can be considered for the correction.

# 3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Issue 1. UE behaviour for leaving condition:

Observation 1: If enteringLeavingReport is not configured or not supported by UE, there is no need for UE to store the concerned cells for which the leaving conditions is fulfilled in cellsMetLeavingCond.

Proposal 1: When the event leaving condition is fulfilled, the UE stores the concerned cell(s) in cellsMetLeavingCond only if enteringLeavingReport is configured in the corresponding reportConfig. The TP1 in annex can be considered for the correction.

Issue 2. Maximum number of leaving cells:

Observation 2: There is the discrepancy between the max cell number in cellsTriggeredList (32) and that in cellsMetLeavingCond/cellsMetReportOnLeaveList (8).

Observation 3: When there are more than 8 cells for which the leaving condition is fulfilled together, it is unclear how the UE can report the concerned cells via cellsMetReportOnLeaveList properly.

Observation 4: It is reasonable to use the common maximum number maxCellReport (8) for both measResultListNR and cellsMetReportOnLeaveList. Also, extending the maximum length of cellsMetReportOnLeaveList from 8 to 32 is NBC correction after R18 ASN.1 freeze.

Observation 5: The UE does not need to report the leaving event for the cells that have not been reported to the gNB before. The gNB would not be interested in the event for the unknown cells.

Proposal 2: The UE stores each of the concerned cell(s) in cellsMetLeavingCond only if the cell has been reported to the gNB before. The TP2 in annex can be considered for the correction.

Issue 3. Exceptional cases of no first/second best:

Observation 6: When there is no cell or only one cell in measResultNeighCells, the UE can not set the first/second best cell in reportedBestNeighbourCell properly.

Proposal 3: The UE can set the first cell in reportedBestNeighbourCell only if measResultNeighCells is not empty and the UE can set the second cell in reportedBestNeighbourCell only if there is more than one entry in measResultNeighCells. The TP3 in annex can be considered for the correction.

# 4	Reference

- 3GPP TS 38.331 v18.2.0, “Radio Resource Control (RRC) protocol specification”.

- R2-2404975, “Discussion on enhancements to measurement report [meas_report_enh]”, Ericsson et al.

# Annex: Text Proposals

## TP1

#### 5.5.4.1	General

If AS security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

<omitted>

2>	if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3> if enteringLeavingReport is configured and reportOnLeave is set to true for the corresponding reporting configuration:

4>	store the concerned cell(s) in the cellsMetLeavingCond defined within the VarMeasReportList for this measId;

3>	remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	if the corresponding reportConfig does not include numberOfTriggeringCells; or

4>	if the corresponding reportConfig includes numberOfTriggeringCells and a measurement report was previously sent to the network for at least one of the concerned cell(s):

5>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	remove all the concerned cell(s) from cellsMetLeavingCond defined within the VarMeasReportList for this measId, if any;

3>	if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

<omitted>

## TP2

### 5.5.4	Measurement report triggering

#### 5.5.4.1	General

If AS security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

<omitted>

2>	if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3> for each of the concerned cell(s):

4> if a measurement report was previously sent to the network for the concerned cell:

5>	store the concerned cell(s) in the cellsMetLeavingCond defined within the VarMeasReportList for this measId;

3>	remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	if the corresponding reportConfig does not include numberOfTriggeringCells; or

4>	if the corresponding reportConfig includes numberOfTriggeringCells and a measurement report was previously sent to the network for at least one of the concerned cell(s):

5>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	remove all the concerned cell(s) from cellsMetLeavingCond defined within the VarMeasReportList for this measId, if any;

3>	if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

<omitted>

## TP3

### 5.5.5	Measurement reporting

#### 5.5.5.1	General

Figure 5.5.5.1-1: Measurement reporting

The purpose of this procedure is to transfer measurement results from the UE to the network. The UE shall initiate this procedure only after successful AS security activation.

The UE shall, for each entry in the VarMeasReportList:

<omitted>

6>	if reportType is set to eventTriggered and reportOnBestCellChange is configured:

7> if the measResultNeighCells is not empty:

8>	if reportOnBestCellChange is set to n1 or n2:

9>	set the first cell in reportedBestNeighbourCell defined within the VarMeasReportList for this measId to the first cell listed in the measResultNeighCells;

7> if there is more than one entry in the measResultNeighCells:

8>	if reportOnBestCellChange is set to n2:

9>	set the second cell in reportedBestNeighbourCell defined within the VarMeasReportList for this measId to the second cell listed in the measResultNeighCells;

<omitted>