3GPP TSG-RAN WG2 Meeting #127                                   R2-2406860

Maastricht, Netherlands, 19th August - 23th August, 2024

Agenda item:		8.3.2.1

Source:		Samsung

Title:			Discussion on the simulation results for RRM measurement prediction

Document for:	Discussion & Decision

# 1	Introduction

In RAN2#126, RAN2 agreed the following table on the prioritization between different evaluation scenarios/cases for RRM measurement prediction.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this contribution, we show our initial evaluation results for the cases with ‘High’ priority and discuss some observations based on the results.

# 2	Discussion

## 2.1	Evaluation results for RRM measurement prediction

In this section, we would like to show our initial evaluation results for the high priority cases and share some observation based on the results. At this time, we focus on the evaluation for L3 cell-level RSRP prediction and beam-level spatial domain prediction is deprioritized since it is already studied in R18 AI/ML for Air SI. Also, our system level simulator adopts all the agreed mandatory simulation assumption that should be used by all the companies as summarized in Annex part.

### 2.1.1 General evaluation parameters/assumptions

As a result of the offline e-mail discussion [POST126][031], some simulation parameters/assumptions are agreed to be reported by the companies with their simulation results [1]. The following Table 1 shows the set of parameters commonly used for all the cases in our evaluation. The other parameters specific to each case are reported in the following sections for each case, separately.

Table 1

| Report parameters |  | Samsung |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 3 |
|  | UE trajectory boundary processing option | boundary-terminated model |
|  | UE speed (30,60,90,120 Km/h) | 120 Km/h |
|  | Whether LOSsoft is modeled or not | Yes |
|  | Spatial consistency option (A or B in TR 38.901) | A |
|  | Number of TX beams | 1 (for FR1) / 32 (for FR2) |
|  | Number of RX beams | 1 (for FR1) / 1 (for FR2) |
|  | L1-filtering length | 40 |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM autoencoder 2 LSTM layers for encoder 2 LSTM layers + 3 dense layers for decoder |
|  | Model complexity in a number of parameters(M) | 4.5K |
|  | Model complexity in model size (e.g. Mbyte) | ~200 KByte |
|  | Computational complexity [FLOPs] | ~20K |

### 2.1.2 FR2 to FR2 intra-frequency temporal domain case A (Case 4)

The following Table 2 shows our simulation assumption and results for the Case 4 (i.e., temporal domain prediction Case A in FR2).

Table 2

| Report parameters |  | Samsung |  |
| --- | --- | --- | --- |
| Reported simulation assumptions | Observation window | 20/100/400/800/1600 msec (1/5/20/40/80 sampling periods) |  |
|  | Prediction window | 400msec (20 sampling periods) |  |
| Data Size (Number of Samples) | Training/validity | 31000/3500 samples* |  |
|  | Testing | 9000 samples* |  |
|  | Model complexity in model size (e.g. Mbyte) | ~200 Kbyte |  |
|  | Computational complexity [FLOPs] | 20K |  |
| AI/ML model input/output | Model input | Historical L3 cell-level RSRP within observation window |  |
|  | Model output | Future L3 cell-level RSRP within prediction window |  |
| Metrics | Average L3 cell level RSRP difference (dB) | Observation window | RSRP diff. (dB) |
|  |  | 20 msec (1 sample) | 1.767 |
|  |  | 100 msec (5 samples) | 1.452 |
|  |  | 400 msec (20 samples) | 1.419 |
|  |  | 800 msec (40 samples) | 1.4 |
|  |  | 1600 msec (80samples) | 1.384 |

*Note: Each sample consists of input RSRP values and output RSRP values.

For Case A prediction, the model is trained to predict the future L3 cell-level RSRP within the prediction window based on the historical L3 cell-lever RSRP within the observation window. Here, we fix the prediction window as the agreed initial value (i.e., 400 msec for FR2) and evaluate the prediction accuracy with different observation window size. The results show that the average L3 RSRP difference can be 1.77/1.45/1.42/1.4/1.38 dB for distinct observation window lengths of 1/5/20/40/80 samples respectively.

Observation 1: For Case A temporal domain prediction in FR2, the evaluation results show that that the average L3 RSRP difference can be 1.77/1.45/1.42/1.4/1.38 dB for distinct observation window lengths of 1/5/20/40/80 samples respectively.

From the evaluation results, it is observed that the L3 RSRP difference can decrease (i.e., prediction accuracy can be improved) as we use the longer observation window size. This makes sense because a longer observation window provides more information for the AI/ML model, which can be used to improve the prediction accuracy. However, we can also see that the rate of accuracy improvement decreases as the observation window lengthens.

Observation 2: As we use a longer observation window, the L3 RSRP difference can decrease (i.e., prediction accuracy can be improved). However, the rate of accuracy improvement decreases as the observation window lengthens.

On the other hand, the longer observation window can increase the signalling/memory overhead in the model training/inference procedure. For example, if we assume the use of gNB-side model for RRM measurement prediction, the UE is required to report the input data (i.e., historical L3 RSRP measurement within the observation window) to the gNB for inference. Therefore, in this case, the use of longer observation window will increase the signalling overhead between UE and gNB and also require larger memory size at the UE side. The same goes for the UE-side model as well since the UE needs to provide the label data (i.e., set of input and output data) to the training entity (e.g., UE-side OTT server).

Observation 3: For RRM measurement prediction, the use of a longer observation window can increase the signalling/memory overhead in LCM procedure (especially for model training/inference).

Based on the observations above, there exists a trade-off between prediction accuracy and signalling/memory overhead depending on the length of observation window. Therefore, we cannot use continuously extend the observation window to improve the prediction accuracy but instead should determine a suitable value for it. In our evaluation,’20 samples’ is chosen as a reasonable value for the observation window length, and this value is commonly used in the other cases (i.e., Case 2/3) as well.

Observation 4: There is a trade-off between prediction accuracy and signaling/memory overhead depending on the length of observation window. A reasonable value for the length of observation window should be determined considering the trade-off relation.

Proposal 1: For RRM measurement prediction, RAN2 to capture the following observations in TR 38.744 to explain the effect of observation window length on prediction accuracy and signaling/memory overhead.

- As we use a longer observation window, the L3 RSRP difference can decrease (i.e., prediction accuracy can be improved).

- The use of a longer observation window can increase the signalling/memory overhead in LCM procedure (especially for model training/inference).

- There is a trade-off between prediction accuracy and signaling/memory overhead depending on the length of observation window.

- A reasonable value for the length of observation window should be determined considering the trade-off relation.

For cell level RRM measurement prediction, RAN2 agreed to study the following sub-cases at RAN2#125bis.

- Sub-case 1: To predict beam level results, then generate cell level results based on the predicted beam results.
(Input: L1 beam-level RSRP, Output: L1 beam-level RSRP  L3 cell-level RSRP after consolidation)

- Sub-case 2: To directly predict cell level results based on cell level results.
(Input: L3 cell-level RSRP, Output: L3 cell-level RSRP)

- Sub-case 3: To directly predict cell level results based on beam level results
(Input: L1 beam-level RSRP, Output: L3 cell-level RSRP)

To check the performance of the sub-cases 1/2/3, we evaluate the accuracy of RRM measurement prediction (i.e., L3 RSRP difference) for each of the sub-cases with the same parameters in Table 1, except the model input/output. The following Table 3 shows the evaluation results.

Table 3

|  | Sub1 (L1 beam L1 beam) | Sub2 (L3 cell L3 cell) | Sub3 (L1 beam L3 cell) |
| --- | --- | --- | --- |
| L3 RSRP difference | 1.62 dB | 1.43 dB | 1.42 dB |

Observation 5: For Case A temporal domain prediction in FR2, the evaluation results show that the average L3 cell level RSRP difference can be 1.62/1.43/1.42 dB for each of the sub-case 1/2/3 respectively.

In our view, the important point here is whether the use of L1 beam-level RSRP input really helps improve the prediction accuracy and the answer from the result is ‘No’. We can see that the subcase 2 with L3 cell-level input shows better or almost same performance compared to the subcase 1&3 with L1 beam-level input in prediction accuracy. On the other hand, it is obvious that the use of L1 beam-level RSRP will result in an increase in signalling/memory overhead for model training/inference. The input data size will increase by up to 64 times when we assume the use of L1 beam-level RSRP. In summary, the use of L1 beam-level input can increase the signalling/memory overhead without much gain in RRM measurement prediction.

Observation 6: Sub-case 2 with L3 cell-level input shows better or almost same prediction accuracy compared to Sub-case 1&3 with L1 beam-level input.

Observation 7: The use of L1 beam-level input can increase signalling/memory overhead without much gain in RRM measurement prediction.

Based on the observation above, we would like to propose the following.

Proposal 2: For RRM measurement prediction, RAN2 to capture the following observation on the use of L1 beam-level input in TR 38.744.

- Sub-case 2 with L3 cell-level input shows better or almost same prediction accuracy compared to Sub-case 1&3 with L1 beam-level input.

- The use of L1 beam-level input can increase signalling/memory overhead without much gain in RRM measurement prediction.

### 2.1.3 FR1 to FR1 intra-frequency temporal domain case B (Case 2)

The following Table 4 shows our simulation assumption for the Case 2 (i.e., temporal domain prediction Case B in FR1).

Table 4

| Report parameters |  | Samsung |
| --- | --- | --- |
| Reported simulation assumptions | Measurement reduction rate (R) | 1/2, 3/4, 5/6, 7/8 |
| Data Size (Number of Samples) | Training/validity | 59000/6500 samples* |
|  | Testing | 9500 samples* |
| AI/ML model input/output | Model input | Historical L3 cell level RSRP with the interval (= sampling period * 1/(1-R) ) |
|  | Model output | Future L3 cell level RSRP with the interval (= sampling period) |

*Note: Each sample consists of input RSRP values and output RSRP values.

For Case B prediction, the model is trained to predict the future L3 cell-level RSRP with the interval of ‘sampling period (i.e., 20msec)’ based on the historical L3 cell-level RSRP with the interval of ‘sampling period *1/(1-R)’. For example, let’s assume the case with measurement reduction rate (= 3/4) as in Figure 1 below. In this scenario, at the specific time instance ‘Tn’, the model inference can be performed to predict the future RSRP values for 3 distinct time instances between the current (Tn) and the subsequent (Tn+1) measurement time instance. With the assumption, the number of prediction time instances can be 1, 3, 5, 7 for the distinct measurement reduction rates 1/2, 3/4, 5/6, 7/8, respectively.

Figure 1. Example of Case B prediction with the measurement reduction rate (=3/4)

Table 5

| Metrics | Average L3 cell level RSRP difference (dB) | Reduction rate | RSRP diff. (dB) |
| --- | --- | --- | --- |
|  |  | 1/2 | 0.188 |
|  |  | 3/4 | 0.282 |
|  |  | 5/6 | 0.417 |
|  |  | 7/8 | 0.526 |

Table 5 shows the evaluation results that the average L3 cell level RSRP difference can be 0.19, 0.28, 0.42, 0.53 dB for each of the different measurement reduction rates 1/2, 3/4, 5/6, 7/8, separately. From the results, we can also find the tradeoff between measurement reduction rate and prediction accuracy. As the higher measurement reduction rate is used for Case B prediction, the measurement overhead at the UE side decreases, but the RSRP difference increases, which can result in HO KPI degradation. From UE’s perspective, it can prefer a higher measurement reduction rate to reduce its measurement overhead, but NW may be conservative about reducing the measurement overhead too much to maintain overall HO performance. Thus, we need to study how UE/gNB can determine or negotiate a suitable value of measurement reduction rate for Case B prediction.

Observation 8: For Case B temporal domain prediction in FR1, the evaluation results show that the average L3 cell level RSRP difference can be 0.19, 0.28, 0.42, 0.53 dB for each of the measurement reduction rate 1/2, 3/4, 5/6, 7/8 respectively.

Observation 9: For Case B prediction, there is a tradeoff between measurement reduction rate and prediction accuracy (i.e., L3 RSRP difference).

Based on the observations above, we would like to propose the following.

Proposal 3: For Case B temporal domain prediction, RAN2 to capture the tradeoff between measurement reduction rate and prediction accuracy in TR 38.744.

Proposal 4: RAN2 to study how UE/gNB can determine a suitable value of measurement reduction rate for Case B prediction.

### 2.1.4 FR1 to FR1 inter-frequency (frequency domain, Case 3)

The following Table 6 shows our simulation assumption/results for the Case 3 (i.e., inter-frequency prediction in FR1).

Table 6

| Report parameters |  | Samsung |
| --- | --- | --- |
| Reported simulation assumptions | Inter-frequency correlation assumption in general (yes or no) | Yes |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no) | Full |
| Data Size (Number of Samples) | Training/validity | 59000/6500 samples* |
|  | Testing | 9500 samples* |
| AI/ML model input/output | Model input | L3 cell level RSRP at the baseline freq. (4GHz) |
|  | Model output | L3 cell level RSRP at another freq. (2GHz) |
| Metrics | Average L3 cell level RSRP difference (dB) | 2.87 |

*Note: Each sample consists of input RSRP values and output RSRP values.

For inter-freq. prediction, the model is trained to predict L3 cell-level RSRP of the cell in the inter-frequency based on L3 cell-level RSRP of the cell in the serving frequency. Specifically, we use 20 samples of the measured L3 RSRP in 4GHz to predict the 20 samples of the L3 RSRP at the same time instances in 2GHz. For the inter-freq. correlation, we assume the case where the cell to be measured and the cell to be predicted are co-located (i.e., located in the same sector/site with the same antenna configuration) and adopt the inter-frequency shadow fading model in TR 38.901.

Figure 2. L3 RSRP of the two cells in different frequency

Figure 2 depicts the true L3 RSRP in 4GHz, the true L3 RSRP in 2GHz, and the predicted L3 RSRP in 2GHz. In general, the results show quite a high correlation between the L3 RSRP of the two cells in different frequency. The correlation coefficient between the L3 RSRP of the two cells is observed as 0.9125.

Observation 10: For the inter-frequency prediction, the evaluation results show a high inter-frequency correlation between the L3 cell-level RSRP of the two co-located cells. The correlation coefficient is observed as 0.9125.

With the high correlation between L3 RSRP of the two cells, we can expect a high prediction accuracy (i.e., low RSRP difference) for the inter-frequency prediction. However, the actual RSRP difference for inter-frequency prediction is evaluated as 2.87 dB that is a bit larger compared to the values in other cases (i.e., the temporal domain prediction Case A&B).

In the inter-frequency prediction, unlike other cases, the L3 RSRP of the cell to be predicted is not provided to the model as input data. Additionally, according to the inter-frequency channel model in TR 38.901, there is no assumed correlation in terms of fast fading between the two different frequencies. Therefore, the observed RSRP difference seems reasonable to us. In our view, for the inter-frequency prediction, the model can be trained to learn the average offset between the L3 RSRP of the two distinct cells, which represents the best outcome we can expect.

Observation 11: For inter-frequency prediction in FR1, the evaluation results show that the average L3 cell level RSRP difference is 2.78 dB when the co-located two cells are operating in 2GHz and 4GHz, separately.

Observation 12: Despite the high correlation between the L3 RSRP of the two cells in different freq., the L3 RSRP difference in inter-frequency prediction is a bit larger compared to the values in other cases (i.e., temporal domain prediction Case A&B).

In our view, the next step should be to check whether the prediction accuracy with an error of about 3dB is good enough to execute the inter-frequency HO without the inter-frequency measurement.

Proposal 5: RAN2 to discuss what level of prediction accuracy (i.e., RSRP difference) is enough to execute the HO without inter-frequency measurement.

# 3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

FR2 to FR2 intra-frequency temporal domain case A (Case 4):

Observation 1: For Case A temporal domain prediction in FR2, the evaluation results show that that the average L3 RSRP difference can be 1.77/1.45/1.42/1.4/1.38 dB for distinct observation window lengths of 1/5/20/40/80 samples respectively.

Observation 2: As we use a longer observation window, the L3 RSRP difference can decrease (i.e., prediction accuracy can be improved). However, the rate of accuracy improvement decreases as the observation window lengthens.

Observation 3: For RRM measurement prediction, the use of a longer observation window can increase the signalling/memory overhead in LCM procedure (especially for model training/inference).

Observation 4: There is a trade-off between prediction accuracy and signaling/memory overhead depending on the length of observation window. A reasonable value for the length of observation window should be determined considering the trade-off relation.

Proposal 1: For RRM measurement prediction, RAN2 to capture the following observations in TR 38.744 to explain the effect of observation window length on prediction accuracy and signaling/memory overhead.

- As we use a longer observation window, the L3 RSRP difference can decrease (i.e., prediction accuracy can be improved).

- The use of a longer observation window can increase the signalling/memory overhead in LCM procedure (especially for model training/inference).

- There is a trade-off between prediction accuracy and signaling/memory overhead depending on the length of observation window.

- A reasonable value for the length of observation window should be determined considering the trade-off relation.

Observation 5: For Case A temporal domain prediction in FR2, the evaluation results show that the average L3 cell level RSRP difference can be 1.62/1.43/1.42 dB for each of the sub-case 1/2/3 respectively.

Observation 6: Sub-case 2 with L3 cell-level input shows better or almost same prediction accuracy compared to Sub-case 1&3 with L1 beam-level input.

Observation 7: The use of L1 beam-level input can increase signalling/memory overhead without much gain in RRM measurement prediction.

Proposal 2: For RRM measurement prediction, RAN2 to capture the following observation on the use of L1 beam-level input in TR 38.744.

- Sub-case 2 with L3 cell-level input shows better or almost same prediction accuracy compared to Sub-case 1&3 with L1 beam-level input.

- The use of L1 beam-level input can increase signalling/memory overhead without much gain in RRM measurement prediction.

FR2 to FR2 intra-frequency temporal domain case A (Case 4):

Observation 8: For Case B temporal domain prediction in FR1, the evaluation results show that the average L3 cell level RSRP difference can be 0.19, 0.28, 0.42, 0.53 dB for each of the measurement reduction rate 1/2, 3/4, 5/6, 7/8 respectively.

Observation 9: For Case B prediction, there is a tradeoff between measurement reduction rate and prediction accuracy (i.e., L3 RSRP difference).

Proposal 3: For Case B temporal domain prediction, RAN2 to capture the tradeoff between measurement reduction rate and prediction accuracy in TR 38.744.

Proposal 4: RAN2 to study how UE/gNB can determine a suitable value of measurement reduction rate for Case B prediction.

FR1 to FR1 inter-frequency (frequency domain, Case 3):

Observation 10: For the inter-frequency prediction, the evaluation results show a high inter-frequency correlation between the L3 cell-level RSRP of the two co-located cells. The correlation coefficient is observed as 0.9125.

Observation 11: For inter-frequency prediction in FR1, the evaluation results show that the average L3 cell level RSRP difference is 2.78 dB when the co-located two cells are operating in 2GHz and 4GHz, separately.

Observation 12: Despite the high correlation between the L3 RSRP of the two cells in different freq., the L3 RSRP difference in inter-frequency prediction is a bit larger compared to the values in other cases (i.e., temporal domain prediction Case A&B).

Proposal 5: RAN2 to discuss what level of prediction accuracy (i.e., RSRP difference) is enough to execute the HO without inter-frequency measurement.

# 4	Reference

- R2-2406095, “Summary of [POST126][031][AIMob] Simulations (OPPO)”, OPPO

# Annex: Simulation assumptions

Table 5.1-1 Simulation assumptions of FR1 and FR2

| Parameters | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz, 15/30KHz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and optional LOSsoft; Inter-frequency correlation model is optional. without UErotation,Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and optional LOSsoft; without UErotation,Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage |
| System BW | 20MHz | 80MHz |
| UE speed | 30,60,90 km/h for study targeting measurement reduction 60,90,120 km/h for study targeting HO performance improvement | 30,60,90 km/h for study targeting measurement reduction 60,90,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Other assumptions are not precluded. |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Other assumptions are not precluded |
| UE Antenna radiation pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| BS Tx Power | 44dBm | 40 dBm (baseline) Other values (e.g., 34 dBm) not precluded |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | companies report one of the spatial consistency procedures: - Procedure A in TR38.901 - Procedure B in TR38.901 | companies report one of the spatial consistency procedures: - Procedure A in TR38.901 - Procedure B in TR38.901 |
| UE trajectory model | 3 options in 38.843 section 6.3.1 | 3 options in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | Companies report which of the following models they used: wrap-around model, circle-bouncing model, boundary-terminated model | Companies report which of the following models they used: wrap-around model, circle-bouncing model, boundary-terminated model |
| Sampling period | 40ms | 20ms |