3GPP TSG-RAN WG2 Meeting #127                                   R2-2406861

Maastricht, Netherlands, 19th August - 23th August, 2024

Agenda item:		8.3.2.2

Source:		Samsung

Title:			Discussion on temporal domain RRM measurement prediction

Document for:	Discussion & Decision

# 1	Introduction

At RAN2#125bis, RAN2 made the following agreements for temporal domain measurement prediction Case A&B.

(RAN2#125bis) - For temporal domain measurement prediction, we will consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level. As baseline we will focus on pure temporal predicition.

In this contribution, we would like to discuss our observation on temporal domain prediction Case A & B and propose to clarify the concept/usage of them.

# 2	Discussion

## 2.1	Definition of Case A/B

For temporal domain measurement prediction, RAN2 agreed to consider Case A and Case B from the RAN1 AI/ML PHY TR, TR 38.843 [1]. In the TR, Case A and Case B are described as below.

To calculate the measurement/RS overhead reduction and summarize results for BM-Case 2, at least when Top-1 beam (pair) is inferred: - Case A: based on number of measurements/RSs and prediction time. An example is shown in Figure 6.3.1-2. - where T2 is the time duration for beam prediction - where Mt is the number of time instances for measurement as AI/ML inputs with a periodicity of Tper - where Pt is the number of time instance(s) for prediction with a periodicity of Tper in T2 - In this case, the non-AI baseline is Option 1 (measured all the beams at each time instance(s) for prediction with a periodicity of Tper in T2) - For Set B= Set A, the RS overhead reduction is 1-Mt/(Mt+Pt). - For Set B (N beams, same number in each time instance) is a subset of Set A (M beams), the RS overhead reduction is - N*Mt/(M*(Mt+Pt)) if no sliding window - 1-N/M if considering sliding window - Case B: based on a periodicity T of the required reference signals for measurements to achieve a certain beam prediction accuracy. An example is shown in Figure 6.3.1-3. - For non-AI baseline (Option 2), every T=X ms reference signals for measurements are needed - For AI, every T=Y ms, reference signals for measurements are needed - In this case, - For Set B = Set A, the RS overhead reduction is 1-X/Y. - For Set B (N beams) is a subset of Set A (M beams), the RS overhead reduction is [1-XN/(YM)].

From the description, it is understood that RAN1 consider both Case A and Case B for measurement/RS overhead reduction. On the other hands, in RAN2 discussion for R19 AI/ML SI, only Case B is considered for measurement reduction (i.e., for 1st study goal). For Case A, we assume the case with sliding window and thus there is no measurement reduction. Additionally, in the description in RAN1 TR, the RS/measurement reduction rate is calculated considering not only the temporal domain prediction (e.g., Mt, Pt, X, Y) but also the spatial domain prediction (e.g., N, M). However, now in RAN2 discussion, both Case A and Case B are applicable only for temporal domain prediction.

Observation 1: The current interpretation on Case A&B in RAN2 is different from the one from R18 AI/ML for air SI.

Therefore, we prefer to have separate definitions of Case A&B for RAN2 discussion and capture them in RAN2 TR 38.744. Based on the agreement/discussion until now, we would like to propose the following.

Proposal 1: RAN2 to capture the following TP in TR 38.744 to have separate definitions of Case A&B for R19 AI/ML for mobility discussion.

- Case A: Temporal domain prediction for the 2nd study goal (i.e., HO KPI improvement).

  - An example is shown in Figure 1.

    - where Tper is the measurement/prediction interval (e.g., SMTC periodicity)

    - where Mt is the number of time instances for measurement in observation window

    - where Pt is the number of time instances for prediction in prediction window

  - In this case,

    - AI/ML model is used to predict future measurement result(s) in prediction window based on actual (past) measurement result(s) in observation window

    - Observation window & Prediction window are sliding.

    - UE performs measurement every Tper as in legacy. There is no measurement reduction.

- Case B: Temporal domain prediction for the 1st study goal (i.e., measurement reduction).

  - An example is shown in Figure 2. 
(Note that it is just one of the examples for Case B. The specific measurement skip pattern is up to companies.)

    - where Tper is the prediction interval (e.g., SMTC periodicity)

    - where Tper’ is the measurement interval (e.g., multiples of Tper)

  - In this case,

    - AI/ML model is used to predict future measurement results at the measurement instances to be skipped based on actual (past) measurement result(s)

    - UE performs measurement every Tper’ (e.g., multiples of Tper) by skipping some measurement instances

    - The measurement reduction rate in the temporal domain (MRRT) is 1-Tper/Tper’

Figure 1. Example of Case A

Figure 2. Example of Case B

## 2.2	Use of temporal domain prediction results

Regarding temporal domain RRM prediction, RAN2 have spent lots of time to make the consensus on the concept of Case A&B and the relation between Case A/B and our 1st/2nd study goal. Based on the discussion until now, in general, we can say that Case A is mainly for HO KPI improvement (i.e., 2nd goal) and Case B is for measurement reduction (i.e., 1st goal). However, it still seems unclear to us how the output of temporal domain prediction (i.e., predicted future measurement results) can be used for the 1st study goal and the 2nd study goal, in practice. Thus, we would like to further discuss that aspect below.

For the 2nd study goal, the output of temporal domain prediction should be used to improve HO KPI. Specifically, the gNB can optimize HO configuration/timing based on the predicted future measurement results. In case of UE-side model, the UE can also use the predicted results for RRM measurement event prediction or RLF/HOF prediction and report the predicted measurement/RRM measurement event/RLF/HOF to the gNB. RAN2 mainly consider Case A prediction (i.e., far future prediction without measurement reduction) for this goal. In Case A prediction, since the UE can get the actual measurement results every Tper (e.g., SMTC periodicity) as in legacy, there is no impact on the legacy RRM operation. I.e., measurement results are used for legacy RRM operations as before. In this approach, measurement results and prediction results are used independently.

Observation 2: For the 2nd study goal, the output of temporal domain prediction (e.g., predicted L3 RSRP) can be used for HO configuration/timing optimization, RRM measurement event prediction, and HOF/RLF prediction.

Observation 3: For the 2nd study goal, since the UE performs measurement every Tper (e.g., SMTC periodicity) as before, there is no impact on the legacy RRM operation. Measurement results and prediction results are used independently.

For the 1st study goal, the output of temporal domain prediction should be used to reduce measurement overhead without HO KPI degradation. We can consider Case B prediction (i.e., near future prediction with measurement reduction) for this goal. For Case B prediction, the UE can skip some measurement time instances to reduce the measurement overhead. Since the UE cannot obtain actual measurement result every Tper (e.g., SMTC periodicity), the RRM operation becomes inaccurate and HO KPI could be degraded in the end. To prevent the HO KPI degradation, both measurement results and prediction results are jointly used for legacy RRM operations (e.g., event-triggered measurement reporting). Figure 3 below shows how measurement results and prediction results are jointly used for legacy RRM operations.

Figure 3. Example of legacy RRM operation (e.g., MR triggering for Event A2) with Case B prediction

In Figure 3, the entering condition for A2 event starts to be fulfilled with the prediction result at time T1. Then, when the condition is fulfilled for all the measurement & prediction results during TTT (i.e., from T1 to T2), the measurement report for A2 event can be triggered at T2. This shows how measurement results and prediction results are jointly used for legacy RRM operations. On the other hand, one might think that the measurement report can be triggered a bit earlier at T2’ since the UE can already have the prediction results until T2 at that timing. However, such a case can be understood as a new AI/ML RRM operation (i.e., measurement event prediction) rather than legacy RRM operations.

Observation 4: For the 1st study goal, the output of temporal domain prediction (e.g., predicted L3 RSRP) can be used to reduce measurement overhead without HO KPI degradation.

Observation 5: For the 1st study goal, since the UE skips some measurements to reduce measurement overhead, RRM operations can become inaccurate. To alleviate the problem, both measurement results and prediction results are jointly used for the legacy RRM operation (e.g., event-triggered measurement reporting).

In summary, we can consider the following two general approaches on how to use measurement results and prediction results for RRM:

- Approach 1: Both measurement results and prediction results are jointly used for legacy RRM.

  - RRM operation is performed based on both measurement results and prediction results.

  - RRM operation is applied at the timing that prediction result is applied rather than at the timing that prediction result is acquired.

- Approach 2: Measurement results and prediction results are used independently.

  - Measurement results are used for legacy RRM operations.

  - Prediction results are used for HO configuration/timing optimization, RRM measurement event prediction, and HOF/RLF prediction.

Based on the observations above, we would like to propose the following.

Proposal 2: For temporal domain RRM measurement prediction, RAN2 to discuss the following two general approaches on how to use measurement results and prediction results for RRM.

- Approach 1: Both measurement results and prediction results are jointly used for legacy RRM.

  - RRM operation is performed based on both measurement results and prediction results.

  - For prediction results, RRM operation is applied at the timing that prediction result is applied rather than at the timing that prediction result is acquired.

- Approach 2: Measurement results and prediction results are used independently.

  - Measurement results are used for legacy RRM operations.

  - Prediction results are used for HO configuration/timing optimization, RRM measurement event prediction, and HOF/RLF prediction.

# 3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Observation 1: The current interpretation on Case A&B in RAN2 is different from the one from R18 AI/ML for air SI.

Proposal 1: RAN2 to capture the following TP in TR 38.744 to have separate definitions of Case A&B for R19 AI/ML for mobility discussion.

- Case A: Temporal domain prediction for the 2nd study goal (i.e., HO KPI improvement).

  - An example is shown in Figure 1.

    - where Tper is the measurement/prediction interval (e.g., SMTC periodicity)

    - where Mt is the number of time instances for measurement in observation window

    - where Pt is the number of time instances for prediction in prediction window

  - In this case,

    - AI/ML model is used to predict future measurement result(s) in prediction window based on actual (past) measurement result(s) in observation window

    - Observation window & Prediction window are sliding.

    - UE performs measurement every Tper as in legacy. There is no measurement reduction.

- Case B: Temporal domain prediction for the 1st study goal (i.e., measurement reduction).

  - An example is shown in Figure 2. 
(Note that it is just one of the examples for Case B. The specific measurement skip pattern is up to companies.)

    - where Tper is the prediction interval (e.g., SMTC periodicity)

    - where Tper’ is the measurement interval (e.g., multiples of Tper)

  - In this case,

    - AI/ML model is used to predict future measurement results at the measurement instances to be skipped based on actual (past) measurement result(s)

    - UE performs measurement every Tper’ (e.g., multiples of Tper) by skipping some measurement instances

    - The measurement reduction rate in the temporal domain (MRRT) is 1-Tper/Tper’

Figure 1. Example of Case A

Figure 2. Example of Case B

Observation 2: For the 2nd study goal, the output of temporal domain prediction (e.g., predicted L3 RSRP) can be used for HO configuration/timing optimization, RRM measurement event prediction, and HOF/RLF prediction.

Observation 3: For the 2nd study goal, since the UE performs measurement every Tper (e.g., SMTC periodicity) as before, there is no impact on the legacy RRM operation. Measurement results and prediction results are used independently.

Observation 4: For the 1st study goal, the output of temporal domain prediction (e.g., predicted L3 RSRP) can be used to reduce measurement overhead without HO KPI degradation.

Observation 5: For the 1st study goal, since the UE skips some measurements to reduce measurement overhead, RRM operations can become inaccurate. To alleviate the problem, both measurement results and prediction results are jointly used for the legacy RRM operation (e.g., event-triggered measurement reporting).

Proposal 2: For temporal domain RRM measurement prediction, RAN2 to discuss the following two general approaches on how to use measurement results and prediction results for RRM.

- Approach 1: Both measurement results and prediction results are jointly used for legacy RRM.

  - RRM operation is performed based on both measurement results and prediction results.

  - For prediction results, RRM operation is applied at the timing that prediction result is applied rather than at the timing that prediction result is acquired.

- Approach 2: Measurement results and prediction results are used independently.

  - Measurement results are used for legacy RRM operations.

  - Prediction results are used for HO configuration/timing optimization, RRM measurement event prediction, and HOF/RLF prediction.

# 4	Reference

- 3GPP TR 38.843: “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.