3GPP TSG-RAN WG2 Meeting #127		R2-2406863

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.6.2

Source: 	ITRI

Title:  	Cell switch command for subsequent inter-CU LTM

Document for:	Discussion and decision

Introduction

In RAN plenary #104, a revised Work Item on NR mobility enhancements Phase 4 was approved in RP-241515 [1]. Objective #1 of this Work Item is to specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM).

As described in the Work Item, the Rel-18 introduced LTM can offer improvements in handover latency and interruption time compared to Layer 3 based mobility. However, its operation is only supported for mobility between cells of the same gNB (same CU). Depending on the deployment of the network this may significantly limit the opportunities to use LTM. To gain the benefits of LTM for a far greater number of handovers, Objective #1 of Rel-19 NR mobility enhancements is to enable LTM operation between cells of different gNBs (i.e. inter-CU). The content of Objective #1 is listed below.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

Discussion

In the previous RAN2 meeting, we reached an agreement that Rel-18 LTM Cell Switch Command (CSC) is baseline to trigger Rel-19 inter-CU LTM cell switch [2].

RAN2#126 Agreement:

Execution:

LTM Cell Switch Command

- R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

As shown in the first section, support for subsequent LTM mobility procedure is listed within Objective #1 aiming at inter-CU LTM. In addition, the following agreements on Rel-19 inter-CU LTM were made in RAN2#125bis meeting [3]:

RAN2#125bis Agreements:

- Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

- UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells.

Based on Objective #1 and the first RAN2#125bis agreement shown above, the serving gNB may be changed during Rel-19 subsequent inter-CU and intra-CU LTM mobility procedure.

Considering Rel-18 LTM procedure, where the cell switch command is sent by the serving gNB, is taken as the baseline, in our view, the cell switch command is sent by the current serving gNB in Rel-19 subsequent inter-CU LTM as well.

Moreover, the second RAN2#125bis agreement above shows that UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells. In our understanding, this means UE measurements for both intra-CU and inter-CU candidate LTM cells are reported to the current serving gNB. Therefore, the current serving gNB is the only RAN node that can decide whether a cell switch should be triggered. In other words, the cell switch should be decided by the current serving gNB, rather than an anchor node e.g. a previous serving gNB. After making the cell switch decision, the current serving gNB can send the corresponding cell switch command to UE by itself. In this way, the cell switch can be triggered immediately after it is decided, i.e. no triggering delay is induced.

Proposal 1: The current serving gNB is responsible for making a cell switch decision and sending the Cell Switch Command (CSC) in Rel-19 subsequent inter-CU LTM.

According to the first RAN2#125bis agreement shown above, Rel-19 inter-CU LTM supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch, where an inter-CU LTM requires PDCP re-establishment while an intra-CU LTM does not. As RAN2 has agreed on using Cell Switch Command (CSC) to trigger an LTM cell switch in Rel-19 inter-CU LTM, we propose including an indication in the CSC to tell UE whether PDCP entity should be re-established. CSC is what the network uses to command UE to perform a LTM cell switch, so it is natural for the CSC to include how UE should perform the cell switch e.g. whether PDCP entity should be re-established.

Proposal 2: Include an indication in the Cell Switch Command (CSC) to indicate whether PDCP entity should be re-established.

Conclusion

In this contribution, the following proposals are made:

Proposal 1: The current serving gNB is responsible for making a cell switch decision and sending the Cell Switch Command (CSC) in Rel-19 subsequent inter-CU LTM.

Proposal 2: Include an indication in the Cell Switch Command (CSC) to indicate whether PDCP entity should be re-established.

Reference

- RP-241515 Revised Work Item: NR mobility enhancements Phase 4, RAN#104, June 2024.

- R2-240xxxx Draft Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan, 20 - 24 May, 2024.

- R2-2404002 Draft Report of 3GPP TSG RAN WG2 meeting #125bis, Changsha, China, 15 - 19 April, 2024.