3GPP TSG-RAN WG2 Meeting #127	R2-2406864

Maastricht, Holland, Aug 19th – 23rd, 2024

Agenda Item:	8.7.2

Source:	ITRI

Title:	Discussion on scheduling enhancements for multi-modal traffic

Document for:	Discussion

1 Introduction

In RAN2 #126, RAN2 agreed to support Multi-Modality awareness in RAN in Rel-19 for UL and DL, and sent a LS to ask SA2 whether/what information could be provided to RAN. The next step is to focus on potential RAN enhancements to efficiently support multi-modal XR applications.

In this document, we discuss scheduling enhancements for multi-modal traffic to meet synchronization requirements.

2 Discussion

First, we justify the need to support synchronization for MM (multi-modality) at RAN. Then some scheduling enhancements are proposed to study.

2.1 Supporting synchronization for multi-modality at RAN

In TR 22.847 [1], the definitions of multi-modal data and synchronization threshold are as follows,

- Multi-modal Data: Multi-modal Data is defined to describe the input data from different kinds of devices/sensors or the output data to different kinds of destinations (e.g. one or more UEs) required for the same task or application. Multi-modal Data consists of more than one Single-modal Data, and there is strong dependency among each Single-modal Data. Single-modal Data can be seen as one type of data.

- Synchronisation threshold: A multi-modal synchronisation threshold can be defined as the maximum tolerable temporal separation of the onset of two stimuli, one of which is presented to one sense and the other to another sense, such that the accompanying sensory objects are perceived as being synchronous.

Based on the above descriptions, we draw an illustration of multi-modal synchronization at application layer in Figure 1. An UE has two MM services, and Figure 1 is considered from an UL point of view. Each MM service has three service flows, tactile (black), visual (red) and audio (blue), and each of them has their own periodicity and traffic pattern. From TR 22.847 [1], the pre-conditions is that based on the service agreement between MNO and immersive multi-modal VR application operator, the application operator may in advance provide the 5G network with the application information including the application traffic characteristics and the service requirement for network connection. The post-conditions is that the user experiences the immersive game reality application enabled by 5G network, and the 5G system address the service requirements of the application.

Figure 1 An example of multi-modal synchronization at application layer

WebRTC (Web Real-Time Communication) has been used to realize the synchronization of video and radio services nowadays. However, for MM XR applications, more complicated synchronization handling exists including the optimization of network transmission, device capability and congestion handling, etc. From our point of view, enhancements of 5G system are needed to address the synchronization requirements for MM traffic, so users will have good experience for MM services if the end-to-end synchronization can be realized at application layer. Some study about QoS handling had been contained in TR 23.700-60 [2]. For RAN, we think that it is beneficial to consider the RAN transmission with MM to avoid becoming the trouble maker of end-to-end synchronization.

Proposal 1: RAN2 is suggested to consider the RAN transmission with Multi-Modality to avoid becoming the trouble maker of end-to-end synchronization.

2.2 Scheduling enhancements for multi-modality at RAN

Before discussing the scheduling enhancements for MM at RAN, it is important to understand the mapping between QoS flow and DRB/LCH. RAN2 has the following agreements at RAN2 #125bis,

- RAN2 assumes that traffic of different modals having different QoS requirements is mapped to different QoS flows

- For different XR traffic flows belonging to the same Multi-modal service and having different QoS requirements, it should be possible to provide differentiated QoS handling over the air. RAN2 should study if that is possible with current mechanism or new ones are needed

- Existing QoS flow to DRB mapping framework is used as a baseline, i.e. up to gNB how to map QoS flows to DRBs

Since different DRBs have different treatments at RAN, we think it is more reasonable that different QoS flows having different QoS requirements of a MM service will be mapped to different DRBs.

Observation 1: different QoS flows having different QoS requirements of a MM service will be mapped to different DRBs.

Based on the different granularity of synchronization enhancement at RAN, two options are analyzed.

- Option 1: Support “Typical synchronization for multi-modality applications” per DRB/LCH

The granularity of inter-dependency is per DRB/LCH in Option 1, so it is simpler and needs less signalling overhead. We can have LCP enhancement at MAC and/or RLC bearer splitting. For example, there is one MM service of the user, and different QoS flows with different QoS requirements are mapped to different LCHs. Tactile flow is mapped to LCH 1, visual flow is mapped to LCH 2, audio flow is mapped to LCH 3 and best effort traffic is mapped to LCH 4. One LCP restriction enhancement based on Multi-modal Service ID (MMSID) may be used to pick up the mapped LCHs to use the UL resource simultaneously for synchronization of MM traffic.

- Option 2: Ensure multi-modality service synchronization by packet level

The granularity of inter-dependency is per packet level in Option 2, so additional synchronization information needs to be provided to UE/gNB. We can have LCP enhancement at MAC and/or PDCP. For example, since different type PDCP SDUs belong to one onset are expected to be received without exceed a synchronization threshold, we may adjust the PDCP discard timer for the PDCP SDUs which belong to one onset to ensure multi-modality service synchronization.

Proposal 2: RAN2 is suggested to consider scheduling enhancements to support synchronization for multi-modal traffic,

Option 1: Support “Typical synchronization for multi-modality applications” per DRB/LCH

Option 2: Ensure multi-modality service synchronization by packet level

Because of the inter-dependencies between the service flows of a MM service, there may be obsolete data buffered at PDCP which are still waiting for transmission if some dependent data had been decided to be discarded. For this situation, a UE may discard these obsolete data to avoid unnecessary transmissions. This concept is similar to the discard function introduced in Re-18, pdu-SetDiscard configuration. If RAN2 agree to study this issue, a LS is suggested to send to SA2 for their feedback.

Proposal 3: RNA2 is suggested to study avoiding unnecessary transmissions of obsolete data at PDCP.

3 Conclusion

In the previous sections we made the following observations:

Observation 1: different QoS flows having different QoS requirements of a MM service will be mapped to different DRBs.

Based on the discussion in the previous sections we propose the following:

Proposal 1: RAN2 is suggested to consider the RAN transmission with Multi-Modality to avoid becoming the trouble maker of end-to-end synchronization.

Proposal 2: RAN2 is suggested to consider scheduling enhancements to support synchronization for multi-modal traffic,

Option 1: Support “Typical synchronization for multi-modality applications” per DRB/LCH

Option 2: Ensure multi-modality service synchronization by packet level

Proposal 3: RNA2 is suggested to study avoiding unnecessary transmissions of obsolete data at PDCP.

4 References

- TR 22.847 Study on supporting tactile and multi-modality communication services

- TR 23.700-60 Study on XR (Extended Reality) and media services