3GPP TSG RAN WG2#127	R2-2406865

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.8.4

Source: 	ITRI

Title: 	Discussions on handling MRB(s) associated with intended service areas

Document for:	Discussion and Decision

1	Introduction

For Rel-19 NR NTN, broadcast services may include the use case that the intended service area is smaller than the coverage of a NTN cell [1].

RAN2 #126 has reached the following agreements for providing MBS broadcast service in NTN [2]:

Agreements: 1. For MBS broadcast service, both EFC and EMC are supported. 2. RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area. 3. MBS broadcast intended service area is provided via system information 4. For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS 5. When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

Further issues are left to the discussion in RAN2#127 meeting:

- Come back next meeting to check whether a UE may not (or should not) establish MRB(s) associated with a service limited to an intended service area when it is not located within the area (up to UE implementation how the UE detects it’s not in the intended service area)

- Come back next meeting to check whether a UE may (or should) release its established MRB(s) associated with a service limited to an intended service area when exiting the area (up to UE implementation how the UE detects it’s not in the intended service area)

In this contribution we discuss and propose to include the information of intended service area in MSBBroadcastsConfiguration, also discuss the feasibility for a UE to check whether it is allowed to establish or may release MRB(s) associated with a MBS broadcast service limited to an intended service area.

2	Discussion

It is agreed that when intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session.

An intended service area presents the information of a geographic area/TN coverage that only applies to MBS services. If the configuration of intended service areas is provided by System Information (e.g, SIB20 or SIB21, the modification of intended service areas would bring about System Information modification and impacts UE that are not interested in MBS services. On the other hand, when the intended service area is configured by MBSBroadcastConfiguration message transmitted on MCCH, the modification of intended service area would only be indicated to MBS interested UE. Therefore, it is proposed to include the configuration of intended service areas in the MBSBroadcastConfiguration message.

Observation 1: The changes of intended service area would impact non-MBS UE if intended service area for MBS services is provided by System Information

Proposal 1: It is proposed to include the intended service area for MBS services in the MBSBroadcastConfiguration message.

RAN2 agreed that when intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session [2].

There may be multiple intended service areas in the coverage of a NTN cell. For example, the coverage of a NTN cell may overlap two countries while a MBS service may intend to reach one of the two countries [3]. It is possible that a MBS session ID associated with multiple intended service areas, and vice versa.

The MBS session ID associated with a MBS service is provided by the MBSBroadcastConfiguration message.

To prevent duplicating the configuration of an intended service area for multiple MBS sessions, the configuration of an intended service area could be given an identity. The association of intended service area and a MBS session could be indicated by the identity and the association could be included in the MBSBroadcastConfiguration message.

Proposal 2: An intended service area could be given an identity. The association of intended service area and a MBS session could be indicated by the MBSBroadcastConfiguration message.

It could rely on network indication whether a MBS broadcast service is only allowed to be received by UE located in the intended service area.

If network indicates that a MBS session is only allowed to be acquired in the intended service area, a UE interested in the MBS session could check whether it is allowed to establish MRB(s) associated with the MBS session when:

- e.g., applying the broadcast MRB establishment procedure, as specified in subclause 5.9.3 of [4], upon start of the MBS session, upon entering a cell providing an MBS broadcast service the UE is interested in, upon becoming interested in the ongoing MBS broadcast service, upon removal of the UE capability limitations inhibiting reception of the ongoing MBS broadcast service UE is interested in; or

- e.g., UE acquiring GNSS information of UE location, or upon modification of the intended service areas.

Observation 2: UE could check whether it is allowed to establish MRB(s) associated with a MBS session when applying the broadcast MRB establishment procedure.

Based on network indication, when a MBS session is not allowed to be received by UE beyond the intended service areas, a UE checks whether the established MRB(s) should be released when the UE acquiring the GNSS information of UE location. It is up to UE implementation when to acquire the GNSS information of UE location.

Observation 3: UE could check whether it is exiting intended service area when the UE acquiring the GNSS information of UE location, and the UE should release the MRB(s) associated with the MBS session limited to the intended service area. It is up to UE implementation when to acquire the GNSS information of UE location.

Accordingly, the following proposals are provided:

Proposal 3: It is proposed that network could indicate whether a MBS broadcast service is only allowed to be received by UE located in the intended service area.

Proposal 4: According to network indication, a UE should not establish MRB(s) associated with a MBS session limited to an intended service area when it is not located within the area (up to UE implementation how the UE detects it’s not in the intended service area).

Proposal 5: According to network indication, a UE should release its established MRB(s) associated with a MBS session limited to an intended service area when exiting the area (up to UE implementation how the UE detects it’s not in the intended service area).

3	Conclusion

Based on the analysis in this contribution, the observations are listed in the following:

Observation 1: The changes of intended service areas would impact non-MBS UE if the information of intended service areas for MBS services is provided by System Information.

Observation 2: UE could check whether it is allowed to establish MRB(s) associated with a MBS session when applying the broadcast MRB establishment procedure.

Observation 3: UE could check whether it is exiting an intended service area associated with an MBS session when the UE acquiring the GNSS information of UE location, and the UE should release the MRB(s) associated with the MBS session according to network indicating the MBS session limited to an intended service area. It is up to UE implementation when to acquire the GNSS information of UE location.

And the following proposals are provided:

Proposal 1: It is proposed to include the intended service area for MBS services in the MBSBroadcastConfiguration message.

Proposal 2: An intended service area could be given an identity. The association of intended service area and a MBS session could be indicated by the MBSBroadcastConfiguration message.

Proposal 3: It is proposed that network could indicate whether a MBS broadcast service is only allowed to be received by UE located in the intended service area.

Proposal 4: According to network indication, a UE should not establish MRB(s) associated with a MBS session limited to an intended service area when it is not located within the area (up to UE implementation how the UE detects it’s not in the intended service area).

Proposal 5: According to network indication, a UE should release its established MRB(s) associated with a MBS session limited to an intended service area when exiting the area (up to UE implementation how the UE detects it’s not in the intended service area).

4	References

[1] RP-241667	Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN meeting #104, China, 2024

[2] R2-240xxxx	Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan, 2024.

[3] R2-2402826	Overview of MBS over NTN, Huawei, HiSilicon, Turkcell, RAN2#125bis, China, 2024.

[4] 3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".