3GPP TSG-RAN WG2#127											     R2-2406868

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.9.3

Source: 	NEC

Title:	        Consideration on UL capacity enhancement for IoT-NTN

Document for:	 Discussion and Decision

- Introduction

In the WID of IoT-NTN [1], reducing the necessary Uplink and Downlink signaling for EDT procedure can be discussed as below:

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support of Capacity enhancements for uplink - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In the last RAN2 meeting [2], some uplink related agreements were reached as below:

Agreements: - RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). - RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this) - If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

In this contribution, we will discuss the enhanced EDT procedure based on the last meeting’s progress and provide our views.

- Discussion

  - DSA and CRDSA

In the last meeting [2], whether DSA and CRDSA can be used for CB PUSCH transmission were discussed. In our understanding, when there are less UEs, since collision probability is very low, DSA and CRDSA are not needed. When there are more UEs, replica transmissions will only increase collision probability. Hence, in these cases, there is no benefit for enabling DSA and CRDSA. Whether DSA and CRDSA are needed is up to the current collision probability.

- Observation 1: Whether DSA and CRDSA are needed is up to the current collision probability.

If DSA and CRDSA are used, firstly, replica transmissions can occupy more resources. Secondly, since it is diversity slotted ALOHA, DSA and CRDSA may increase the latency of enhanced EDT procedure. Thirdly, when CRDSA is used, some interference cancelation techniques need to be further discussed in RAN1. And some corresponding scheme evaluations are also needed in order to select an efficient interference cancelation technique. Besides, we are not sure whether there are only small specification impacts, which needs to be further confirmed in RAN1.

- Observation 2: DSA and CRDSA may increase the latency of enhanced EDT procedure. And study and evaluation of some interference cancelation technologies need to be done in RAN1.

  - CB PUSCH resource

According to the last meeting agreement, Msg3 resource should be contention based PUSCH resource. Then for Msg3 resource configuration, the following two options may be considered:

- Option 1: CB PUSCH resource is configured by broadcast signaling similar to PRACH resource of EDT procedure.

- Option 2: CB PUSCH resource is configured by dedicated signaling, such as RRC release message similar to PUR procedure.

For option 1, since PUSCH resource is configured by system information, when the camped cell changes, enhanced EDT procedure is still used. But this option can increase collision probability due to common resource can be used for all UEs within the cell.

For option 2, the collision probability may be controlled by configuring shared UE number for a configured PUSCH resource by dedicated signaling. But compared with option 1, when the camping cell changes, dedicated PUSCH resource configuration will become invalid, especially in NGSO system. Some solving schemes are needed, for example, how to make dedicated PUSCH resource valid in cell other than the cell configured it.

For contention based Msg3 PUSCH resource configuration, the following two options may be considered:

- Option 1: CB PUSCH resource is configured by broadcast signaling

- Option 2: CB PUSCH resource is configured by dedicated signaling

For CB PUSCH resource, like in EDT, UE’s coverage levels should be supported. Besides, considering multi-carrier operation is supported in NB-IoT, PUSCH resource provided by anchor carrier and different non-anchor carriers should also be considered.

Introduce CB PUSCH resources for different coverage levels and different carriers.

UE selects among PUSCH resources provided by anchor or non-anchor carriers corresponding to the UE’s coverage level.

Another issue is whether one CB PUSCH resource associate to UE’s coverage level and a single carrier is mapping as one resource or multiple resources (e.g., a resource pool). In such cases, other than coverage level and carrier, finer granularity of resource configurations considering subcarrier level or OCC level may be needed.

RAN2 to discuss finer granularity of resource configurations considering subcarrier level or OCC level besides coverage level and carrier level.

  - Contention resolution

After the first UL message is transmitted, UE needs to determine whether the transmission using enhanced EDT is successful or not by the contention resolution during a configured time window. For legacy EDT procedure, RA-RNTI and preamble can identify a group of UEs in Msg2, and then contention resolution Id is used to further identify each UE. For PUR, a PUR-RNTI can be configured for each UE with PUR resource.

- RNTI: A new RNTI for enhanced EDT procedure may be introduced, and the following two options are considered

- Option 1: A pre-configured RNTI, it may be used for one UE or multiple UEs (further contention resolution is needed).

- Option 2: RNTI is calculated by UE.

- Contention resolution window: Contention resolution timer should be introduced and corresponding UE behaviors to start and stop the timer also need to be considered.

- Contention resolution ID: if calculated RNTI or RNTI shared by multiple UEs is used, contention resolution ID should be used for further contention resolution. Then, the current 48 bits contention resolution ID may be reused for enhanced EDT procedure.

Regarding RNTI, two options can be considered:

- Option 1: A pre-configured RNTI for UE using enhanced EDT

- Option 2: A new RNTI calculated by UE based on PUSCH resource

Introduce contention resolution timer to receive the DL message.

  - Efficient delivery of DL message

In PUR for CP CIoT optimisations, if there is no pending DL data or signalling, the NW may send L1 ACK containing TAC or L2 MAC containing TAC to the UE to update the TA and terminate the PUR procedure. If there is DL data, the NW can send the RRCEarlyDataComplete message to the UE. For UP CIoT optimisations, the NW can send the RRCConnectionRelease message to keep the UE in RRC_IDLE. Then for enhanced EDT procedure, in case of there is no pending DL data or signaling, if contention resolution ID is needed, L2 ACK may be used. If dedicated RNTI is used, contention resolution ID is not needed, then L1/L2 ACK may be used. For CP CIoT optimisations with pending DL data and UP optimizations, the NW sends RRCEarlyDataComplete or RRCConnectionRelease message to the UE.

For CP CIoT optimizations, when there is no pending DL data and signaling, L1/L2 ACK may be used for efficient delivery.

Besides, if calculated RNTI or RNTI shared by multiple UEs is used, DL messages from multiple UEs with the same RNTI may be multiplexed into one MAC PDU, such that DL message may be efficiently delivered.

  - Failure handling and fallback

Same as 2 or 4 step RACH procedure, we need to define the UE behavior If a UE failed to receive the corresponding contention resolution/response in the contention resolution window. One option is similar to RACH procedure that the UE is allowed to re-initiate CB Msg3 transmission for a configured number of times before declaring that the completely failure of CB Msg3 transmission. Another question is whether and how to do power control and power ramping for CB Msg3 transmission and retransmission.

It is fundamentally important that NW could control/spread out the load over the CB resources for msg3 transmission. E.g., as in RACH procedure, when the load is too high temporarily, and NW could ask UE to apply a backoff for coming up EDT over CB resources. We may introduce tool for NW to generally control a reasonable amount of UEs to use CB msg3 transmission and configure good amount of CB PUSCH resource for it. But that could not solve the temporary high load.

Like 2 step RACH could fall back to 4 step RACH, if CB EDT fails, it is preferable that UE can fall back to normal RACH-based EDT, but we need to discuss the condition and trigger of this fallback e.g., after maximum time of CB  EDT retransmission or after receiving the fallback command from NW.

RAN2 to discuss the following failure handling and fallback issues

- CB-Msg3 retransmission

- NW controlled backoff

- The condition/trigger for a UE to fallback from CB EDT to RACH-based EDT

- Conclusion

This contribution provides our analysis and views for the enhanced EDT, the following observations and proposals are listed below:

- Observation 1: Whether DSA and CRDSA are needed is up to the current collision probability.

- Observation 2: DSA and CRDSA may increase the latency of enhanced EDT procedure. And study and evaluation of some technologies need to be done in RAN1.

- For contention based Msg3 PUSCH resource configuration, the following two options may be considered:

- Option 1: CB PUSCH resource is configured by broadcast signaling

- Option 2: CB PUSCH resource is configured by dedicated signaling

Introduce CB PUSCH resources for different coverage levels and different carriers.

UE selects among PUSCH resources provided by anchor or non-anchor carriers corresponding to the UE’s coverage level.

RAN2 to discuss finer granularity of resource configurations considering subcarrier level or OCC level besides coverage level and carrier level.

Regarding RNTI, two options can be considered:

- Option 1: A pre-configured RNTI for UE using enhanced EDT

- Option 2: A new RNTI calculated by UE based on PUSCH resource

Introduce contention resolution timer to receive the DL message.

For CP CIoT optimizations, when there is no pending DL data and signaling, L1/L2 ACK may be used for efficient delivery.

RAN2 to discuss the following failure handling and fallback issues

- CB-Msg3 retransmission

- NW controlled backoff

- The condition/trigger for a UE to fallback from CB EDT to RACH-based EDT

- References

[1] RP-240776, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.

[2] Draft_R2_126_Meeting_Report_v1.