3GPP TSG-RAN WG2 Meeting #127	R2-2406872

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:		8.8.5

Source:		Lenovo

Title:		UE location verification in NTN regenerative architecture

Document for:		Discussion

- Introduction

The latest RAN2#126 meeting made the following agreements regarding support of UE location verification in NTN regenerative architecture:

- RAN2 assumes that network verified UE positioning is supported for regenerative architecture.

In this contribution we discuss some potential issues and optimizations for UE location verification when NTN regenerative payload is involved.

- Discussion

The UE location verification can be initiated by network using NI-LR procedures assisted by any 3GPP supported positioning method. For the network-based positioning methods, the LMF makes use of UE and/or BS reported measurement results (including angle, identity, time, and signal strength) for TRP(s) and the location(s) of the TRP(s) to calculate the UE location. In TN the TRP is assumed to be fixed on earth while in NTN the NGSO satellite as TRP is mobile, and such NTN characteristic brings in further challenges for UE location verification.

One of the obvious impacts is that NGSO satellite has limited coverage area and in-service duration, and as a result the positioning procedure could be interrupted when the satellite is involved in the positioning procedure as an TRP. To complete an initiated positioning procedure, it shall be guaranteed that UE can receive the positioning signal from the involved TRP(s). As shown in Figure 1 a1) and a2), for transparent architecture wherein TRPs involved in the positioning procedure may belong to the same gNB on the ground, the issue is not that obvious as the gNB can simply update the positioning measurement configuration of TRPs (e.g., removing the expired one and adding a new one) without severe interruption. However as shown in Figure 1 b1), for regenerative architecture wherein TRPs involved in the positioning procedure are different gNBs, the TRP expiration means gNB change and therefore the update of positioning measurement configuration requires inter-gNB interaction as well as longer interruption. For the worst case of single TRP operation in Figure 1 b2), the gNB change leads to mobility procedure (e.g., handover) and the positioning procedure could be severely delayed or completely terminated.

Observation 1: The impact on positioning procedure of TRP change is much worse in regenerative architecture compared to that in transparent architecture.

Figure 1. Impact of TRP change on positioning in NTN transparent and regenerative architectures

One possible direction to mitigate the impact is to preliminary inter-gNB information exchange to allow a quick reconfiguration of positioning measurement. However, this requires additional inter-gNB signalling overhead, more standard impact as well as precise estimation of gNB change, and it cannot solve the cases when earth-moving cell is involved (the gNB change timing is only available at UE) and when mobility procedure is triggered in single TRP operation (the gNB change inevitably leads to handover). On the other side, in NTN regenerative architecture the inter-gNB information exchange itself may also be restricted by inter-satellite connectivity e.g., ISL.

Observation 2: Preliminary inter-gNB information exchange is inefficient for reducing positioning procedure interruption due to gNB change in NTN regenerative architecture.

Another possible direction is to prevent the occurrence of interruption e.g., a positioning procedure will not be initiated if the time before gNB change is not enough to complete it. This can be easily implemented by UE reporting of the TRP availability information (e.g., gNB change timing) to the LMF via LPP or to the gNB via Uu. With such information the LMF may make better decision on initiating positioning procedure e.g., for UE location verification. The information may further help gNB in inter-gNB information exchange preparation and mobility decision to guarantee the completion of future positioning procedure.

Observation 3: UE reporting of gNB change (i.e., TRP expiration or addition) is helpful in preventing interruption of positioning procedure due to NGSO satellite capability restriction or movement.

Therefore, we think this is a useful optimization that can be considered in RAN2 at least for NTN regenerative architecture.

Proposal 1: For NTN regenerative architecture, RAN2 to discuss UE providing information of gNB change to LMF for reducing interruption or latency of UE location verification procedures.

- Conclusion

In this contribution we discuss potential issue and optimization for UE location verification in NTN regenerative architecture. The following observations are given:

Observation 1: The impact on positioning procedure of TRP change is much worse in regenerative architecture compared to that in transparent architecture.

Observation 2: Preliminary inter-gNB information exchange is inefficient for reducing positioning procedure interruption due to gNB change in NTN regenerative architecture.

Observation 3: UE reporting of gNB change (i.e., TRP expiration or addition) is helpful in preventing interruption of positioning procedure due to NGSO satellite capability restriction or movement.

And it is proposed that:

Proposal 1: For NTN regenerative architecture, RAN2 to discuss UE providing information of gNB change to LMF for reducing interruption or latency of UE location verification procedures.