3GPP TSG-RAN WG2 Meeting #127	R2-2406873

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:		8.8.6

Source:		Lenovo

Title:		NR satellite assistance information provisioning for LTE

Document for:		Discussion

- Introduction

The latest RAN2#126 meeting made the following agreements regarding LTE to NR-NTN IDLE mobility:

- For idle mode mobility from EUTRA TN to NR NTN, NB-IoT UEs are considered not in the scope. - For idle mode mobility from EUTRA TN to NR NTN, we don’t consider specific optimizations for BL UEs and UEs in CE. - SIB24 is reused to provide the NR NTN cell reselection related information (e.g. frequency information, SMTC config, etc.), introducing a satellite ID list in per frequency. The EUTRA cell provides the satellite assistance information for NR neighbor cell per satellite, as identified by the satellite ID. - To support the idle mode mobility from EUTRA TN to NR NTN, the satellite assistance information for NR NTN neighbor cells is needed and should include the following parameters: Satellite ephemeris information, TA common information, k-Mac, epoch time, validity duration, ntn-PolarizationDL (FFS if mandatory or optional) - The Ephemeris information/epoch time/k-mac/validity duration IEs defined in SIB33 specified in TS36.331 should be reused for NR satellite assistance information. - The signalling format for ntn-PolarizationDL and TA common related configurations within NTN-Config specified in TS38.331 should be introduced in TS36.331 for NR satellite assistance information. - RAN2 will decide in the next meeting which of the following options to adopt for the provision of the NR satellite assistance information (based on TPs provided by the WI RRC Rapporteur): - Option 1: Introduce a new SIB to include the NR satellite assistance information. - Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33. - Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information. - Introduce the clarification in the field description of measTimingConfig (configured via SIB24 in TS 36.331) that it is configured based on the assumption that the gNB-UE propagation delay equals to 0 ms, and UE can adjust the offset based on the actual propagation delay, when the corresponding frequency is associated with a satellite ID. Working Assumption: - NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell (can come back in the next meeting to see if the WA can be confirmed)

In this contribution we discuss the signalling details to support LTE to NR-NTN IDLE mobility.

- Discussion

For this objective it has been noted that the existing signaling introduced for IoT-NTN and mobility procedures between NR TN and NR-NTN shall be reused as much as possible. Following this principle, we provide further analysis and discussion on how to reuse the existing signaling.

For NR-NTN the necessary satellite assistance information of neighbour cells is provided in SIB19 as the structure allows to include both serving cell and neighbour cell information. For IoT-NTN things are different as the SIB31 only includes serving cell satellite assistance information while SIB33 is used for the neighbour cells. Meanwhile for TN to NTN mobility, it has been supported that an NR TN cell can broadcast SIB19 including only neighbour NTN cell satellite assistance information. The same principle can be simply leveraged by LTE TN, i.e., an LTE TN may broadcast SIB33 including neighbour NTN cell satellite assistance information. However, by reusing SIB33, another case of coexistence of IoT-NTN and NR-NTN may also need to be further considered, i.e., is it possible that an LTE TN cell needs to provide satellite assistance information for both IoT-NTN and NR-NTN neighbour cells? In our understanding the precondition of this question is that an LTE TN cell needs to provide satellite assistance information for IoT-NTN neighbour cells. As RAN2 have decided that NB-IoT UEs are considered not in the scope, this is related to the discussion that whether eMTC UEs are considered in scope.

Observation 1: If eMTC UE is in the scope of LTE TN to NR-NTN mobility, it is necessary to study to provide satellite assistance information for both IoT-NTN and NR-NTN neighbour cells in an LTE TN cell.

Proposal 1: RAN2 to decide whether eMTC UE is in the scope of LTE TN to NR-NTN mobility.

For providing NR satellite assistance information, the following 3 options were proposed in the latest meeting but no decision was made:

- Option 1: Introduce a new SIB to include the NR satellite assistance information.

- Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

- Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information.

In our understanding Option 1 has the most specification impact, and there seems no sufficient reason to introduce a new SIB especially considering Option 2 can do the work by adding new IE. Meanwhile it can be observed that from the structure perspective Option 3 has the least specification impact as no new SIB or IE will be introduced. However as we discussed above, if eMTC UE is in the scope of LTE TN to NR-NTN mobility, the case of providing satellite assistance information for both IoT-NTN and NR-NTN neighbour cells has to be considered. As a result, in Option 3 we have to study how to include both as well as how to differentiate them. Meanwhile the restriction of maximum 4 satellites is also to be considered. For Option 2 things go easier as different IE names can be used.

Observation 2: If providing satellite assistance information for both IoT-NTN and NR-NTN neighbour cells in an LTE TN cell is a valid case, Option 3 needs to further consider how to include both.

Proposal 2: For NR satellite assistance information provisioning, RAN2 adopts Option 2, i.e. define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

In the scenario of LTE TN to NR-NTN mobility, the serving cell is an LTE TN cell wherein signal-strength-based criteria are applied for neighbour cell measurement initiation. Although NTN introduced time/distance-based criteria as well but it is obvious that these time/distance-based criteria is not needed for a serving TN cell, regardless the serving cell is LTE or NR. For time/distance-based cell reselection which is not supported in NTN mobility, there is also no further motivation to have this in TN to NTN mobility.

Proposal 3: Confirm the working assumption: NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell.

- Conclusion

In this contribution we discuss the signalling details to support LTE to NR-NTN IDLE mobility. The following observations are given:

Observation 1: If eMTC UE is in the scope of LTE TN to NR-NTN mobility, it is necessary to study to provide satellite assistance information for both IoT-NTN and NR-NTN neighbour cells in an LTE TN cell.

Observation 2: If providing satellite assistance information for both IoT-NTN and NR-NTN neighbour cells in an LTE TN cell is a valid case, Option 3 needs to further consider how to include both.

And it is proposed that:

Proposal 1: RAN2 to decide whether eMTC UE is in the scope of LTE TN to NR-NTN mobility.

Proposal 2: For NR satellite assistance information provisioning, RAN2 adopts Option 2, i.e. define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

Proposal 3: Confirm the working assumption: NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell.