3GPP TSG-RAN WG2 Meeting #127	R2-2406874

Maastricht, Netherlands, Aug 19th – 23rd, 2024                Revision of R2-2404801

Agenda item:		8.9.2

Source:		Lenovo

Title:		Store and Forward support in IoT NTN

Document for:		Discussion

- Introduction

The latest RAN2#126 meeting made the following agreements regarding Store and Forward (S&F) operations in IoT NTN:

- For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite): - The UE sends uplink data signalling to eNB when service link is available and the eNB stores it. - When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN. - The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available). - The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again. - S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed. - RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN.

In this contribution we discuss the potential impacts on Uu to support S&F operations in IoT NTN.

- Discussion

S&F operation is an operation mode of a 5G system with satellite-access, where the 5G system can provide some level of service (in storing and forwarding the data) when satellite connectivity is intermittently/temporarily unavailable, e.g., to provide communication service for UEs under satellite coverage without a simultaneous active feeder link connection to the ground segment. Although the principles and some architectural details of S&F operation are still under SA2 discussion, RAN2 have also made some initial progress in the latest RAN2#126 meeting and is expected to discuss further on possible Uu impacts.

## Uu information exchange for S&F operation

Referring to SA2 works as in TR22.865, at least the following scenarios or use cases can be considered for S&F operations: MO, MT, data transfer for IoT devices in remote areas, and emergency report. The main different between S&F operation and normal operation in NTN is whether there is a continuous end-to-end connectivity path between the UE, the satellite and the ground network i.e., whether it requires the service and feeder links to be active simultaneously. Such differences have brought in some new requirements as consolidated by SA2:

- support S&F operation for authorized UEs;

- support to inform a UE whether S&F Satellite operation is applied;

- support to allow the operator or a trusted 3rd party to apply an S&F data retention period;

- support to allow the operator or a trusted 3rd party to apply an S&F data storage quota;

- support a mechanism to configure and provision specific required QoS and policies for UE’s data;

- support to provide related information (e.g. estimated delivery time to the authorised 3rd party) to an authorized UE.

- support to inform an authorised 3rd party whether S&F Satellite operation is applied for communication with a UE and to provide related information (e.g. estimated delivery time to the authorised UE).

- support forwarding of the stored data from one satellite to another satellite (e.g., which has an available feeder link to the ground network), through ISLs.

- support suitable means to resume communication between the satellite and the ground station once the feeder link becomes available.

- support mechanisms for a UE to register with the network when the network is in S&F Satellite operation.

- support mechanisms to authorize subscribers for receiving services when the network is in S&F Satellite operation.

Observation 1: At least from RAN2 perspective, solutions may also be developed to fulfil some requirements of S&F operation as consolidated by SA2.

According to the listed cases and requirements, the services that can leverage S&F operations have some specific characteristics including delay-tolerant (could be hours) and remote (source or target could be reached only be limited satellite coverage), and the UE may need some availability information about the S&F operation supported by the network, e.g., for which area or for how long the data can be delivered, or some functional information about the S&F operation supported by the network, e.g., support of providing delivery status, support of cast types for delivery (unicast, multicast and broadcast), support of eNB/gNB-only S&F (no CN entity is involved during the S&F operation).

Observation 2: For S&F operations, it may also be necessary to indicate UE some availability information about the S&F operation.

On the other hand, network may also need to be aware of UE requirements for S&F operation, e.g., to configure UE for S&F data transmission/reception or to understand for which area or for how long the data shall be delivered, or whether providing delivery status is needed, or which cast types for delivery is expected.

Observation 3: For S&F operations, the network may need to know UE’s service requirements for S&F operation.

When both the network and UE supports S&F operation, the UE needs to determine when an S&F operation service can be initiated e.g., whether a service can leverage S&F operation. From protocol design perspective, the AS-NAS functional spilt in UE for S&F shall be specified, e.g., which stratum is responsible for the initiation and the other stratum can provide assistances. Moreover, AS-NAS interaction could be needed to ensure appropriate S&F initiation and data delivery, e.g., start/suspend NAS->AS S&F data delivery when satellite coverage is valid/invalid to avoid congestion.

Observation 4: Metrics and interactions to initiate an S&F operation at UE may need to be specified.

Based on the above analysis, the following aspects can also be considered for RAN2’s study on supporting S&F operations:

Proposal 1: To support S&F operation and fulfill requirements from SA2’s work, RAN2 to study the following:

- how to indicate UE the availability information about the S&F operation (e.g., estimated delivery time);

- how to indicate network the UE’s service requirements for S&F operation;

- AS-NAS functional split and interactions to support S&F operation (may pending on SA2 future work).

- Conclusion

In this contribution we discuss the requirements of S&F and analyze the potential impacts on Uu to support S&F operations in IoT NTN. The following observations are given:

Observation 1: At least from RAN2 perspective, solutions may also be developed to fulfil some requirements of S&F operation as consolidated by SA2.

Observation 2: For S&F operations, it may also be necessary to indicate UE some availability information about the S&F operation.

Observation 3: For S&F operations, the network may need to know UE’s service requirements for S&F operation.

Observation 4: Metrics and interactions to initiate an S&F operation at UE may need to be specified.

And it is proposed that:

Proposal 1: To support S&F operation and fulfill requirements from SA2’s work, RAN2 to study the following:

- how to indicate UE the availability information about the S&F operation (e.g., estimated delivery time);

- how to indicate network the UE’s service requirements for S&F operation;

- AS-NAS functional split and interactions to support S&F operation (may pending on SA2 future work).