3GPP TSG-RAN WG2 #127	R2-2406876

Maarstricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	   8.1.2.2

Source:		Lenovo

Title:	LCM for UE-sided model in beam management

Document for:		Discussion and Decision

# Introduction

In this paper, we discuss issues related to LCM for UE sided model for beam management use case.

# Discussion

## 2.1	Applicability Determination

- Option 1: Joint decision by UE and NW In this option, UE-side additional conditions are known by UE internally, and NW-side additional conditions are known at network-side internally. First of all, UE decides UE-considered applicable functionalities based on UE-side additional conditions (e.g. UE speed, scenario, hardware capabilities, model availability, etc). UE reports UE-considered applicable functionalities to network. Network further checks its NW-side additional conditions by implementation and decides the final applicable functionalities by providing the corresponding configuration in Step 5. However, the UE may also need to include NW-side additional condition of the corresponding reported UE-considered applicable functionalities in Step 4. Example of Step 3-5 of Option 1 is shown as below: - Option 2: UE decision UE decides the applicable functionalities based on UE-side additional conditions (internally known by UE) and NW-side additional conditions. This option seems to be a special case for Option 1 where network sends NW-side additional conditions to UE in Step 3 in advance for applicable functionality determination. Example of Step 3-5 of Option 2 is shown as below:

In the RAN2 email discussion [POST126][032][AI/ML PHY] LCM (Intel/Samsung), two approaches for determining the applicability of a UE sided AIML functionality were discussed.

- Option 1: Joint decision by UE and NW

- Option 2: UE decision

Option 2 is aligned with the last time RAN2 agreement, thus can be taken as baseline.

RAN2#126 Agreement For BM use case, As a baseline the UE determines whether a functionality is applicable. Existing UAI framework is used at least for proactive reporting of applicable functionality. FFS reactive

In the meanwhile, we believe Option 1 is also possible and should be supported as well alongside Option 2. Because Option 2 presumes the UE is always aware of the NW side additional condition, while it is upon NW’s decision whether to share the current NW side additional condition to the UE.

In some scenario, if the current NW side additional condition is not provided to the UE, UE can still determine and report to the NW a list of AIML functionalities that are considered applicable from UE point of view and the associated NW-side additional condition under which the AIML functionality will become fully applicable. Then NW can further determine considering its current NW-side additional condition.

For applicability determination, RAN2 supports both (Option 1) joint UE&NW decision and (Option 2) UE decision, which option will be used depends on if the NW-side additional condition is provided by the NW, e.g., in OtherConfig.

It is quite common understanding that UAI based framework will be used for UE to proactively report the applicability related info according to the gNB configuration (i.e., in OtherConfig). Given that gNB might only be interested in specific UE-sided AI functionalities based on the demand of the gNB, it is reasonable to support the configuration of the UE by the gNB (i.e., via OtherConfig) to report applicability related info pertaining to a subset of the supported UE-side AI functionalities (i.e., as indicated in UECapabilityInformation).

gNB can configure the UE (i.e., via OtherConfig) to report applicability information pertaining to particular UE-side AI functionalities among those supported ones (i.e., as indicated in UECapabilityInformation).

Another aspect related to the UAI configuration for applicability related info report is the concept of prohibit timer. In the legacy, prohibit timer is usually configured to avoid frequent report of UAI. Then the question is whether the concept of prohibit timer is needed for the applicability related info report as well, e.g., after the UAI is triggered in the following two cases, should UE starts a prohibit timer in case:

- Case#1: UAI triggered due to a non-applicable UE sided AI functionality becoming applicable

- Case#2: UAI triggered due to an applicable UE sided AI functionality becoming now non-applicable

In case#1, after sending the UAI, the UE sided AI functionality could be activated by NW. Then it is quite critical to not put any restriction (e.g. prohibit timer) on the next UAI report for UE to timely notify gNB about the non-applicability at any time to deactivate the UE sided AI functionality before too late. In this sense, it would be better not to restrict the report of non-applicability using a prohibit timer.

In case#2, after sending the UAI, the UE sided AI functionality could be deactivated and UE may use another UE sided AI functionality or fallback to non-AIML method. A prohibit timer can be used in this case to avoid UE immediately sending a new UAI indicating the deactivated AI functionality now becomes applicable again, which may lead to frequent or unnecessary UE sided AI functionality activation/deactivation. And it does not harm if the UE continues using the non-AIML method for a while.

UE can send UAI indicating a UE sided AI functionality becomes non-applicable at any time without restriction (e.g., prohibit timer).

UE can be configured with a prohibit timer that after UE reports via UAI that a UE sided AI functionality becomes non-applicable, UE will not send another UAI message for the same UE-sided AI functionality again within a time period.

## AIML functionality activation/deactivation

In the last RAN2 meeting, it was agreed to support “network decision, network-initiated” scenario as a baseline for UE-sided model manangement. “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” can be further considered, while “UE-autonomous, UE’s decision is not reported to the network” is excluded.

Agreements:

1	For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline.  The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

2	“UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

### Network decision and Network-initiated

In the RAN2 email discussion [POST126][032][AI/ML PHY] LCM (Intel/Samsung), it is also discussed which signalling can be used by the NW to activate or deactivate the UE sided AI functionality. It can be via either

- RRC reconfiguration message, or

- L1/L2 activation/deactivation signalling

In our understanding, both approaches can be supported. When providing the inference configuration associated with a UE-sided AI functionality via RRC reconfiguration message, gNB can set an initial “activated” or “deactivated” state. After that, gNB can still rely on RRC reconfiguration message to further activate/deactivate the UE sided AI functionality. Alternatively, if the inference configuration of that UE-sided AI functionality has already been provided by gNB, gNB can also use MAC CE to activate/deactivate.

When providing the inference configuration to UE via RRC reconfiguration message, gNB can set the initial state of the AIML functionality to activated or deactivated.

For a UE-sided model with inference configuration provided, gNB can also use MAC CE to activate/deactivate a UE side AIML functionality and switch among AI functionalities in a more dynamic way.

Even if gNB wants to activate a UE-sided model, the UE may not follow the gNB instruction based on internal constraints such as available memory, processing capabilities, and energy consumption. For instance, a gNB might issue a command to activate an AIML functionality; however, the UE could decline if it lacks sufficient memory or processing power.

Moreover, it is also possible that when gNB decides to activate an UE-sided AI functionality according to a previous applicability report from UE, the same UE-sided AI functionality becomes non-applicable due to the change of UE side condition.

In those cases, UE may refuse to activate the UE sided AI functionality.

UE may refuse to comply the gNB decided activation of a UE-sided AI functionality due to the related UE sided AI functionality becoming non-applicable (e.g., change of UE side condition) or other UE factors (e.g., memory, energy consumption).

On the other hand, the UE is generally expected to comply with gNB directives regarding UE-sided model deactivation/fallback. We currently identify no compelling rationale for a UE to resist instructions to deactivate or fallback from a given functionality.

UE is expected to comply with gNB decided deactivation of a UE-sided AI functionality.

From another aspect, if the UE-sided functionality management is decided by gNB, it doesn’t matter whether UE will comply the gNB decision (upon selection/activation/deactivation/fallback), UE shall always reply and inform gNB about the UE compliance to avoid any misalignment.

RAN2 confirms that after receiving the activation/deactivation command from gNB, UE will always reply and inform gNB whether UE complies successfully, e.g., via RRCReconfigurationComplete message or a MAC CE depending on the activation signalling.

### 2.2.2	UE decided vs. NW decided deactivation/fallback

Another point we would like to raise is that because using AIML method will probably consume more computing resources and power, and it is the UE-sided model/functionality we are talking about, thus in principle UE should be always able to trigger the deactivate/fallback a AIML model/functionality considering internal constraints such as available memory, processing capabilities, and energy consumption.

Regarding which entity makes the final deactivation/fallback decision, we see two alternatives:

- Alternative 1, UE-decided: UE could make the deactivation/fallback by itself and sends the decision to gNB.

- Alternative 2, NW-decided: UE could send the deactivation/fallback preference to gNB, and it’s upon gNB to make the final decision. In this case, a reasonable gNB implementation is expected to follow the UE preference.

For deactivating one UE-sided functionality for AIML based BM, UE can always trigger it due to the related UE sided AI functionality becoming non-applicable (e.g., change of UE side condition) or other UE factors (e.g., memory, energy consumption).

RAN2 discusses whether:

  - The UE triggered functionality deactivation is decided by UE as well, and the decision is sent to gNB (i.e., UE autonomous, decision reported to gNB), or

  - The UE triggered functionality deactivation is sent as UE preference to gNB (e.g., via UAI), and the decision is made by gNB (i.e., UE initiated, gNB decision). gNB is expected to follow UE’s preference in this case.

## 2.3	Inference Result Reporting

RAN1#116bis Agreement For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. Agreement For UE-sided model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting - FFS on the details in the CSI-ReportConfig, at least considering: - Alt 1: one CSI-ResourceConfigId is configured for Set B - FFS: how UE can determine the information about set A - Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B - FFS: How to configure resource set(s) for Set A and Set B in CSI-ResourceConfig - Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately - Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId - FFS: how to configure/indicate separate resource set(s) for Set A - Note: separate CSI-ReportConfig for Set A and Set B are not precluded. - Note: Not perform measurement for Set A and only perform measurement for Set B subject to the CSI-ReportConfig - FFS on the association between Set A and Set B with or without additional IE - Other necessary configuration are not precluded.

In the last RAN1 meeting, it has been agreed to use CSI reporting framework for gNB to configure Set A and Set B beams, and for UE to report the spatial prediction result (AIML based BM case 1).

From RAN2 point view, RAN2 can simply confirm that the L1 measurement framework will be used for configuring/reporting the inference result of the UE side AI/ML model for beam management use case.

RAN2 confirms the L1 measurement framework will be used for configuring/reporting the inference result of the UE side functionality for AIML based BM.

# 3	Conclusion

Based on the discussion above, we observe:

No table of figures entries found.

Based on the discussion above, we propose:

Proposal 1	For applicability determination, RAN2 supports both (Option 1) joint UE&NW decision and (Option 2) UE decision, which option will be used depends on if the NW-side additional condition is provided by the NW, e.g., in OtherConfig.

Proposal 2	gNB can configure the UE (i.e., via OtherConfig) to report applicability information pertaining to particular UE-side AI functionalities among those supported ones (i.e., as indicated in UECapabilityInformation).

Proposal 3	UE can send UAI indicating a UE sided AI functionality becomes non-applicable at any time without restriction (e.g., prohibit timer).

Proposal 4	UE can be configured with a prohibit timer that after UE reports via UAI that a UE sided AI functionality becomes non-applicable, UE will not send another UAI message for the same UE-sided AI functionality again within a time period.

Proposal 5	When providing the inference configuration to UE via RRC reconfiguration message, gNB can set the initial state of the AIML functionality to activated or deactivated.

Proposal 6	For a UE-sided model with inference configuration provided, gNB can also use MAC CE to activate/deactivate a UE side AIML functionality and switch among AI functionalities in a more dynamic way.

Proposal 7	UE may refuse to comply the gNB decided activation of a UE-sided AI functionality due to the related UE sided AI functionality becoming non-applicable (e.g., change of UE side condition) or other UE factors (e.g., memory, energy consumption).

Proposal 8	UE is expected to comply with gNB decided deactivation of a UE-sided AI functionality.

Proposal 9	RAN2 confirms that after receiving the activation/deactivation command from gNB, UE will always reply and inform gNB whether UE complies successfully, e.g., via RRCReconfigurationComplete message or a MAC CE depending on the activation signalling.

Proposal 10	For deactivating one UE-sided functionality for AIML based BM, UE can always trigger it due to the related UE sided AI functionality becoming non-applicable (e.g., change of UE side condition) or other UE factors (e.g., memory, energy consumption).

Proposal 11	RAN2 discusses whether:

a.	The UE triggered functionality deactivation is decided by UE as well, and the decision is sent to gNB (i.e., UE autonomous, decision reported to gNB), or

b.	The UE triggered functionality deactivation is sent as UE preference to gNB (e.g., via UAI), and the decision is made by gNB (i.e., UE initiated, gNB decision). gNB is expected to follow UE’s preference in this case.

Proposal 12	RAN2 confirms the L1 measurement framework will be used for configuring/reporting the inference result of the UE side functionality for AIML based BM.

# Reference