3GPP TSG-RAN WG2 #127	R2-2406877

Maarstricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	   8.1.3

Source:		Lenovo

Title:	Data collection for NW-sided model training in beam management

Document for:		Discussion and Decision

# Introduction

In this paper, we discuss issues related to the training data collection for NW sided model for beam management use case.

# Discussion

## 2.1	MDT based framework for OAM-centric data collection

RAN2#126 Agreement Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

In the last RAN2 meeting, it was agreed that immediate MDT will be taken as the baseline framework for OAM centric data.

At the same time, there are two kinds of MDT framework, namely management-based MDT and signalling-based MDT. For management-based MDT, OAM configures the MDT configuration to gNB directly without pointing to a specific UE, and it is upon gNB to select the UE according to the configured criteria. Comparatively, for signalling-based MDT, the data collection is pointing to a specific UE via CN, and gNB configures the UE measurement report accordingly.

Comparing to signalling-based MDT, management-based MDT framework seems more suitable for training data collection among a group of UEs that fulfils certain criteria (e.g., cell or frequency or channel condition as discussed for the CSI/BM/Positioning use cases). In addition, management-based MDT is inherently anonymized meaning OAM does not need to understand the data is from which UE, as such it is user privacy friendly.

For management-based MDT, OAM configures the MDT configuration to gNB directly without pointing to a specific UE, and it is upon gNB to select the UE according to the configured criteria.

For signalling-based MDT, the data collection is pointing to a specific UE via CN, and gNB configures the UE measurement report accordingly.

Since management-based MDT and signaling-based MDT would have different implications and specification impacts to RAN3/SA2/SA5, it is suggested that RAN2 decides if management-based MDT or signaling-based MDT is assumed or whether it will be left for RAN3/SA2/SA5 to discuss this.

RAN2 is suggested to clarify if management-based or signaling-based MDT is assumed, or it will be left for RAN3/SA2/SA5 discussion. RAN2 considers LS coordination with RAN3/SA2/SA5.

## 2.2	Report logged L1 result in RRC message

RAN2#126 Agreement For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. FFS how to handle case when single RRC message is not sufficient. FFS if there will be any further enhancement needed pending RAN1 agreement. Enhance the immediate MDT framework to support periodical reporting. FFS whether and what event-based reporting is supported and FFS on network request reporting

In the last RAN2 meeting, it was also agreed to report logged L1 measurement results in RRC message from UE to gNB. In the legacy, for logged MDT, a UE variable VarLogMeasReport is used to log the UE measurements in RRC Idle/Inactive state. Similarly, for the logging of L1 measurement results in RRC connected state, we believe a new UE variable different than legacy VarLogMeasReport could be introduced considering the legacy VarLogMeasReport contains information dedicated for logged MDT and cannot be reused.

#### VarLogMeasReport The UE variable VarLogMeasReport includes the logged measurements information. VarLogMeasReport UE variable -- ASN1START -- TAG-VARLOGMEASREPORT-START VarLogMeasReport-r16 ::= SEQUENCE { absoluteTimeInfo-r16 AbsoluteTimeInfo-r16, traceReference-r16 TraceReference-r16, traceRecordingSessionRef-r16 OCTET STRING (SIZE (2)), tce-Id-r16 OCTET STRING (SIZE (1)), logMeasInfoList-r16 LogMeasInfoList-r16, sigLoggedMeasType-r17 ENUMERATED {true}, identityList-r18 CHOICE { plmn-IdentityList-r18 PLMN-IdentityList2-r16, snpn-ConfigIDList-r18 SNPN-ConfigIDList-r18 } } -- TAG-VARLOGMEASREPORT-STOP -- ASN1STOP

In legacy logged MDT, a VarLogMeasReport UE variable is used to log UE measurements in RRC Inactive/Idle state and containing information related to MDT.

RAN2 introduces a new UE variable (similar as VarLogMeasReport) for logging the L1 measurement results in RRC connected state. Then, when transmitting the logged L1 measurement results using RRC MeasurementReport message, each message will convey the logged L1 measurement results stored in the new UE variable.

In RAN1 reply LS R1-2310681, RAN1 listed the requirements for data collection of different purposes. We observe the following facts from RAN1 analysis:

- Among all use cases except inference for AIML based positioning, the required data collection latency is

  - Relaxed (e.g., minutes, hours, days, or no latency requirement) for AIML training,

  - Near-real-time (e.g., several tens of msecs to a few seconds) for AIML monitoring,

  - Time-critical (e.g., a few msecs) for AIML inference

- For AIML inference for AIML based beam management, RAN1 explicitly agreed that L1 report similar to legacy CSI can be used

One aspect we noticed related to the transmission of training data collection is that the training data collection for all use cases has relaxed latency requirement (e.g., minutes, hours, days, or no latency requirement). Therefore, the transmission of training data sent over air interface can be in the manner of best effort, e.g. low priority transmission.

Training data collection for all use cases has relaxed latency requirement (e.g., minutes, hours, days, or no latency requirement) and can be transmitted in best effort manner over air interface.

To facilitate transmissions in a best-effort manner, while simultaneously ensuring no interference with critical signaling transmissions (for instance, those related to RRC configuration), one intuitive approach involves utilizing a Radio Bearer that possesses a lower priority level than SRB1 for the conveyance of training data. This strategy mirrors existing practices observed in the transmission of Quality of Experience (QoE) reports, which are currently transmitted over SRB4 or SRB5—both of which are designated lower priority levels compared to SRB1.

Considering the relaxed latency requirement of training data, the transmission of logged L1 measurement results via RRC MeasurementReport message in the UL should be of lower priority than at least SRB1.

Furthermore, in the context of event-driven reporting for training data, such as logged L1 measurements, for AIML based BM, we believe RSRP threshold-based event should be considered. For example, a gNB may be interested to train an AIML model that is capable of beam prediction if UE is at the center of the serving cell, at the edge of the serving cell, or in between. Depending on the gNB’s interest, UE can be configured to start/stop data collection if the measured beam/cell level RSRP is below/above certain threshold.

RAN2 supports event triggered data collection/report, UE may start/stop L1 measurement result logging and reporting according to configured event, e.g., the measured RSRP is above/below a threshold.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	For management-based MDT, OAM configures the MDT configuration to gNB directly without pointing to a specific UE, and it is upon gNB to select the UE according to the configured criteria.

Observation 2	For signalling-based MDT, the data collection is pointing to a specific UE via CN, and gNB configures the UE measurement report accordingly.

Observation 3	In legacy logged MDT, a VarLogMeasReport UE variable is used to log UE measurements in RRC Inactive/Idle state and containing information related to MDT.

Observation 4	Training data collection for all use cases has relaxed latency requirement (e.g., minutes, hours, days, or no latency requirement) and can be transmitted in best effort manner over air interface.

Based on the discussion above, we propose:

Proposal 1	RAN2 is suggested to clarify if management-based or signaling-based MDT is assumed, or it will be left for RAN3/SA2/SA5 discussion. RAN2 considers LS coordination with RAN3/SA2/SA5.

Proposal 2	RAN2 introduces a new UE variable (similar as VarLogMeasReport) for logging the L1 measurement results in RRC connected state. Then, when transmitting the logged L1 measurement results using RRC MeasurementReport message, each message will convey the logged L1 measurement results stored in the new UE variable.

Proposal 3	Considering the relaxed latency requirement of training data, the transmission of logged L1 measurement results via RRC MeasurementReport message in the UL should be of lower priority than at least SRB1.

Proposal 4	RAN2 supports event triggered data collection/report, UE may start/stop L1 measurement result logging and reporting according to configured event, e.g., the measured RSRP is above/below a threshold.

# Reference