3GPP TSG-RAN WG2 #127	R2-2406878

Maarstricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	   8.1.3

Source:		Lenovo

Title:	Data collection for NW-sided model training in positioning

Document for:		Discussion and Decision

- Introduction

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

Among all positioning sub-use cases, (1st priority) Case 3a, (1st priority) Case 3b, (2nd priority) Case 2b are related to NW-sided AI model. In this paper, we discuss data collection issues for NW-sided AI model training for case 3a, 3b, and 2b.

- Discussion

2.1	Data collection for case 3a, 3b, 2b

(1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning.

RAN1#116bis Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

RAN1 has agreed that for case 3a, the label and its related data can be generated by at least LMF. Since it’s an AIML model deployed at gNB-side, RAN2 can assume the gNB-side model is either trained by the gNB itself or by the OAM. In that case, how does gNB retrieves the training data from LMF will be upon RAN3.

At this moment, it is unclear if the data generated by LMF is based on existing measurement/report from UE without LPP impact or there could be possible LPP impact as well. For example, if label comprises LOS/NLOS indication, there may be no further LPP impacts, however if the label also includes PRU SRS Tx Location, then possible LPP impacts are expected to retrieve/update the PRU location. RAN2 is thus suggested to wait for RAN1 progress before discussing the possible LPP impact.

For (case 3a) NG-RAN node assisted positioning with gNB-side model, RAN2 assumes gNB can retrieve the required training data from LMF via NRPPa with details upon RAN3.

RAN2 waits for RAN1 progress before discussing the possible LPP impact, if any, to support training data collection for case 3a.

(1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning. It is only for UL based direct positioning.

In the RAN1 last meeting, RAN1 assumes the training data (e.g., label and time stamp) for sub use case 3b can be generated by PRU or non-PRU UEs, and how the training data is transferred from PRU to NW is out of RAN1 scope.

From RAN2 point of view, since case 3b is related to LMF-sided AIML model, it seems natural for LMF to collect the required training data (e.g., label and time stamp) from PRU via LPP protocol.

RAN1#117 Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved.

For (case 3b) NG-RAN node assisted positioning with LMF-side model, RAN2 supports LMF collecting training data (e.g., label and time stamp) from PRU and non-PRU UE with estimated location via LPP protocol.

(2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning.

Although case 2b is considered as 2nd priority according to the WID, from our observation on RAN1 discussion, there is no artificial blocker to 2nd priority sub use cases. Actually, RAN1 is still trying to make progress on 2nd priority sub-use cases in parallel with other 1st priority sub use cases.

R1-2401546 Summary#3 AIML-positioning-v018_DCM_after_Thur_offline Guidance from RAN1 Vice Chairman at Tuesday online session On how to handle 1st and 2nd priority sub-cases, Xiaodong gave the following guidance verbally. The principle is that progress of 1st priority cases is not hindered by 2nd priority cases. RAN1 does not have any artificial blocker to 2nd priority sub-cases.

Working Assumption For training data generation of AI/ML based positioning Case 2a and 2b, the channel measurement and its related data (e.g., time stamp) are generated by PRU and/or non-PRU UE. Working Assumption For training data generation of AI/ML based positioning Case 2b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope.

From RAN2 point of view, to keep the same pace with RAN1, it is suggested to consider both 1st and 2nd priority sub use cases for AIML based positioning.

In RAN1 discussion, 2nd priority sub use case (e.g., case 2b) is under discussion in parallel with other 1st priority sub use cases.

RAN2 is suggested to consider both 1st and 2nd priority sub use cases for AIML based positioning.

For (case 2b) UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning, RAN1 has made assumption in their last meeting that the training data including channel measurements, label, and time stamp, can be generated by PRU or non-PRU UE with estimated location. It seems also straight forward for LMF to collect the required training data via LPP protocol.

For (case 2b) UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning, RAN2 supports LMF collecting training data (e.g., channel measurements, label and time stamp) from PRU and non-PRU UE via LPP protocol.

2.2	Data collection via LPP protocol

In terms of the principles for reporting of the training data from PRU/non-PRU UE to LMF, we believe similar principle compared to training data collection in AIML based BM could apply.

For instance, the PRU/non-PRU UE could log multiple instances of channel measurements associated with a ground truth label and a time stamp, then UE could report multiple logged instances in one LPP message to LMF. The LPP message conveying the collected training data could be transmitted with lower priority than other LPP messages and can be triggered periodically or event-based. W.r.t event-based report, we believe area-based event can be considered. For example, a LMF may be interested to train an AIML model that is capable of determine UE location if UE is within an area of interest (e.g., in a campus, or in a factory). Therefore, UE can be configured to start/stop data collection if the UE moves into or outside the area of interest.

With respect to the related LPP procedure, in our view, the existing location information transfer procedure based on RequestLocationInformation and ProvideLocationInformation LPP messages can be enhanced to serve the purpose.

For AIML based positioning with LMF-side model, the legacy location information transfer procedure (i.e., RequestLocationInformation and ProvideLocationInformation LPP messages) is enhanced to support the data collection.

RAN2 supports the logging of measured training data (e.g., channel measurements, label, time stamp) at PRU/non-PRU UE, and the logged multiple instances of measured training data is reported to LMF via LPP message as configured by LMF.

The LPP message conveying the measured training data can be reported periodically or event-based from PRU/non-PRU UE to LMF with lower priority than other LPP messages.

For AIML based positioning with LMF-side model, PRU/non-PRU UE may start/stop training data collection/report according to area-based event, e.g., the UE moves in or leaves an area.

3	Conclusion

Based on the discussion above, we observe:

Observation 1	In RAN1 discussion, 2nd priority sub use case (e.g., case 2b) is under discussion in parallel with other 1st priority sub use cases.

Based on the discussion above, we propose:

Proposal 1	For (case 3a) NG-RAN node assisted positioning with gNB-side model, RAN2 assumes gNB can retrieve the required training data from LMF via NRPPa with details upon RAN3.

Proposal 2	RAN2 waits for RAN1 progress before discussing the possible LPP impact, if any, to support training data collection for case 3a.

Proposal 3	For (case 3b) NG-RAN node assisted positioning with LMF-side model, RAN2 supports LMF collecting training data (e.g., label and time stamp) from PRU and non-PRU UE with estimated location via LPP protocol.

Proposal 4	RAN2 is suggested to consider both 1st and 2nd priority sub use cases for AIML based positioning.

Proposal 5	For (case 2b) UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning, RAN2 supports LMF collecting training data (e.g., channel measurements, label and time stamp) from PRU and non-PRU UE via LPP protocol.

Proposal 6	For AIML based positioning with LMF-side model, the legacy location information transfer procedure (i.e., RequestLocationInformation and ProvideLocationInformation LPP messages) is enhanced to support the data collection.

Proposal 7	RAN2 supports the logging of measured training data (e.g., channel measurements, label, time stamp) at PRU/non-PRU UE, and the logged multiple instances of measured training data is reported to LMF via LPP message as configured by LMF.

Proposal 8	The LPP message conveying the measured training data can be reported periodically or event-based from PRU/non-PRU UE to LMF with lower priority than other LPP messages.

Proposal 9	For AIML based positioning with LMF-side model, PRU/non-PRU UE may start/stop training data collection/report according to area-based event, e.g., the UE moves in or leaves an area.