3GPP TSG-RAN WG2 Meeting#127	R2-2406883

Maastricht, NL, 19 - 23 August 2024

Agenda item:	 8.10.2

Source:		Lenovo

Title:		Discussion on MRO for R18 mobility

Document for:		Discussion and Decision

1	Introduction

It was agreed in RAN2#125bis meeting that [1]:

For LTM MRO, RAN2 considers the following three connection failure cases:

- -	Too late LTM

- -	Too early LTM

- -	LTM to wrong cell

For too late LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure.

- -	Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell.

- -	Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell.

For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

- -	Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure.

- -	Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell.

LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

- -	Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure.

- -	Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

RAN2 considers SHR, RA report and RLF for MCG LTM SON.

RAN2 will start work on MCG LTM.

RAN2 to study failure and near failure scenarios for CHO with candidate SCGs.

It was further agreed in RAN2#126 meeting that [2]:

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD.

Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field.

Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

RAN2 will look into failure and near failure scenarios for subsequent execution of SCPAC and see if/what enhancements are needed.

In this paper, we would discuss the details of MRO for R18 mobility features, including LTM, CHO with candidate SCG(s) and subsequent CPAC.

2	Discussion

  - 2.1 MRO for LTM

We would start work with MCG LTM to mainly discuss RLF report, SHR and RA report for MCG LTM.

To reduce interruption time during PSCell change procedure, SCG LTM is introduced in R18 with same priority as MCG LTM. SCG LTM failure case and even near failure successful SCG LTM case should be avoided to improve network performance, we can further discuss MRO for SCG LTM when the discussion of MRO for MCG LTM is stable.

    - 2.1.1 MRO for connection failure in a LTM procedure

RAN2#126 meeting agreed to reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report for LTM failures. Currently, timeConnFailure IE is used to indicate the time elapsed since the last HO execution until connection failure, to adopt it for LTM, this IE can indicate the time elapsed between last LTM cell switch execution and connection failure, similar as legacy, reconnectCellId IE can be used to indicate the cell in which the UE comes back to connected after connection failure and after failing to perform reestablishment.

Proposal 1: Reuse the timeConnFailure IE to indicate the time elapsed between last LTM cell switch execution and connection failure.

RAN2#126 meeting also agreed to log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field. The existing choCellId IE is CHO specific, which indicates the candidate target cell for conditional handover included in condRRCReconfig that the UE selected for CHO based recovery while T311 is running. It seems that no existing IE in the RLF report can be reused for LTM recovery cell, it is better to introduce a new IE to indicate cell id of the LTM candidate cell in which the UE performs LTM failure recovery.

Proposal 2: Introduce a new IE to indicate cell id of the LTM candidate cell in which the UE performs LTM failure recovery.

    - 2.1.2 MRO for near failure in a LTM procedure

In legacy, SHR is introduced to detect sub-optimal successful handover. For one LTM cell switch execution in a MCG LTM procedure, it would be a near-failure successful LTM PCell switch. The UE may store near-failure successful MCG LTM related information to assist network to perform MRO analysis and optimisation. Since a LTM procedure is triggered based on L1 measurement report, RAN2#126 meeting agreed to log L1 measurements for serving cell, target cell and other LTM candidate cells if available in RLF report upon RLF or mobility failure, similar as failure case due to wrong selection of LTM candidate cell, it is beneficial for the UE to report L1 measurement results for source cell, target cell and other LTM candidate cells if available in SHR when the LTM cell switch execution is successful.

Proposal 3: It is beneficial for the UE to report L1 measurement results for source cell, target cell and other LTM candidate cells if available in SHR when LTM cell switch execution is successful.

In legacy, for analysis of such sub-optimal successful handover, the UE may collect SHR based on the triggers configured by the network, if received, and makes the SHR available to the network. For legacy SHR, T310/T312/T304 related trigger threshold is configured by network to trigger SHR storing or generating at UE side.

LTM is a procedure intended to reduce interruption time during mobility, that is why early TA acquisition procedure may be triggered by PDCCH order or realized through UE-based TA measurement. In the former case, the gNB/gNB-DU to which the LTM candidate cell belongs calculates the TA value and sends it to the gNB/gNB-DU to which the serving cell belongs via gNB-CU, the serving cell sends the TA value in the LTM cell switch command MAC CE when triggering LTM cell switch. In the latter case, the UE performs TA measurement for the LTM candidate cells after being configured by RRC, the UE applies the TA value measured by itself and performs RACH-less LTM upon receiving the cell switch command. If the valid TA value is provided in the cell switch command, the UE applies the TA value as instructed by the network. In the case where UE-based TA measurement is configured, but no valid TA value is provided in the cell switch command, the UE applies the valid TA value by itself if available. Meanwhile, the UE performs RACH-less LTM cell switch upon receiving the cell switch command. If no valid TA value is available, the UE performs RACH-based LTM cell switch. If RACH-less based LTM is not configured or is not initiated or failed, the interruption time will increase even LTM is successful. Therefore, it is necessary to store and report early TA acquisition related information or RACH based LTM related information in SHR to optimize LTM related configuration. To enable this, LTM specific trigger condition(s) for SHR needs to be considered, e.g. TA value acquired in early TA acquisition is invalid, TA value measured by UE is invalid, RACH-less LTM cell switch is not performed or performed but not successful.

Proposal 4: Introduce LTM specific trigger condition(s) for SHR, e.g. TA value acquired in early TA acquisition is invalid, TA value measured by UE is invalid, RACH-less LTM cell switch is not performed or performed but not successful.

    - 2.1.3 RA report for a LTM procedure

In a LTM procedure, depending on the availability of a valid TA value, the UE performs either a RACH-less LTM or RACH-based LTM cell switch:

- the UE performs RACH-less LTM cell switch upon receiving the cell switch command:

  - if the TA value is provided in the cell switch command, the UE applies the TA value as instructed by the network. For this case, before cell switch command, early TA acquisition/UL synchronization procedure is triggered by PDCCH order to an LTM candidate cell, or the UE performs RACH for early UL synchronization or RACH for early TA acquisition;

  - if UE-based TA measurement is configured, but no TA value is provided in the cell switch command, the UE applies the TA value by itself if available.

- if no valid TA value is available, the UE performs RACH-based LTM cell switch upon receiving the cell switch command:

  - the UE performs CFRA to the target cell, if contention-free Random Access Resources are provided in LTM Cell Switch Command MAC CE;

  - or, the UE performs CFRA to the target cell, if no contention-free Random Access Resource is provided in LTM Cell Switch Command MAC CE, but contention-free Random Access Resources are provided in the RRC Reconfiguration message for the LTM procedure;

  - or, the UE performs CBRA to the target cell, if no contention-free Random Access Resource is provided.

To optimize RA resources for LTM procedure, the UE may report L1 measurement results (of source cell, or LTM candidate cell, or target cell) during LTM procedure in the RA report. Additionally, for optimization of early UL synchronization or early TA acquisition, a new RA purpose to indicate that the random access procedure was triggered by a PDCCH order for early UL synchronization or early TA acquisition may be included in the RA report.

Proposal 5: The UE can report L1 measurement results during LTM procedure, and a new RA purpose to indicate that the random access procedure was triggered by a PDCCH order for early TA acquisition, in the RA report.

  - 2.2 MRO for CHO with candidate SCG(s)

As discussed in RAN3#124, it is FFS on whether to include failure and near failure scenarios related with configuration of CHO with candidate SCG(s) and CHO only. Since the cases related with combined configuration seems a bit complex, we would first consider the cases that the UE is only configured with configuration of CHO with candidate SCG(s), when it is stable, we can further discuss the cases related with combined configuration.

When only a configuration of CHO with candidate SCG(s) is configured to the UE, the UE does not execute PCell change with PSCell addition/change until the execution conditions for both the candidate PCell and the associated candidate PSCell are met. There may be following cases:

- case a: CHO execution condition of a candidate PCell is fulfilled but CPAC execution condition of the associated candidate PSCell is not fulfilled; or,

- case b: CHO execution condition of a candidate PCell is not fulfilled but CPAC execution condition of the associated candidate PSCell is fulfilled; or,

- case c: neither CHO execution condition of a candidate PCell is fulfilled nor CPAC execution condition of the associated candidate PSCell is fulfilled; or

- case d: CHO execution condition of a candidate PCell is fulfilled, and CPAC execution condition of the associated candidate PSCell is fulfilled.

    - 2.2.1 MRO for connection failure in a CHO with candidate SCG(s)

RAN2#126 meeting agreed to enhance the RLF report with additional information regarding the state of the two execution conditions and see later if can enhance the SCGFailureInformation report.

When an MCG failure (e.g. HOF or RLF) happens e.g. in above case a/b/c/d, to help network know whether/how to modify the configuration for the CHO with candidate SCG(s) procedure (e.g. optimize CHO execution condition, CPAC execution condition, list of CHO candidate PCell(s), or list of CPAC candidate PSCell(s)), the type of the first fulfilled execution condition if any (e.g. CHO execution condition or CPAC execution condition) can be stored or reported by the UE in RLF report, additionally, time duration between two fulfilled execution conditions if any (e.g. the time elapsed between CHO execution condition is fulfilled and the associated CPAC execution condition is fulfilled) can be stored and reported by the UE in RLF report.

Proposal 6: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) if any, and time duration between two fulfilled execution conditions if any, can be stored and reported by the UE in RLF report.

Similarly, an SCG failure may happen in above case a/b/c/d, for SCG failure happened in the CHO with candidate SCG(s) procedure, information regarding the state of the two execution conditions, e.g. the type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) if any, and time duration between two fulfilled execution conditions if any, can be stored and reported by the UE in the SCG Failure Information message.

Proposal 7: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) if any, and time duration between two fulfilled execution conditions if any, can be stored and reported by the UE in SCG Failure Information message.

    - 2.2.2 MRO for near failure in a CHO with candidate SCG(s)

In above case d, a near-failure successful PCell change and/or a near-failure successful PSCell addition or PSCell change may happen.

For legacy SHR or SPR, T310/T312/T304 related trigger threshold is configured by network to trigger SHR or SPR. It seems that there is no issue to reuse legacy SHR or SPR trigger threshold for the CHO with candidate SCG(s) procedure.

When CHO with candidate SCG(s) is performed, both CHO execution condition of the target PCell and corresponding CPAC execution condition of the associated target PSCell are fulfilled, there is a possibility that the time duration between the fulfilment of CHO execution condition and the fulfilment of CPAC execution condition is higher than a threshold, which may cause a near-failure successful CHO with candidate SCG(s). Therefore, besides legacy T310/T312/T304 related trigger threshold, RAN2 can consider new trigger condition(s) for SHR or SPR for the CHO with candidate SCG(s), e.g. a threshold related to time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell. When the time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell is higher than the threshold, SHR and SPR may be generated by the UE.

Proposal 8: Introduce new trigger condition(s) for SHR or SPR for a CHO with candidate SCG(s), e.g. a threshold related to time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell.

No matter what is the trigger condition(s) for SHR or SPR for a CHO with candidate SCG(s), for sub-optimal successful CHO execution, to help network know how to modify the configuration of the CHO with candidate SCG(s) procedure, R17 SHR can be enhanced, e.g. the type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) and the time duration between two fulfilled execution conditions (e.g. the time elapsed between CHO execution condition is fulfilled and the associated CPAC execution condition is fulfilled) can be stored and reported by the UE in SHR.

Proposal 9: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SHR.

Similarly, for any sub-optimal successful CPAC, R18 SPR can be enhanced, e.g. the type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) and the time duration between two fulfilled execution conditions (e.g. the time elapsed between CHO execution condition is fulfilled and the associated CPAC execution condition is fulfilled) can be stored and reported by the UE in SPR.

Proposal 10: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SPR.

  - 2.3 MRO for subsequent CPAC

In R18, intra-SN subsequent CPAC initiated by the SN, inter-SN subsequent CPAC initiated by either MN or SN are supported. Subsequent CPAC is only supported for NR-DC. The subsequent CPAC mechanism is defined as a conditional PSCell addition or change procedure that is executed after a PSCell addition, a PSCell change, a PCell change or an SCG release based on pre-configured subsequent CPAC configuration of candidate PSCell(s) without reconfiguration and re-initiation of CPC/CPA. The UE keeps the configured subsequent CPAC configuration (unless the network indicates to release it) and evaluates the execution conditions of candidate PSCells after completion of a PSCell addition, a PSCell change, a PCell change or an SCG release.

    - 2.3.1 MRO for connection failure in a subsequent CPAC procedure

The following failure may happen in initial CPAC execution phase or following subsequent CPC execution phase in a subsequent CPAC procedure:

- an SCG failure occurs before initial CPC execution or following subsequent CPC execution is executed;

- an SCG failure occurs during initial CPAC execution phase or following subsequent CPC execution phase, i.e. the UE fails to access the target PSCell of initial CPAC execution or following subsequent CPC execution;

- an SCG failure occurs shortly after successful initial CPAC execution or following subsequent CPC execution.

In legacy PSCell change or CPAC, SCG Failure Information message can be used by the UE to report SCG failure related information. Similarily, when a SCG failure happens in initial CPAC execution phase or following subsequent CPC execution phase, the SCG Failure Information message in Uu can be taken as the baseline. To enable network know whether the received SCG Failure Information message is for an initial CPAC execution or a following subsequent CPC execution, an indication concerning the SCG Failure Information message is for a initial CPAC execution or a following subsequent CPC execution may be included in the SCG Failure Information message‎.

Proposal 11: An indication concerning the SCG Failure Information message is for an initial CPAC execution or a following subsequent CPC execution may be included in the SCG Failure Information message.

    - 2.3.2 MRO for near failure in a subsequent CPAC procedure

In R18, SPR is introduced to detect sub-optimal successful PSCell addition or PSCell change or CPAC, RAN2 specified that the SPR is not sent immediately after a successful PSCell addition or PSCell change or CPAC, and only the latest successful PSCell addition or PSCell change or CPAC is stored or reported by the UE.

A near-failure successful CPAC execution may occur during initial CPAC execution phase or at least one following subsequent CPC execution phase. In legacy, SPR is introduced to detect sub-optimal successful PSCell addition or change, the SPR is not sent immediately after a successful PSCell addition or change, and only the latest successful PSCell addition or change is stored or reported by the UE. For subsequent CPAC, we should discuss whether SPR can be reused or whether to introduce a new successful report, especially considering that multiple CPAC executions are performed in a subsequent CPAC procedure. If SPR is reused, we should further discuss whether only the latest successful CPAC execution should be included in the SPR as legacy, or whether SPR should be enhanced to include multiple successful CPAC executions.

Proposal 12: RAN2 to discuss whether SPR can be reused for subsequent CPAC.

3	Conclusion

In this paper, the details of MRO for R18 mobility features are discussed. We have the following proposals:

MRO for LTM

Proposal 1: Reuse the timeConnFailure IE to indicate the time elapsed between last LTM cell switch execution and connection failure.

Proposal 2: Introduce a new IE to indicate cell id of the LTM candidate cell in which the UE performs LTM failure recovery.

Proposal 3: It is beneficial for the UE to report L1 measurement results for source cell, target cell and other LTM candidate cells if available in SHR when LTM cell switch execution is successful.

Proposal 4: Introduce LTM specific trigger condition(s) for SHR, e.g. TA value acquired in early TA acquisition is invalid, TA value measured by UE is invalid, RACH-less LTM cell switch is not performed or performed but not successful.

Proposal 5: The UE can report L1 measurement results during LTM procedure, and a new RA purpose to indicate that the random access procedure was triggered by a PDCCH order for early TA acquisition, in the RA report.

MRO for CHO with candidate SCG(s)

Proposal 6: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) if any, and time duration between two fulfilled execution conditions if any, can be stored and reported by the UE in RLF report.

Proposal 7: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) if any, and time duration between two fulfilled execution conditions if any, can be stored and reported by the UE in SCG Failure Information message.

Proposal 8: Introduce new trigger condition(s) for SHR or SPR for a CHO with candidate SCG(s), e.g. a threshold related to time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell.

Proposal 9: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SHR.

Proposal 10: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SPR.

MRO for subsequent CPAC

Proposal 11: An indication concerning the SCG Failure Information message is for an initial CPAC execution or a following subsequent CPC execution may be included in the SCG Failure Information message.

Proposal 12: RAN2 to discuss whether SPR can be reused for subsequent CPAC.

Reference

[1] Chairman notes of RAN2#125bis meeting

[2] Chairman notes of RAN2#126 meeting