3GPP TSG-RAN WG2 Meeting#127	R2-2406884

Maastricht, NL, 19 - 23 August 2024

Agenda item:	 8.10.5

Source:		Lenovo

Title:		Discussion on MRO for MR-DC SCG failure

Document for:		Discussion and Decision

1	Introduction

RAN2#126 meeting discussed the LS from RAN3 [1], and agreed that

Reply to RAN3 that we will only do EN-DC. RAN2 understands that whether also supporting (NG)EN-DC has no additional RAN2 impact hence RAN3 can decide. If later we get time we can consider other options.

Based on the achieved agreements, in this paper, we would further discuss the details of MRO for MR-DC SCG failure.

2	Discussion

Actually, MRO for SCG failure in NR-DC is specified in R17. Based on RAN2#126 meeting’s agreements, we would mainly focus on MRO for EN-DC scenario in R19.

When SCG failure occurs in EN-DC, SCGFailureInformationNR message may be used to report SCG failure related information. Currently, only failure type and measurement results are included in the SCGFailureInformationNR message in TS36.331. To support MRO for NR-DC in R17, the SCGFailureInformation message is enhanced to include previousPSCellId, failedPSCellId, timeSCGFailure, failure type and RA info. Similar as enhancements for SCGFailureInformation message for NR-DC SCG failure, to support MRO for SCG failure in EN-DC to optimize EN-DC related configuration , it is beneficial to enhance SCGFailureInformationNR message, e.g. to include previousPSCellId, failedPSCellId, timeSCGFailure and RA info.

Proposal 1: To support MRO for SCG failure in EN-DC, enhance SCGFailureInformationNR message to include previousPSCellId, failedPSCellId, timeSCGFailure and RA info.

For EN-DC scenario, since the SCG failure information reported by the UE is always encoded in the format of the MN RAT, but the SN is in the different RAT from the MN, if the MN just forwards the SCG failure information from the UE to the SN, it is impossible for the SN to decode it. To solve the issue, the MN decodes SCG failure information encoded in MN RAT, then re-organizes the information with SN RAT format, a new inter-node message may be introduced or the CG-ConfigInfo inter-node message may be reused to forward the re-organized SCG failure information from MN to SN.

Proposal 2: To support MRO for SCG failure in EN-DC, RAN2 to discuss whether to introduce a new inter-node message or reuse the CG-ConfigInfo inter-node message to forward SCG failure information from MN to SN.

3	Conclusion

In this paper, MRO for EN-DC SCG failure are discussed. We have the following proposals:

Proposal 1: To support MRO for SCG failure in EN-DC, enhance SCGFailureInformationNR message to include previousPSCellId, failedPSCellId, timeSCGFailure and RA info.

Proposal 2: To support MRO for SCG failure in EN-DC, RAN2 to discuss whether to introduce a new inter-node message or reuse the CG-ConfigInfo inter-node message to forward SCG failure information from MN to SN.

Reference

[1] R3-242195, LS on support of MRO for MR-DC SCG failure, ZTE