3GPP TSG-RAN WG2 Meeting #127					           	R2-2406889

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.5.2

Source: 	Lenovo

Title: 	Issues on the procedure of on-demand SSB SCell operation

Document for:	Discussion and decision

# Introduction

In RAN#102, a new work item “Enhancements of network energy savings for NR” was agreed [1]. The objectives of the work item are the following:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In RAN1#117, some agreements were achieved [2].

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell.

  - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell.

    - This DCI signaling does not provide SCell activation/deactivation.

    - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc.

  - FFS: Scenarios where the above signalings are applicable

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported.

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE

  - FFS: Whether the RRC is newly introduced or existing RRC is reused

In this contribution, we provide analysis and proposals on the 1st objective, i.e., procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA.

# Discussion

UE triggered on-demand SSB

Current NR supports activation/deactivation of an SCell based on traffic needs, for example different traffics being transmitted in different serving cells based on the different characteristics of the PHY layer e.g., PCell is deployed in a carrier frequency, and SCell is deployed in another carrier frequency This is beneficial in terms of energy saving in both UE side and gNB side. For UL, the UE can identify the traffic arrival prior to the gNB.

Observation 1: The preferred traffics can be different in PCell and SCell based on the gNB deployment, and the UE can identify the traffic arrival prior to the gNB.

At the same time, it is supported to configure limited number of SSBs to transmit from the spec. and implementation view of point. For example, the SSBs that provide good channel quality can be configured to the UE based on the UE measurement reports. It is beneficial to reduce power consumption comparing with the full set of SSBs transmission.

Observation 2: To reduce power consumption on SSB transmission, the gNB might transmit only limited number of SSBs in the SCell.

There could be a case that the channel quality measured from the configured SSBs changes to a low level. It may be identified by the gNB much later than by the UE because of the possible long-periodicity measurement report.

Therefore, it is beneficial if the UE could send a wake-up signal to request additional or updated SSBs transmission.

Observation 3: In case the channel condition of the SSB(s) is not good enough with the limited number of SSBs transmission, the UE can identify the issue prior to the gNB.

Proposal 1: UE triggered on-demand SSB operation is supported.

gNB triggered on-demand SSB

On the other hand, gNB triggered on-demand SSB SCell operation was agreed in RAN1.

Referring to RAN1 progress, it was agreed to provide at least Frequency of the on-demand SSB, SSB positions within an on-demand SSB burst by using signalling similar to ssb-PositionsInBurst and Periodicity of the on-demand SSB via higher layer RRC signalling. Since the existing RRC message can configure cell specific parameters of a UE's serving cell, e.g. when configuring a UE with a SCell or with an additional cell group (SCG), we can reuse the RRC message to configure the on-demand SSB related information.

Proposal 2: Reuse existing RRC message to configure the on-demand SSB related information to UE.

Considering the potential usages of On-demand SSB transmission listed in the objective, e.g. T/F synchronisation, SCell activation, one on-demand SSB configuration can work well for this purpose. Introduction of too much complexity shall be avoided.

Proposal 3: It is not necessary to configure more than one on-demand SSB configuration to avoid introducing of too much complexity.

# Conclusion

In summary, we have following observations and proposals for on-demand SSB for SCell in carrier aggregation:

Observation 1: The preferred traffics can be different in PCell and SCell based on the gNB deployment, and the UE can identify the traffic arrival prior to the gNB.

Observation 2: To reduce power consumption on SSB transmission, the gNB might transmit only limited number of SSBs in the SCell.

Observation 3: In case the channel condition of the SSB(s) is not good enough with the limited number of SSBs transmission, the UE can identify the issue prior to the gNB.

Proposal 1: UE triggered on-demand SSB operation is supported.

Proposal 2: Proposal 2: Reuse existing RRC message to configure the on-demand SSB related information to UE.

Proposal 3: It is not necessary to configure more than one on-demand SSB configuration to avoid introducing of too much complexity.

# References

- RP-234065, “New WID: Enhancements of network energy savings for NR”, RAN#102.

- RAN1#117 chairman note.