3GPP TSG-RAN WG2 Meeting #127						R2- 2406890

Maastricht, The Netherlands, 19-23 August 2024

Agenda Item:	8.5.4

Source: 	Lenovo

Title:  	Paging statistics from field and PRACH adaptation

Document for:	Discussion and decision

# Introduction

In RAN2#125bis (Changsha), paging adaptation was discussed, and following agreements were achieved:

Agreements on paging adaptation: 1. From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle. 2. For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF. 3. For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE: - Option 1: Prevent the access of legacy UE via barring; - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

Companies have already proposed bundling of Paging Frames in the last meeting, even keeping the total number of Paging occasions same. This was also studied and captured in the TR 38.864.

This paper intends to check if it is possible to achieve better energy saving with respect to paging transmission. Also, PRACH adaptation related issues are discussed in this paper.

# Discussion

Some logs from different cities and operators were collected on Motorola/ Lenovo phones and used to analyse network configuration for paging and related statistics.

## Field Logs from some cities in India (Bangalore), China (Wuhan), Brazil (Jaguariuna) and USA (Chicago)

Table 1: Field logs from Bangalore, India

Table 2: Field logs from Operator-1, Brazil

Table 3: Field logs from Operator-2, Brazil

Table 4: Field logs from Operator-1, Chicago

Table 5: Field logs from Operator-2, Chicago

Table 6: Field logs from Wuhan, China

From the field logs some clear observations can be made:

Observation#1a: The capacity utilization (i.e., the total number of RRC Paging records that could be sent with the used paging configuration) is much less than 1% in all observed cases.

Observation#1b: Average number of paging records contained in one RRC message (when Paging is sent) is close to 2. The real average accounting all configured paging occasions will be much less.

Observation#1c: Maximum number of records included in the same Paging message is much less than 32 (max is 9 as seen in Brazil and 8 in China).

Looking at the above observations coming from fields logs we think that the current 3GPP Paging design is highly over dimensioned. Going forward, to save energy we would propose to use a simpler and leaner Paging scheme balancing network energy consumption, false alarm rate and paging latency.

Proposal 1: NES work should aim a paging scheme that is closer to the real field demand i.e., not overdesigned (for hypothetically high capacity).

In the below we propose such a scheme.

## Paging enhancement

In a non-multibeam scenario 4 out of 10 subframes per PF can be used for paging that is there can be at most 4 PO per PF. A paging message can only be sent in a PO. The paging message can at most include 32 paging records in the paging message where each paging record includes the UE identities of the UEs being paged. The theoretical paging capacity as maximum number of UEs paged per second in an NR non-multibeam cell is thus limited by:

Supported Paging Capacity per second=NPF×NPOperPF×NUEperPO.

The parameters are defined as follows:

-	Paging Frames (PF) per second: NPF

-	Paging Occasions (PO) per PF: NPOperPF

-	Maximum number of paging records in paging message: NUEperPO

The current paging transmission system in 5G NR is over-dimensioned and even if only a few UEs need to be transmitted, it may have to wake up and transmit paging often if the UEs to be paged are listening to paging at different time occasions, calculated as in 3GPP TS 38.304.

Many companies have talked about bundling the Paging frames and Paging occasions such that the total number of paging occasions are kept the same. Taking an example from TR 38.864: Figure 1Error! Reference source not found. illustrates an example paging frame and occasion when N is configured to T/4, and Ns is configured to 4. When N is set to T/4, gNB potentially may need to transmit paging every 4 frames.

Figure 1: Legacy Paging frame and paging occasion allocation when N = T/4 and Ns = 4

It might be possible to support the same paging transmission density by squeezing the POs to consecutive slots or frames, such that gNB does not necessarily need to wake up frequently. Figure 2 illustrates some examples of potential enhancement that could be made for paging.

Figure 2: Shifting PFs/ POs into consecutive frames or slots while maintaining the same paging occasion density

The above scheme is the basis of energy saving and the 3GPP TR 38.864 based on simulation results concludes:

One source observed that for BS category 1 and at empty load case, statically adapting paging configuration could provide BS energy savings by 0.2%~6.7% when paging load (resource used for paging) is 0.2%~0.5%, and the gain can be up to 42.3% when paging load is increased up to 3.6%. The gain could also increase as the number of SSB increases. Performance of dynamically adapting paging configurations is not provided. The above energy saving gains were achieved with SSB periodicity of 80ms or 160ms.

The enhancements proposed so far keep the number of Total POs same as today. This results in same number of RRC Paging messages (PDSCH) and as field statistics show there is room to pack the 32 paging records in the paging message in better way to minimize PDSCH transmission. For example, instead of 3 Paging messages with 2 paging records each, one paging message with 6 records saves PDSCH transmission. Same applies to Paging DCI transmissions. So, we think just as a start RAN2 can aim to have just one Paging frame containing a single paging occasion (PO) in each DRX Cycle, defined by the network in cell specific manner. The calculation of “the single PO” may not depend on UE_ID.

A single PO per DRX Cycle will “merge” both the options on the table. For this N = length of the Paging Cycle (which is the extension of the current value of ‘N’ beyond 64, 128…) and Ns = 1. The single PO per DRX Cycle is also bundling of PFs to the maximum extent – where not only PFs are bundled to 1 but also POs are bundled to be just 1. Companies are also encouraged to verify the real situation on field and correct/ verify the understanding from Observations 1a, 1b and 1c. Below we proposed just one fixed PO per Paging DRX Cycle:

Proposal 2: RAN2 consider a simple solution for R19 where only one fixed PO is used in a cell specific Paging DRX Cycle.

As legacy UEs can’t know about the R19 changes, the legacy paging occasions (PF+POs) are kept as it is and since their number will dwindle with time, the required paging for such UEs will go down further.

Proposal 3: For legacy UEs the legacy paging mechanism can be continued.

## Clarification on the objective

It is captured in TR38.864 that Paging on a NES cell without SIB or a non-anchor NES cell without SSB and SIB is not supported.

Referring to the WID and RAN1 progress, the scenario that can apply paging enhancement is not clear. Can paging enhancement be applied in the case of NES operation without SIB1/SSB? We think the paging enhancement discussed in this agenda item is limited to the paging enhancement of a cell with legacy SIB1 and SSB transmission. As to the paging enhancement for a cell without SIB1 and/or SSB, it shall be discussed in other agenda items e.g., AI8.5.2, AI 8.5.3.

Proposal 4: RAN2 to clarify the paging enhancement is only applied in a cell with legacy/ regular SIB1 and SSB transmission, i.e., a cell without needing an Anchor cell concept.

The discussion on adaptation of common signal/channel transmissions includes adaptation of SSB, PRACH and paging occasions in time domain.

Referring to RAN1 progress on adaption of SSB, it is supported for a cell to serve both legacy UEs and Rel-19 NES-capable UEs. Therefore, we can consider the legacy UE can camp and access a cell applying Rel-19 NES techniques. Then when paging adaptation is also applied in the cell, the legacy UE shall not be prevented to access the cell via cell barring indication. To avoid the potential conflicts, option 1 (Prevent the access of legacy UE via barring) for paging adaptation can be excluded.

Observation 5: Referring to RAN1 progress, option 1 (Prevent the access of legacy UE via barring) may be excluded to support common channel adaptation e.g. adaptation of SSB, PRACH and paging occasions applied in one cell.

Proposal 5: To avoid the impact on legacy UE, Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs) is selected.

## RACH enhancement

In RAN1#116bis discussion on the RACH adaptation for Rel-19 took place with the following agreements:

Agreement For adaptation of PRACH in time-domain, support at least the following: - Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - Configuration of additional PRACH resources is provided by semi-static signalling - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs - FFS: adaptation mechanism for additional PRACH resources - Note: No change to the existing PRACH configuration tables in 38.211 Agreement For adaptation of PRACH in time-domain, support the following: - SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any) - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources - FFS: SSB-RO mapping for the additional PRACH resources

According to RAN1 agreement, the configuration of additional PRACH resources is provided by semi-static signaling. From point view of RAN2 specification, the configuration of additional PRACH resources is separately configured from the legacy RACH configuration in SIB 1, wherein the configuration of additional PRACH resources needs to indicate at least the PRACH configuration index, Number of SSBs per RACH occasion. The other config(s) e.g., msg1-FDM, msg1-FrequencyStart, may be also separately configured from the legacy RACH configuration for flexibility. If the other config(s) are not separately configured, UE default reuses the config in legacy RACH configuration.

Proposal 6: RAN2 to agree the additional RACH occasion for adaptation in time domain is separately configured in RACH configuration from legacy RACH configuration in SIB1.

Proposal 7: RAN2 to agree the separate RACH configuration indicates at least RA config Index and Number of SSBs per RACH occasion, the other config(s) e.g., msg1-FDM, msg1-FrequencyStart, may be also separately configured. If the other config(s) are not separately configured, UE default reuses the config in legacy RACH configuration.

# Conclusion

In summary, we have following observations and proposals for adaptation of common signal/channel transmissions:

Observation#1a: The capacity utilization (i.e., the total number of RRC Paging records that could be sent with the used paging configuration) is much less than 1% in all observed cases.

Observation#1b: Average number of paging records contained in one RRC message (when Paging is sent) is close to 2. The real average accounting all configured paging occasions will be much less.

Observation#1c: Maximum number of records included in the same Paging message is much less than 32 (max is 9 as seen in Brazil and 8 in China).

Proposal 1: NES work should aim a paging scheme that is closer to the real field demand i.e., not overdesigned (for hypothetically high capacity).

Proposal 2: RAN2 consider a simple solution for R19 where only one fixed PO is used in a cell specific Paging DRX Cycle.

Proposal 3: For legacy UEs the legacy paging mechanism can be continued.

Proposal 4: RAN2 to clarify the paging enhancement is only applied in a cell with legacy/ regular SIB1 and SSB transmission, i.e., a cell without needing an Anchor cell concept.

Observation 5: Referring to RAN1 progress, option 1 (Prevent the access of legacy UE via barring) may be excluded to support common channel adaptation e.g. adaptation of SSB, PRACH and paging occasions applied in one cell.

Proposal 5: To avoid the impact on legacy UE, Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs) is selected.

Proposal 6: RAN2 to agree the additional RACH occasion for adaptation in time domain is separately configured in RACH configuration from legacy RACH configuration in SIB1.

Proposal 7: RAN2 to agree the separate RACH configuration indicates at least RA config Index and Number of SSBs per RACH occasion, the other config(s) e.g., msg1-FDM, msg1-FrequencyStart, may be also separately configured. If the other config(s) are not separately configured, UE default reuses the config in legacy RACH configuration.

# References

[1] RP-234065, “New WID: Enhancements of network energy savings for NR”, RAN#102.

[2] RAN2#125bis chairman notes.

[3] RAN2#126 chairman notes.