3GPP TSG-RAN WG2 Meeting #127	R2-2406891

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda item:	7.5.3

Source:		Lenovo, CMCC

Title:			Update RX_NEXT based on PDCP SN gap report

Document for:	Discussion and Decision

- Introduction

This contribution discusses how the state variable RX_NEXT should be handled based on the PDCP SN gap report.

- Discussion

PDCP receiver updates RX_NEXT to the largest COUNT value associated with the discard PDCP SDU plus 1 if RX_NEXT <= COUNT value associated with the last PDCP SDU indicated in the PDCP SN gap report in TS 38.323:

- else if RX_NEXT <= COUNT value associated with the last PDCP SDU indicated in the PDCP SN gap report: - update RX_NEXT to the largest COUNT value associated with the discarded PDCP SDU plus 1.

As analyzed in [1], if there is some expected high importance PDCP SDU with COUNT value <= the largest COUNT value associated with the discarded PDCP SDU, this current updating behavior for the RX_NEXT would cause the t-reordering timer to run. If the PDCP SDU with COUNT value <= the largest COUNT value associated with the discarded PDCP SDU is received after the t-reordering timer expires, it will be abandoned. The risk of prematurely abandoning the expected PDCP SDU has been discussed last RAN2 meeting. In Rel-18, if the PDCP PDU has been delivered to lower layer, it would continue the transmission even the remaining time becomes zero, we consider the later arriving data, especially for the high importance PSI, is deserved to delivery to upper layer. Therefore, we consider this as some issue and think that the concerned case should be addressed.

In our view there could be 2 options considered:

- Option 1. update RX_NEXT to the COUNT value of the first PDCP SDU which is not considered as discarded and expected to be received, with COUNT value > RX_NEXT if RX_NEXT is equal to any COUNT value associated with the discarded PDCP SDUs.

For example, in figure 1, current RX_DELIV is equal to 11 and current RX_NEXT is equal to 13, UE has received PDCP SDUs in green boxes before receiving PDCP SN gap report. PDCP entity updates after having received the PDCP SN gap report RX_NEXT (13) to the COUNT value (14) of the first PDCP SDU which is not considered as discarded, with COUNT value > RX_NEXT if RX_NEXT is equal to any COUNT value associated with the discarded PDCP SDUs (as indicated in the SN gap report). Thus, RX_NEXT is still equal to 14, it is equal to RX_DELIV (14), the t-reordering timer does start since RX_DELIV (14) = RX_NEXT (14). Therefore, the potential issue can be avoided.

Figure 1. update RX_DELIV and RX_NEXT after receiving PDCP SN gap report

- Option 2. Keep RX_NEXT unchanged if receiving PDCP SN gap report.

In legacy R15, PDCP receiver only updates RX_NEXT based on the COUNT of received PDCP SDU with COUNT value larger than or equal to current RX_NEXT. If t-Reordering is not running and RX_DELIV < RX_NEXT, PDCP receiver starts t-reordering timer as following in TS 38.323:

- if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT: - update RX_REORD to RX_NEXT; - start t-Reordering.

If RX_NEXT is only updated till receiving a PDCP SDU with count larger than or equal to current RX_NEXT, i.e., as legacy, there is no risk in prematurely abandoning expected PDCP SDUs. In figure 2, if the RX_NEXT (13) is equal to a COUNT value (13) of a discarded PDCP SDU indicated in the PDCP SN gap report, the current RX_DELIV is updated to the COUNT value (14) of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded, with COUNT value > RX_DELIV, leave the RX_NEXT as it was. Thus, the t-reordering timer does start since RX_DELIV (14) > RX_NEXT (13). Therefore, the potential issue can be avoided. However, it can happen that RX_NEXT is less than RX_DELIV. If needed, RAN2 can discuss whether RX_NEXT is updated to RX_DELIV if current RX_NEXT is less than the current RX_DELIV updated based PDCP SN gap report.

Figure 2. update RX_DELIV and not update RX_NEXT after receiving PDCP SN gap report

Both options can work, we slightly prefer the option 1 since Option 1 is more reasonable.

Observation 1. If updating RX_NEXT to the largest COUNT value associated with the discarded PDCP SDU plus 1, it has risk of abandoning some PDCP SDU with SN below the largest count, which is expected to receive.

Observation 2. If RX_NEXT is only updated till receiving a PDCP SDU with count larger than or equal to current RX_NEXT, i.e., as legacy, it does not bring the risking, however it can happen that the RX_NEXT is less than the RX_DELIV.

Proposal 1:  PDCP entity updates RX_NEXT to the COUNT value of the first PDCP SDU which is not considered as discarded and expected to be received, with COUNT value > RX_NEXT if RX_NEXT is equal to any COUNT value associated with the discarded PDCP SDUs.

- Conclusion

In this contribution, we discuss RX_NEXT handling based on PDCP SN gap report. We have the following proposals:

Observation 1. If updating RX_NEXT to the largest COUNT value associated with the discarded PDCP SDU plus 1, it has risk of abandoning some PDCP SDU with SN below the largest count, which is expected to receive.

Observation 2. If RX_NEXT is only updated till receiving a PDCP SDU with count larger than or equal to current RX_NEXT, i.e., as legacy, it does not bring the risking, however it can happen that the RX_NEXT is less than the RX_DELIV.

Proposal 1:  PDCP entity updates RX_NEXT to the COUNT value of the first PDCP SDU which is not considered as discarded and expected to be received, with COUNT value > RX_NEXT if RX_NEXT is equal to any COUNT value associated with the discarded PDCP SDUs.

- References

### R2-2404287, PDCP SN gap report and updating RX_NEXT, Nokia, Nokia Shanghai Bell.