3GPP TSG-RAN WG2 Meeting #127						R2-2406892

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	 8.7.1.1

Source:	 Lenovo

Title:	 Discussion on RAN2 Replies to LS

Document for:	Discussion and Decision

- Introduction

This paper discusses RAN2 replies to RAN3 LS on UL PSI based PDU discarding in NR-DC (R3-243957) and SA2 LS on FS_XRM Ph2 (S2-2407351).

- Discussion

  - Replies to RAN3 LS on UL PSI based PDU discarding in NR-DC

For UL PSI based PDCP discard, as per RAN2’s agreement, gNB can configure two PDCP discard timers to UE: legacy PDCP discardTimer and discardTimerForLowImportance for SDUs of low importance.  And gNB can explicitly activate/deactivate PSI based discarding by means of an activation/deactivation MAC CE.

For NR-DC split bearer, the DRB is served by both MCG and SCG, and each has its own MAC entity. It is possible both MN and SN can send the PSI based Discard Activation/Deactivation MAC CE of the same DRB to the UE. As currently specified by MAC and PDCP specs, when UE receives the Activation or Deactivation indication from either MN or SN or both, UE shall follow the command and activate or deactivate the PSI based discard.

TS 38.321 5.18.34 Activation/deactivation of PSI-based SDU discard The network activates and deactivates PSI-based SDU discard by sending the PSI-Based SDU Discard Activation/Deactivation MAC CE described in clause 6.1.3.73. The PSI-based SDU discard is initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync. The MAC entity shall for each DRB configured with discardTimerForLowImportance: 1> if a PSI-Based SDU Discard Activation/Deactivation MAC CE is received activating the PSI-based SDU discard for the DRB: 2> indicate the activation of the PSI-based SDU discard for the DRB to upper-layers. 1> if a PSI-Based SDU Discard Activation/Deactivation MAC CE is received deactivating the PSI-based SDU discard for the DRB: 2> indicate the deactivation of the PSI-based SDU discard for the DRB to upper-layers.

TS 38.323 5.2.1 Transmit operation At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall: - if discardTimerForLowImportance is configured and PSI based SDU discard is activated, and the PDCP SDU belongs to a low importance PDU Set: - start the discardTimerForLowImportance associated with this PDCP SDU; - else: - start the discardTimer associated with this PDCP SDU (if configured).

- Observation 1: As currently specified by MAC and PDCP specs, when UE receives an Activation or Deactivation indication from either MCG or SCG or both, UE shall activate or deactivate the PSI based discard.

- In some cases, the UE may receive contradicting instructions of Activation/Deactivation of the PSI based discarding from MCG and SCG if there is no coordination between MN and SN. In [3], it was proposed that when UE receives the Activation indication from both MN and SN, the UE shall activate the PSI based discard. The main intention of the proposal is that if only one node/CG associated with a UL split-bearer is congested, UE still can use the other node/CG without problem. It should be noted, that such intended behaviour would require a specification change.

- We think that we can leave the issue to the network’s solutions or implementations, e.g.  network only sends the activation indication in case both MCG and SCG are congested. there would be several network based solutions that should be discussed by RAN3:

-	Solution 1: Only PDCP terminated node can send the MAC CE. This solution has strong restriction on NW implementation.

-	Solution 2: Once one node sends the MAC CE to UE, the node informs the other node about the information.

-	Solution 3. NW determines one node which can send the MAC CE to UE. When the peer node wants to deactivate/activate the PSI based discarding, the peer node sends the request to the node.

RAN2 replies to RAN3 that as currently specified by MAC and PDCP specs, when UE receives an Activation or Deactivation indication from either MCG or SCG, UE shall follow the command and activate or deactivate PSI based discarding. RAN2 thinks network based solutions or implementations should be used for avoiding contradicting instructions of Activation/Deactivation of the PSI based discarding from MN and SN.

  - Replies to SA2 LS on FS_XRM Ph2

- Question1 [for SA4, RAN2 and RAN3]:

  - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

- Question2 [for SA4 and RAN2]: There is some discussion about time to next burst.

  - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6?

  - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

- Currently, for DL 5GC may provide XR traffic assistance information to gNB through NG AP TSC Assistance Information (TSCAI). The XR traffic assistance information includes

-	UL and/or DL Periodicity;

-	N6 Jitter Information (i.e. between UPF and Data Network) associated with the DL Periodicity.

- For UL, uplink assistance information (jitter range, burst arrival time, UL data burst periodicity) per QoS flow is sent by the UE to gNB via UE Assistance Information.

- Based on above assistance information, gNB can know the periodicity and jitter range for data burst for both UL and DL data transmission. If any update in the assistance information, gNB can get the updated information by signalling i.e., 5GC can sent updated information by NG-AP signing and UE can report the updated information by UE Assistance Information. It is under assumption that the XR traffic assistance information does not change very frequently.

- In Release 18, RAN2 had discussed the inter-burst time according to the incoming LS in [4]. In the incoming LS, it is stated as:

In addition to marking the last PDU of the data burst, SA4 sees the benefit of using additional bits to indicate inter-burst time, which may change dynamically due to various reasons, including application-layer rate control. SA4 believes that this can enable the RAN to switch to the most appropriate power state. SA4 kindly requests feedback from SA2 and RAN2 on the value and feasibility of such solution and if that can be supported within Rel. 18 timeframe.

- However, RAN2 had not identified the inter-burst time as particularly useful. Firstly, RAN2 did not understand that how frequently the inter-burst time changes. It was assumed that the periodicity and jitter of a data burst do not change frequently. And RAN2 did not identify whether this information is helpful to RAN, for example whether the information can be useful for power saving or radio resource utilization.

- Following the Release 18 discussion, it is still unclear why the time interval between data bursts changes frequently and what’s the benefits for RAN using it.

RAN2 thinks that it is unclear why the periodicity and time interval between data bursts changes frequently and does not identify that the time to next burst is beneficial for RAN resource scheduling.

3	Conclusion

This paper discusses RAN2 replies to RAN3 LS on UL PSI based PDU discarding in NR-DC (R3-243957) and SA2 LS on FS_XRM Ph2 (S2-2407351). And we propose:

- Observation 1: As currently specified by MAC and PDCP specs, when UE receives an Activation or Deactivation indication from either MCG or SCG or both, UE shall activate or deactivate the PSI based discard.

- RAN2 replies to RAN3 that as currently specified by MAC and PDCP specs, when UE receives an Activation or Deactivation indication from either MCG or SCG, UE shall follow the command and activate or deactivate PSI based discarding. RAN2 thinks network based solutions or implementations should be used for avoiding contradicting instructions of Activation/Deactivation of the PSI based discarding from MN and SN.

- RAN2 thinks that it is unclear why the periodicity and time interval between data bursts changes frequently and does not identify that the time to next burst is beneficial for RAN resource scheduling.

Reference

[1] R3-243957 LS on UL PSI based PDU discarding in NR-DC

[2] S2-2407351. LS on FS_XRM Ph2

[3] R2-2402387	Issues related to PSI-based discard	Qualcomm Incorporated

[4] R2-2304659	LS out on the N6 PDU Set Identification (S4-230739; contact: Intel)	SA4