3GPP TSG-RAN WG2 Meeting #127	R2-241xxxx

Maastricht, Netherlands, 19th – 23rd Aug 2024

Agenda item:		8.7.5

Source:		Lenovo

Title:		AM RLC enhancement

Document for:		Discussion and Decision

- Introduction

In this contribution we discuss RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

- Discussion

The Rel-19 Work Item on XR Phase 3 was agreed in RP-240791.

The Rel-19 XR Phase 3 objectives are as follows: Specify Enhancements for Scheduling, as follows: - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

In RAN2#126, the following discussion on RLC related enhancements for Rel-19 took place with the following agreements:

- For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach - RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. - For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side - For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned - To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism - Impact on capacity should be considered - RAN2 focuses on the enhancements for UL traffic

2.1 Avoiding Unnecessary Retransmissions

In the previous meetings, it has been agreed that for avoiding unnecessary retransmissions, two approaches - Tx-based Approach and Rx-based Approach - will be considered.

The Tx-based approach consists of the RLC transmitter informing its peer entity of the obsolete SDUs and then stops the retransmission of such obsolete SDUs. There are different flavors of this approach, as follows:

- RLC status-report based discard:

In this option, the PDCP transmitting entity provides information to the RLC transmitter of the PDUs whose discardTimer or discardTimerforLowImportance has expired. The RLC transmitter then provides this information of the obsolete SDUs within a new control PDU to the RLC receiver (e.g., Outdated SN Report). In response to such a report, the RLC receiving entity transmits a status report containing an ACK for those SDUs indicated as outdated/ obsolete and updates its reception window, the state variables, timers etc as per legacy procedure. The RLC transmitter upon reception of the status report, discards the obsolete SDUs as per legacy procedure.

- RLC proactive discard:

Here, the PDCP transmitting entity provides information to the RLC transmitter of the PDUs whose discardTimer or discardTimerforLowImportance has expired. The RLC transmitter provides this information of the SDUs for discard within a new control PDU to the RLC receiver (e.g., Discard SN Report). The RLC transmitter then proactively discards theseSDUs and updates its transmission window. The RLC receiver upon reception of the discard SN report, updates its reception window, state variables, timers etc.

- PDCP-controlled update:

With this option, the PDCP transmitter informs the RLC transmitter of the discarded packets and can send the discard information, e. g., PDCP SN gap report to the receiving PDCP entity. The receiving PDCP entity may further notify the RLC receiving entity to update its receiving window respectively, i.e., update the corresponding state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly as needed based on the provided information on discarded packets.

The Rx-based approach consists of the RLC receiver indicating to its peer entity the SDUs that are abandoned, i.e., those RLC SDUs whose discardTimer or discardTimerforLowImportance has expired or those SDUs that are no longer anticipated for by PDCP.

- Timer-based update:

The RLC receiver can maintain an additional timer that dictates how long the RLC AM receiver needs to try to receive/ recover a packet, such that upon expiry of the timer, the RLC receiver transmits a status report to its peer entity containing an ACK for those SDUs that are abandoned. The timer can be started when the RLC delivers a gap (i.e., a packet out of order) to PDCP.

- RLC status-report based discard:

Here, the PDCP receiving entity informs the RLC receiver of the packets whose discardTimer or discardTimerforLowImportance or t-Reordering has expired (i.e., the packets that need to be abandoned). The RLC receiver accordingly transmits a status report containing an ACK for all the SDUs indicated as abandoned to its peer entity. The RLC receiver then updates its reception window, state variables and timers as per legacy procedure. The RLC transmitter, upon reception of the status report, discards the SDUs indicated as ACKed (i.e., the abandoned SDUs) and updates its transmission window as per legacy procedure.

The Tx-initiated as well as Rx-initiated approaches both work to resolve the issue of retransmission of obsolete RLC PDUs. With the Tx-initiated approach, there have been some comparison to the MRW procedure in UMTS where the Tx-initiated approach may be more complex if specified similar to the MRW procedure (due to the need for reliably transmitting the MRW SUFI and receiving the MRW_ACK SUFI). In our view, the Tx-initiated approach may not be so comparable to MRW as RLC no longer concatenates packets, and hence the new RLC ‘Discard SN report’ may be more easily specified (e.g., similar to PDCP SN gap report). Here, additional timers or thresholds may be configured to ensure that the discard SN report is reliably received by the RLC receiver without the need to explicitly specify an additional discard SN report ACKs (corresponding to the MRW_ACK SUFI). Furthermore, the discard of packets at the RLC transmitter could also be based on legacy procedure (i.e., by receiving an ACK from its peer entity as mentioned in the RLC status-report based discard option) to avoid any additional outdated SN report ACK (corresponding to the MRW_ACK SUFI).

The Rx-initiated approach poses a risk of abandonment of RLC SDUs corresponding to PDCP Control PDUs. Hence, some additional mechanism is required to prevent/ combat the loss of PDCP Control PDUs. Furthermore, if t-StatusProhibit is running, the update of transmission window at RLC may be stalled after the RLC receiver has already received the obsolete PDU indication from PDCP, leading to additional delays on the user-plane. Hence, the Tx-initiated approach is preferred.

Observation 1. It is benefical to enable the discard of RLC PDUs associated with an SN whose discardTimer has expired in order to prevent the unnecessary retransmission of such PDUs.

Proposal 1. RAN2 to consider the following options for Tx-Initiated Approach:

-	RLC status-report based discard

-	RLC proactive discard

-	PDCP-controlled update

Proposal 2. RAN2 to consider the following options for Rx-Initiated Approach:

- Timer-based update

- RLC status-report based discard

2.2 Timely Retransmissions

RLC-AM feedback based on current polling parameters are not well adapted for short packet delay budgets applicable to XR traffic. The data volume of PDU set of XR traffic is variable, i.e., the number of PDUs or the size of the PDUs of a PDU set is variable. One possibility, the polling parameter and t-PollRetransmit is set to small value for triggering more frequent polling to the RLC receiver, and gNB frequently transmits status report, which can cause higher overhead.

In legacy, a polling can be triggered if both the transmission buffer and the retransmission buffer become empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU, or if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling). However, if there are multiple PDU sets in transmission or retransmission buffer, the corresponding polling may not be triggered after the transmission of a PDU set.

And the legacy receiving side of an AM RLC entity shall trigger a STATUS report when t-Reassembly expires upon

detection of reception failure of an AMD PDU. However, if the SN of lost RLC PDUs is continuous, the receiver RLC cannot detect the data loss till the correct reception of a RLC PDU with higher RLC SN, therefore the status report may not be triggered timely in this case.

Hence, the RLC retransmission based on current RLC-AM feedback mechanism may not be timely triggered, which causes the packets to arrive too late at the receiver, thus impacting the user experience.

Observation 2. It is difficlut to set the polling related parameters and status prohibit timer for a timely status report, to adapt for XR traffic with short packet delay budgets and variable data volume of PDU set, without significantly increasing the overhead.

Observation 3. If there are multiple PDU sets in transmission or retransmission buffer, the polling may not be triggered after the transmission of a PDU set.

Observation 4. If the SN of lost RLC PDUs is continuous, the status report may not be triggered in a timely manner. If gNB frequently transmits status reports, it causes higher overhead.

To avoid the late RLC retransmission, RLC transmitter may trigger early RLC retransmission for data based on the delay status, provided the transmitting entity did not receive a status report, assuming polling has been transmitted to the peer RLC entity. For example, the PDCP entity delivers a delay-critical indication to the corresponding AM RLC entity, and the delay critical data may be determined based on a separately configured threshold, different than R18 value.

Additionally, another condition that can be considered to enable autonomous retransmissions is the PSI value. For example, only those packets with PSI value greater than a preconfigured threshold are eligible for autonomous retransmissions. Furthermore, the cancellation of an autonomous retransmission can be considered when such a retransmission was triggered and an ACK (e.g., HARQ ACK or a RLC status-report containing an ACK for the corresponding PDU) is received before it is submitted to lower layers. Lastly, RAN2 can also look into scheduling enhancements (e.g., LCP enhancements) for packets that are autonomously retransmitted to avoid any delays in transmission of such data before the discardTimer expires.

In addition, the enhancement of polling mechanism may be considered for the early RLC retransmission. For example, the transmitter RLC entity can trigger a new polling based on the end of PDU set, and if the remaining discard time of the data in the PDU set is below a threshold. Another condition could be that there is more than one PDU set in transmission buffer after transmission of a PDU set, in which case, the AM RLC transmitter entity may poll its peer AM RLC receiver entity in order to trigger STATUS reporting at the peer AM RLC entity. And if gNB receives the polling and a t-StatusProhibit is running, it may ignore the running timer and send a status report to the peer AM RLC receiver entity at least when the status report is triggered by this new polling condition, and if there is a NACK status report to transmit.

Autonomous retransmissions can congest the link if configured poorly, due to excessive transmissions of redundant RLC PDUs. Hence, autonomous retransmission can be network controlled to provide additional control when this function is used, e.g., via RRC configuration for each RLC entity. We think both enhancements can be used together without it conflicting the other, each may use a different threshold. The new polling trigger may avoid/reduce the number of early RLC retransmission.

Proposal 3. To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options:

- RLC transmitter triggers a new polling based on PDU set for data with remaining delay below a threhold.

- RLC transmitter triggers autonomous RLC retransmission for data with remaining delay below a threhold.

- Additional conditions such as PSI-based threshold for triggering autonomous RLC retransmissions.

Proposal 4. The enhanced options to achieve timely retransmissions on RLC layer is configred per RLC entiy by RRC.

- Conclusion

In this contribution, we discuss the RLC AM enhancement for XR traffic with small delay budget. We have the following observation and proposals:

Observation 1. It is benefical to enable the discard of RLC PDUs associated with an SN whose discardTimer has expired in order to prevent the unnecessary retransmission of such PDUs.

Proposal 1. RAN2 to consider the following options for Tx-Initiated Approach:

-	RLC status-report based discard

-	RLC proactive discard

-	PDCP-controlled update

Proposal 2. RAN2 to consider the following options for Rx-Initiated Approach:

- Timer-based update

- RLC status-report based discard

Observation 2. It is difficlut to set the polling related parameters and status prohibit timer for a timely status report, to adapt for XR traffic with short packet delay budgets and variable data volume of PDU set, without significantly increasing the overhead.

Observation 3. If there are multiple PDU sets in transmission or retransmission buffer, the polling may not be triggered after the transmission of a PDU set.

Observation 4. If the SN of lost RLC PDUs is continuous, the status report may not be triggered in a timely manner. If gNB frequently transmits status report, it causes higher overhead.

Proposal 3. To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options:

- RLC transmitter triggers a new polling based on PDU set for data with remaining delay below a threhold.

- RLC transmitter triggers autonomous RLC retransmission for data with remaining delay below a threhold.

- Additional conditions such as PSI-based threshold for triggering autonomous RLC retransmissions.

Proposal 4. The enhanced options to achieve timely retransmissions on RLC layer is configred per RLC entiy by RRC.

- References

TS 38.322 v 18.0.0

R2-2402734, AM RLC enhancement, Lenovo