3GPP TSG-RAN WG2#127											      R2-2406894

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.8.2

Source: 	NEC

Title:	        Consideration on downlink coverage enhancement

Document for:	 Discussion and Decision

- Introduction

In RAN1 #116 [1], three kinds of beam footprints for analytical evaluation were raised as below.

Agreement For system level study based on analytical evaluation: - N1 beam footprints are in state “off” - These beam footprints are not served by any signal (no satellite service in this area) - N2 beam footprints are in state “common messages only” - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access. - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation - N3 beam footprints are in state “active traffic” - These beam footprints have X active (e.g. VoNR) users each. - These beam footprints are also served the necessary information for cell discovery and initial access - N1 + N2 + N3 = “Total number of beam footprints “ - N1, N2, N3, X are to be reported by companies. - Resource utilization obtained under the assumptions above is to be reported by companies. - Other assumptions made in the evaluation are to be reported by companies, e.g. power sharing scheme, beam hopping scheme, etc.

In this contribution, we will discuss cell/beam based DTX for DL coverage enhancement and provide our views.

- Discussion

Due to the limited power and limited feeder link bandwidth, only limited beams can be simultaneously active. In RAN1 #116[1], some agreements about simulation assumptions were reached, for example, only 10.02 and 1.5% beams can be active in case of 1058 and 800 beams in total for FR1 and FR2 respectively, i.e. only a small portion of beams can be simultaneously active.

Besides, RAN1 also raised that three beam states can be used for system level study based on analytical evaluation. Wherein, N1 state means that there is no any served signals in this area covered by N1 beam footprints, i.e. the NW cannot transmit any signals during this state. N3 state is similar to active period of cell DTX of NES or there is no cell/beam DTX, i.e. the normal transmissions can be performed during this state. For N2 state, there is only common messages according to RAN1’s agreement, i.e. only some necessary information for cell discovery and initial access can be transmitted, such as SSB, SIB1, SIB19, paging and messages of random access procedure.

Observation 1: No any signal can be transmitted during the N1 state and only common messages can be transmitted during the N2 state.

Then, in order to make flexible and dynamic power allocation among the different satellite beams for DL coverage enhancement, beam hopping mechanism may be considered for all satellite beams, which means that some beams are active and some beams are non-active within a certain duration. Cell DTX mechanism introduced in Rel-18 NES may be baseline. A possible cell/beam level pattern for a beam transaction among the N1, N2/N3 beam states is shown in Figure 1 below. In this figure, cell/beam is in non-Active state within N1 duration and cell/beam is in Semi-Active state within N2 duration. While during N3 state, cell/beam is in Active state.

Figure 1 Cell/beam DTX mechanism based on three beam states

Proposal 1: In order to achieve beam hopping, cell/beam DTX mechanism should be introduced.

Since the NW cannot transmit any signals during N1 state, whether common information can be impacted or not will be discussed in RAN1. In our understanding, even if common information cannot be impacted, for example, SSB, SIB1/19 and paging are only transmitted during N2/N3 state, since random access procedure may not be completed during N2/N3 state (considering limited dwell time), transmission of some messages during random access procedure may also be impacted by cell/beam DTX pattern, such as Msg2, Msg3 and Msg4. Hence, it is better to transmit cell/beam DTX pattern to UEs in RRC_IDLE/RRC_INACTIVE state.

Proposal 2: Cell/beam DTX/DRX pattern can be informed to the UE in RRC_IDLE/INACTIVE state.

When cell/beam DTX is configured, even if all RACH occasions are configured within the dwell time, it is possible that running RAR window/MSGB window can encounter N1 state as shown in figure 2 below. As we know, for NTN, only the maximum 10ms RAR window may be configured, RAR window can expire during N1 state, such that random access procedure fails, i.e. cell/beam DTX can increase failure number of random access procedure even if 40ms window is used.

Figure 2 Cell/beam DTX impact on random access procedure

Another case is that when Msg2 is received, Msg3 can be transmitted during N1 state based on the timing relation as shown in figure 3 below. If the UE transmits Msg3 during N1 state, even if the network receives Msg3, it only transmits Msg4 during N2/N3 state. Then it is very possible that contention resolution timer expires, which can also make random access failure.

Figure 3 Cell/beam DTX impact on random access procedure

Based on the analysis above, when cell/beam DTX is configured, some impacts on random access procedure needs to be considered.

Proposal 3: When cell/beam DTX is configured, some impacts on random access procedure needs to be considered.

- Conclusion

This contribution provides our analysis and views for the DL coverage enhancement, the following observation and proposals are listed below:

Observation 1: No any signal can be transmitted during the N1 state and only common messages can be transmitted during the N2 state.

Proposal 1: In order to achieve beam hopping, cell/beam DTX mechanism should be introduced.

Proposal 2: Cell/beam DTX/DRX pattern can be informed to the UE in RRC_IDLE/INACTIVE state.

Proposal 3: When cell/beam DTX is configured, some impacts on random access procedure needs to be considered.

- References

[1] Draft_Minutes_report_RAN1#116_v010.