3GPP TSG-RAN WG2 Meeting #127	           R2-2406895

Maastricht, Netherlands, Aug 19th – Aug 23rd, 2024

Agenda item:	8.5.2

Source:	China Telecom

Title:	Discussion on on-demand SSB

Document for:	Discussion

- Introduction

According to the WID [1], one of the objectives is on-demand SSB SCell operation:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

RAN1 have agreed to support on-demand SSB SCell operation triggered by gNB and focus on the design of solutions for on-demand SSB triggered by gNB in recent meetings. In this paper, we would like to discussion on the three potential triggering methods from RAN2’s perspective.

- Discussion

RAN1 have agreed to support on-demand SSB SCell operation triggered by gNB. But it is not clear whether to support the other two triggering methods or not. A high level analysis of the RAN2 impacts of the three triggering methods is as the following:

- SCell activation/deactivation signaling

In this method, on demand SSB transmission in SCell is triggered and decided by the network. The RAN2 impact may also include UE behavior besides SCell activation/deactivation signaling. On the other hand, whether to send SCell SSB/activate SCell or not is up to the network implementation or other inputs needs to be further discussed.

- UE uplink wake-up-signal using an existing signal/channel

It seems that the method has more RAN2 impact than the other two. On demand SSB transmission in SCell is triggered by the UE in connected mode configured with CA. The final decision should be made by the network.

Before we talk about the RAN2 impact of this method, whether L1 signalling, L2 signalling or RRC signalling is used as the UE uplink wake-up-signal needs to be decided, which may be decided by RAN1. If L2 signalling or RRC signalling is selected, of course RAN2 spec will be impacted to adopt the signalling design. The subsequent procedure corresponding to the specific signalling also needs be further discussed.

- cell on/off indication via backhaul

In our understanding, the impact of “cell on/off indication via backhaul” is mainly about inter-node signaling, and the impact is limited. This triggering method can be used alone or be used together with SCell activation/ deactivation signaling or UE uplink wake-up-signal method. If the method is used alone, it means that the UE may not know when the SCell without SSB begins to send SSB. Thus the UE may not know when to begin the SSB monitoring or the UE has to blindly monitor the SSB.

Generally, from RAN2’s perspective, the three triggering methods are feasible. Taking the whole solution into account, we think the potential RAN2 impacts of the three triggering methods overlap with each other. A combination of different triggering methods can provide more flexibility for the network regarding the on demand SSB SCell operation.

Proposal 1: From RAN2’s perspective, the triggering methods of UE uplink wake-up-signal using an existing signal/channel, and cell on/off indication via backhaul are also feasible, which can provide more flexibility for the network.

Proposal 2: Before discussing on-demand SSB SCell operation triggered by UE, RAN2 wait for RAN1’s decision on whether L1 signaling, L2 signaling or RRC signaling is used as the UE uplink wake-up-signal.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: From RAN2’s perspective, the triggering methods of UE uplink wake-up-signal using an existing signal/channel, and cell on/off indication via backhaul are also feasible, which can provide more flexibility for the network.

Proposal 2: Before discussing on-demand SSB SCell operation triggered by UE, RAN2 wait for RAN1’s decision on whether L1 signaling, L2 signaling or RRC signaling is used as the UE uplink wake-up-signal.

4	References

- [1] RP-241650, Revised WID: Enhancements of network energy savings for NR, Ericsson, Apple.