3GPP TSG-RAN WG2 Meeting #127	           R2-2406896

Maastricht, Netherlands, Aug 19th – Aug 23rd, 2024

Agenda item:	8.5.3

Source:	China Telecom

Title:	Discussion on left issues of on-demand SIB1

Document for:	Discussion

- Introduction

According to the WID [1], the checkpoint for normative work is in RAN#105. RAN2 has made good progress on on-demand SIB1 during the past meetings. In this paper, we discuss on the left issues of on-demand SIB1 and further clarify the scenarios.

2. Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] • Triggering method by uplink wake-up-signal using an existing signal/channel. • Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective • Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. • Checkpoint for normative work in RAN#105

- Discussion

RAN2 has discussed whether the UE can acquire UL WUS configuration from the NES cell after the UE camps on NES cell during RAN2#126 meeting, but no agreement was reached.

When the UE stays in the NES cell, the UE may need to re-acquire SIB1. According to the agreements, once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell. Then, the UE can acquire other SIBs from the NES cell using legacy mechanism after re-acquiring of SIB1 from the NES cell. It is natural for the UE to obtain UL WUS configuration from the SIB of the camped NES cell, since the UE is able to obtain all SIBs from the camped NES cell. However, the assumption is the UL WUS configuration remains unchanged.

The question is how to update UL WUS configuration after the UE camps on the NES cell. If we support the UE obtains the updated UL WUS configuration from the NES cell after the UE camps on NES cell, it means that scenario 3 (i.e. Case 1 in RAN1) is supported. However, Case 1 has been excluded in RAN1. Then the UE reacquires UL WUS configuration from cell A.

Proposal 1: After the UE camps on NES cell, UE reacquire UL WUS configuration from cell A when the UL WUS configuration changes.

Furthermore, we think the UE should be allowed to reselect from one NES cell to another NES cell. In this scenario, the UE obtain the UL WUS configuration of neighbor NES cell from the camped NES cell before sending UL WUS to the neighbor NES cell. Thus we suggest to further clarify Scenario 1a based on the version agreed during RAN2#126 meeting as below:

- Scenario 1a: Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell. Cell A can be the neighbor NES cell with on-demand SIB1 or the neighbor cell with SIB1 always on.

a.	More than one Cell A may provide configuration for the same NES cell.

b.The same Cell A may assist more than one NES Cells.

Proposal 2: The Cell A mentioned in Scenario 1a can be a neighbor NES cell with on-demand SIB1 or a neighbor cell with SIB1 always on.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: After the UE camps on NES cell, UE reacquire UL WUS configuration from cell A when the UL WUS configuration changes.

Proposal 2: The Cell A mentioned in Scenario 1a can be a neighbor NES cell with on-demand SIB1 or a neighbor cell with SIB1 always on.

4	References

- [1] RP-241650, Revised WID: Enhancements of network energy savings for NR, Ericsson, Apple.