3GPP TSG-RAN WG2 Meeting #127	           R2-2406897

Maastricht, Netherlands, Aug 19th – Aug 23rd, 2024

Agenda item:	8.5.4

Source:	China Telecom

Title:	Discussion of adaption of paging occasions

Document for:	Discussion

- Introduction

RAN2 have the following agreements on adaption of paging occasions up to now:

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

- For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

In this paper, we would like to further discuss the legacy UE compatibility options and paging adaptation options captured above.

- Discussion

In terms of compatibility of legacy RRC_IDLE/RRC_INACTIVE UE, although RAN1 have no agreements on paging adaption, RAN1#117 meeting have the following agreements about PRACH adaptation:

For adaptation of PRACH in time-domain, support at least the following case(s) - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains - FFS: whether additional conditions are needed to support the above cases - FFS: Additional case whether full/partial overlap in both time and frequency is allowed - Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources

According to the above agreements, separate PRACH resources are configured for legacy UEs and Rel-19 NES UEs. In our understanding, the logic also applies to paging adaption compatibility of legacy RRC_IDLE/RRC_INACTIVE UE, i.e. paging resources for legacy UEs and Rel-19 NES UEs are separate.

However, if both paging resources for legacy UEs and Rel-19 NES UEs are configured by the network, it is possible that the wake-up total POs in NES cell is more than the normal cell. For example, the PFs/POs calculated by legacy UEs camped on the cell providing separate paging resources are disperse enough. Whether or when to turn on paging adaption is up to network implementation.

Proposal 1: Separate paging resources for legacy UEs and Rel-19 NES UEs, i.e. Option 2, is used for the compatibility of legacy RRC_IDLE/RRC_INACTIVE UE.

Regarding high level solutions of paging occasions adaptation in time domain, we think that extend the values of N to have increased interval between PFs and compensating decrease in number of PFs by increasing POs per PF, i.e. solution b), has minimum spec impact and reuses the legacy principle.

Proposal 2: For adaptation of paging occasions in time domain, extend the values of N to have increased interval between PFs and compensating decrease in number of PFs by increasing POs per PF.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: Separate paging resources for legacy UEs and Rel-19 NES UEs, i.e. Option 2, is used for the compatibility of legacy RRC_IDLE/RRC_INACTIVE UE.

Proposal 2: For adaptation of paging occasions in time domain, extend the values of N to have increased interval between PFs and compensating decrease in number of PFs by increasing POs per PF.