3GPP TSG-RAN WG2 Meeting #127	           R2-2406898

Maastricht, Netherlands, Aug 19th – Aug 23rd, 2024

Agenda item:	8.13.2

Source:	China Telecom

Title:	Discussion on multi-hop relay discovery and reselection

Document for:	Discussion

- Introduction

According to the WID [1], one of the objectives is:

1. Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. A. Relay discovery and (re)selection [RAN2] B. Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] C. Impact on SRAP and QoS handling for multi-hop [RAN2] D. Control plane procedures [RAN2, RAN3]

In this paper, we would like to discuss the potential enhancements to relay discovery and (re)selection to support additional hops.

- Discussion

2.1	General

Rel-17 introduced single-hop sidelink based L2 and L3 U2N Relay. The L2/L3 U2N Relay UE should be in coverage. However, for multi-hop relay scenario, the Relay UE(s) except last Relay UE, i.e., first Relay UE or second Relay UE, can be in coverage or out of coverage, since it communicates with gNB via last Relay UE.

Proposal 1: first relay UE or second relay UE can be in coverage or out of coverage.

For backward compatibility, in our understanding, Rel-19 U2N Relay UE(s) should support Rel-17 U2N Remote UE(s), in order to make more Remote UEs located in poor coverage able to communicate with the network via Relay UE(s).

Proposal 2: RAN2’s work assumption is that Rel-19 U2N Relay UE(s) support Rel-17 U2N Remote UE(s).

2.2	Relay discovery

The Rel-17 U2N Relay UE uses a maximum Uu RSRP threshold, a minimum Uu RSRP threshold, or both, which are configured by the network, to determine if it can transmit Relay discovery messages to U2N Remote UE(s). This is also applicable for last relay UE in Rel-19. However, the Uu RSRP measurement is unavailable for Rel-19 U2N Relay UE which may be out of coverage, such as first relay and second relay.

Take one additional hop relay scenario for example, first relay UE out of coverage can use a maximum SD-RSRP/SL-RSRP threshold, a minimum SD-RSRP/SL-RSRP threshold, or both, to determine if it can transmit Relay discovery messages to U2N Remote UE(s). The maximum SD-RSRP/SL-RSRP threshold, and minimum SD-RSRP/SL-RSRP threshold can be configured by the network via RRC signalling. With the extended threshold conditions, the interference introduced by first relay UE is limited while close enough to last relay UE. In addition, the extended threshold conditions also apply to two additional hops relay scenario.

Proposal 3: first relay UE or second relay UE can use a maximum SD-RSRP threshold, a minimum SD-RSRP threshold, or both, to determine if it can transmit Relay discovery messages to U2N Remote UE(s).

Proposal 4: first relay UE or second relay UE can use a maximum SL-RSRP threshold, a minimum SL-RSRP threshold, or both, to determine if it can transmit Relay discovery messages to U2N Remote UE(s).

2.3	Relay selection/reselection

The Rel-17 U2N Remote UE performs SL-RSRP/SD-RSRP measurements and uses them for Relay selection and reselection radio criteria. A Rel-17 U2N Relay UE is considered suitable by a Rel-17 U2N Remote UE in terms of radio criteria if the SL-RSRP/SD-RSRP measured by U2N Remote UE towards the U2N Relay UE exceeds configured threshold. The SL-RSRP/SD-RSRP threshold introduced for relay selection/reselection in Rel-17 can be reused for Rel-19 U2N Remote UE to select/reselect multi-hop relay UE, since the expected minimum PC5 signal quality of each hop can be the same.

Proposal 5: the SL-RSRP/SD-RSRP threshold introduced for relay selection/reselection in Rel-17 can be reused in Rel-19.

Rel-17 defined the following cases triggering U2N Relay UE reselection:

-	PC5 signal strength of current U2N Relay UE is below a (pre)configured signal strength threshold;

-	Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by U2N Relay UE via PC5-RRC signalling;

-	When U2N Remote UE receives a PC5-S link release message from U2N Relay UE;

-	When U2N Remote UE detects PC5 RLF;

-	Indicated by upper layer.

However, in terms of the second bullet, multi-hop relay will introduce new cases. Single-hop U2N Relay UE communicates with the network directly, which can be considered as last relay UE in multi-hop relay. The first relay UE may perform path switch, such as switch from indirect path to direct path, indirect path to indirect path. In that case, the first relay UE may need to indicate path switch to the connected remote UE, which may trigger the remote UE to perform relay UE reselection. In another case, the first relay may reselect its relay UE, and the reselected relay UE’s serving cell may be different from or the same as the former serving cell. Thus we think Rel-19 will introduce new indication type to trigger U2N Relay UE reselection, such as path switch.

Proposal 6: introduce new indication type to trigger U2N Relay UE reselection, such as path switch.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: first relay UE or second relay UE can be in coverage or out of coverage.

Proposal 2: RAN2’s work assumption is that Rel-19 U2N Relay UE(s) support Rel-17 U2N Remote UE(s).

Proposal 3: first relay UE or second relay UE can use a maximum SD-RSRP threshold, a minimum SD-RSRP threshold, or both, to determine if it can transmit Relay discovery messages to U2N Remote UE(s).

Proposal 4: first relay UE or second relay UE can use a maximum SL-RSRP threshold, a minimum SL-RSRP threshold, or both, to determine if it can transmit Relay discovery messages to U2N Remote UE(s).

Proposal 5: the SL-RSRP/SD-RSRP threshold introduced for relay selection/reselection in Rel-17 can be reused in Rel-19.

Proposal 6: introduce new indication type to trigger U2N Relay UE reselection, such as path switch.

4	References

- [1] RP-241609, New WID on NR sidelink multi-hop relay, LG Electronics, InterDigital, FirstNet.