3GPP TSG-RAN WG2 Meeting #127	R2-2406899

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	China Telecom

Title:	Random access procedure for Ambient IoT

Agenda Item:	8.2.4

Document for:	Discussion and Decision

- Introduction

In RAN2#126 meeting, random access for Ambient IoT has been discussed and following agreements were reached [1]:

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

4  The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

5  “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

1   A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

2   A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.   “Msg2” usage/presence can be further discussed

Agreement

-	From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

Based on the existing process of discussions, A-IoT random access steps including failure handling need to be further discussed. In this contribution, we provide our views on procedures of random access for A-IoT in detail.

- Random access Procedures

2.1 Contention-based resolution

In the last meeting, we discussed the basic procedures for CB-RA. The content required for the different procedures and failure handling are necessary to be discussed now. For the convenience of study, the terminology of ‘access occasion’ and ‘access round’ should be defined. Different from NR RACH, the A-IoT device has extremely low timing accuracy. Therefore, we can adopt a message to indicate the start/end of one or multiple devices’ access. Msg0 can be used to start one access round. Access occasion means the opportunity for one device to finish access. One access round can include multiple access occasions, and the number of access occasions in one round depends on the reader’s indication. The device can randomly select access occasions according to the indication from Msg0.

Proposal 1: One access round may consist of multiple access occasions.

Proposal 2: The device can randomly select one occasion in one access round.

- For 3-step CB-RA, the device sends an ID to the reader by A-IoT Msg1. Referring to the design of RFID, this ID is randomly generated. If the reader successfully receives Msg1 and further determines there are no collisions, the reader replies with an response message to the device denoted as Msg2, which includes the received temporary device identification and ACK message. If the device receives the next access occasion/round message instead of the access response Msg2 after a period of time, it indicates RA failure on the previous occasion.

Proposal 3: For the A-IoT Msg1 during the 3-step CB-RA process, the ID is randomly generated by the device.

Proposal 4: For 3-step CB-RA, A-IoT Msg2 carries the ACK message and the temporary device identification received from the device.

Proposal 5: For 3-step CB-RA, if the device receives the next access occasion/round message instead of the access response Msg2, it indicates RA failure.

Regarding the success or failure of A-IoT Msg3, the transmission of UL/DL data can indicate the success of Msg3. Due to transmission collision of multiple devices or A-IoT Msg3 transmission failure, the reader needs to send a failure indication message to the device when it does not receive A-IoT Msg3 after sending A-IoT Msg2. This action also indicates the device should re-access later. How to achieve re-access (i.e. select another occasion in this round to re-access or re-access in the next round) should be further discussed. In our understanding, if collision events or response loss happens resulting in access failure, these failed devices can re-access in the next access round according to the indication by the reader.

Proposal 6: The reader should send a failure indication message to the device if it can't receive the A-IoT Msg3 after sending A-IoT Msg2.

Proposal 7: RAN2 to support that A-IoT devices can re-access in the next access round if access failure occurs.

For 2-step CB-RA in A-IoT, it is different from the 2-step RA procedures specified in NR Uu because there may be only one PDRCH UL channel for A-IoT based on RAN1’s agreement. Considering A-IoT Msg1 is the only UL RA message, it should send the permanent device ID to the reader. We think it is unnecessary to send the random ID because it will cause additional power assumptions and signalling overhead. Then, when the reader receives A-IoT Msg1, it will send Msg2 with an ACK message and the received device ID.

Proposal 8: A-IoT Msg 1 in 2-step CB-RA contains the permanent device ID and A-IoT Msg 2 contains the above ID and ACK message.

2.2 Contention-free resolution

As for the contention-free access procedures, it is necessary for A-IoT devices. If the device has been identified by the reader, the reader can reserve its priori information and channel resource for a moment. The detail procedures for the contention-free solution are shown in Fig.1.

- Reader sends trigger signal containing device identifier and scheduling information denoted as Msg0. For the generation of scheduling information, we think it can be further discussed.

- The AIoT device receives the trigger signal. If it can successfully decode the signal and match the device identifier, it will make contention-free access based on the scheduling information and respond reader by Msg1 with a temporary ID.

- Once decoding Msg1 by the reader, the reader can respond ACK. If not, the reader can indicate a NACK. This step is also optional. Through the subsequent data transmission process, it can indicate the success of the access procedure.

Fig.1. a basic contention-free access procedure

Proposal 9: For contention-free access procedure, the reader sends a trigger signal containing the device identifier and scheduling information; When an A-IoT device receives the trigger signal, it can respond including a temporary ID message if the device identifier matches.

Proposal 10: Whether the reader sends ACK/NACK to the device after receiving Msg1 can be further discussed.

2.3 Other aspects of RA procedure

In legacy NR systems, UE can know which random access methods are supported by network via SIB, and decide which random access method to use based on conditions such as measured RSRP. RAN2 is currently investigating both 3-steps CB-RA and 2-steps CB-RA. Since there are no legacy SIB messages in AIoT scenarios and the terminal does not support measurements, etc., RAN2 needs to discuss whether both methods will be supported in the future and how to support them if both are supported.

Proposal 11: RAN2 needs to discuss whether both methods will be supported in the future and how to support them if both are supported.

- Conclusion

In this contribution, we discuss the random access procedures for Ambient IoT, and have the following proposals:

Proposal 1: One access round may consist of multiple access occasions.

Proposal 2: The device can randomly select one occasion in one access round.

Proposal 3: For the A-IoT Msg1 during the 3-step CB-RA process, the ID is randomly generated by the device.

Proposal 4: For 3-step CB-RA, A-IoT Msg2 carries the ACK message and the temporary device identification received from the device.

Proposal 5: For 3-step CB-RA, if the device receives the next access occasion/round message instead of the access response Msg2, it indicates RA failure.

Proposal 6: The reader should send a failure indication message to the device if it can't receive the A-IoT Msg3 after sending A-IoT Msg2.

Proposal 7: RAN2 to support that A-IoT devices can re-access in the next access round if access failure occurs.

Proposal 8: A-IoT Msg 1 in 2-step CB-RA contains the permanent device ID and A-IoT Msg 2 contains the above ID and ACK message.

Proposal 9: For contention-free access procedure, the reader sends a trigger signal containing the device identifier and scheduling information; When an A-IoT device receives the trigger signal, it can respond including a temporary ID message if the device identifier matches.

Proposal 10: Whether the reader sends ACK/NACK to the device after receiving Msg1 can be further discussed.

Proposal 11: RAN2 needs to discuss whether both methods will be supported in the future and how to support them if both are supported.

# References

- 3GPP TSG-RAN WG2 Meeting #126 chairman notes.