3GPP TSG-RAN WG2 Meeting #127	R2-2406900

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.4.2

Source:	China Telecom

Title:	LP-WUS Operation in RRC_IDLE/INACTIVE

Document for:	Discussion and Decision

- Introduction

In the RAN2’s last meeting, the progress is captured in [1]:

- RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. - The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - - LP-SS configuration - - LP-WUS configuration - - Entry/exit condition for LP-WUS monitoring (FFS if it always configured) - Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. - Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. - RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. - RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8. - From RAN2 perspective, no new procedure is introduced for SI reception/updates.

In this contribution, we discuss the procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE modes including sub-grouping, time gap, entry/exit condition, configuration and dynamic PO aspects.

- Discussion

Time gap between LP-WUS and PDCCH monitoring

According to RAN1 agreements, the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring for RRC_CONNECTED is introduced for RRC CONNECTED mode. Similarly, this kind of parameter is also necessary to be defined for RRC IDLE/INACTIVE modes. When LP-WUS and PEI are simultaneously activated, the time gap refers to the offset between LP-WUS reception and MR monitoring for PEI. If UE does not support PEI or PEI is not activated, the time gap refers to the offset between LP-WUS reception and MR monitoring for paging. It is efficient to configure the minimum time gap through SIB. The value of the minimum time gap is up to RAN1 discussion.

Proposal 1. RAN2 to define the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring for RRC IDLE/INACTIVE modes.

Enable/disable LP-WUS monitoring

The LP-WUS configuration by broadcasting is cell-specific and common for all the UEs in the cell. However, it is not applicable to enable LP-WUS monitoring for all users. Considering different device types or traffic types, it may be necessary to enable/disable a certain user or some kinds of users to monitor LP-WUS. Moreover, it’s better to re-enable LP-WUS monitoring for these users that generate movement or location updates. This relevant problem has been discussed during the SI stage and no agreements have been achieved. We have the view that RAN2 to discuss the method to enable/disable LP-WUS monitoring per UE.

Proposal 2. RAN2 to discuss the method to enable/disable LP-WUS monitoring per UE.

LP-WUS Subgrouping

In the last RAN2 meeting, many companies proposed the method of UE_ID-based subgrouping for LP-WUS, but no consensus has been reached due to time constraints. We have agreed that the PEI subgrouping method is taken as a baseline for LP-WUS subgrouping. A similar formula as PEI can be used as a baseline for the UE_ID based subgrouping. However, in case LP-WUS is used together with PEI, an independent subgrouping ID should be designed for LP-WUS and PEI separately to reduce false alarm rates. In this way, UE from the same LP-WUS subgroup can be relevant to different PEI subgroups. In order to reduce the impact on the modification of Spec, we can modify the parameters (i.e. UE_ID) in the formula as PEI. FFS on the details.

Proposal 3. For UE_ID-based subgrouping for LP-WUS, we can modify the parameters based on the design of PEI. FFS on the details.

Entry/exit condition of using LP-WUS

Regarding the entry/exit condition of using LP-WUS, RAN2 has reached a consensus on the basic conditions. Based on the existing agreements for entry conditions of using LP-WUS, when the serving cell measurement performed by MR is above threshold(s), it is not safe to start to monitor LP-WUS because the coverage of LP-WUS may be not good enough to monitor LP-WUS. Therefore, additional measurement by LR is essential. Only after LR measurement also satisfies the threshold, UE starts monitoring LP-WUS.

Proposal 4. For the entry conditions of using LP-WUS, LP measurement is also needed except for MR measurement.

Configuration

RAN1 has divided the UE into three categories including OFDM-based WUR, OOK-based WUR and the UE supports the above two types of WURs. Different kinds of devices can bring different performance gains, i.e. latency, and accuracy. Therefore, the categories of WUR can be configured in SIB to indicate users in the cell.

Proposal 5. Which kind of WUR(i.e. OFDM-based, OOK-based, both types) can be configured in SIB.

Whether to support dynamic PO

As for the dynamic PO, RAN1 has discussed and reached the following agreement:

Agreement It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up. - FFS: support of UE monitoring dynamic PO

The dynamic PO indicates that UE starts to monitor PO after receiving network indication, which is different from waiting for legacy paging occasion. From the perspective of latency, the dynamic PO can indeed reduce latency. However, it also leads to complex signalling processes and protocol impacts. Thus, we think it is not necessary to support dynamic PO at this stage.

Proposal 6. From the RAN2 perspective, it is not necessary to support dynamic PO at this stage.

- Conclusion

Based on the above discussion, we have the following proposals.

Proposal 1. RAN2 to define the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring for RRC IDLE/INACTIVE modes.

Proposal 2. RAN2 to discuss the method to enable/disable LP-WUS monitoring per UE.

Proposal 3. For UE_ID-based subgrouping for LP-WUS, we can modify the parameters based on the design of PEI. FFS on the details.

Proposal 4. For the entry conditions of using LP-WUS, LP measurement is also needed except for MR measurement.

Proposal 5. Which kind of WUR(i.e. OFDM-based, OOK-based, both types) can be configured in SIB.

Proposal 6. From the RAN2 perspective, it is not necessary to support dynamic PO at this stage.

- References

3GPP TSG RAN WG2 #126 RAN2 Chair’s Notes.