3GPP TSG-RAN WG2 Meeting #127	R2-2406901

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.4.4

Source:	China Telecom

Title:	Discussion on LP-WUS in RRC_CONNECTED

Document for:	Discussion and Decision

- Introduction

According to the objective for LP-WUS in the new WID [1], we discussed the relevant problems in the last RAN2#126 meeting with the following agreements [2].

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

In this contribution, we will discuss the RAN2-related issues on the procedure of LP-WUS for RRC_CONNECTED mode.

- Discussion

As for the activation and deactivation of LP-WUS monitoring, RAN1 has reached the following agreements in RAN1#116 meeting [3]:

Agreement - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

Based on the above agreement, in case LP-WUS monitoring is enabled, there would exist at least one mechanism for activation/deactivation of LP-WUS monitoring, which can improve LP-WUS monitoring efficiency on the condition of the poor channel. In our view, we think that Option 2 is more flexible. It can be a new signalling or existing signalling to activate/deactivate LP-WUS monitoring. However, the detailed design also depends on RAN1’s further discussion. For Option 3, if the channel quality is not good enough to transmit LP-WUS when the timer is still running, it may be invalid for UE to monitor the LP-WUS. Thus, this option is not more suitable than Option 2. Option 4 is also feasible as discussed in the SI stage, we can postpone the discussion on it according to the progress of the LP-WUS operation.

Proposal 1: In RRC_CONNECTED mode, option 2 is more flexible for activation/deactivation of LP-WUS monitoring. Details are FFS.

In RAN1#116-bis meeting, RAN1 further discussed the LP-WUS procedures to trigger PDCCH monitoring, which is summarized as follows [4]:

Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered. - RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2 - Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any

For Option 1-1, RAN2 has a consensus that the LP-WUS monitoring occasion is located at a configured time offset before the start of drx-onDurationTimer. The specific configuration method for the time offset can be further discussed by RAN2. The simplest method is through SIB to indicate the time offset from the RAN2 perspective. MR may select its own sleep state according to the configured time offset. Moreover, it can also configured by special RRC signalling. Before configurations, UE may report its supported time offset to gNB for reference in subsequent configurations. The specific method needs to be further discussed.

Proposal 2: For Option 1-1, the time offset can be configured by SIB or RRC signalling. Details are FFS.

For the LP-WUS procedures to trigger PDCCH monitoring, we have discussed Option 1-1. For Option 1-2, LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. Regarding whether UE performs PDCCH monitoring during legacy C-DRX active time, RAN1 provides two options as follows:

- Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS. This method can be understood as adding additional LP-WUS monitoring outside C-DRX active time based on Option 1-1, which means that there are two different mechanisms to monitor PDCCH during the outside C-DRX active time. We think it brings high complexity to execute and has huge impact on the spec.

- Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS. This is an independent process to monitor PDCCH during the whole DRX cycle and the action of UE is aligned for PDCCH monitoring. Hence, it is more flexible and needs to be further discussed from the RAN2 perspective.

Proposal 3: From the RAN2 perspective, Option 1-2-1 can be excluded and Option 1-2-2 needs to be further discussed.

Regarding how to stop PDCCH monitoring, we think it can refer to the design of Rel-17, i.e. using PDCCH skipping or a timer to indicate the end of PDCCH monitoring. Whether to define a new timer or reuse a  legacy DRX timer can be discussed.

Proposal 4: To stop PDCCH monitoring triggered by LP-WUS, a PDCCH skipping indication or a timer can be considered.

- Conclusion

In this contribution, we discuss the procedures to monitor PDCCH triggered by LP-WUS for RRC_CONNECTED mode. The proposals are summarized as follows.

Proposal 1: In RRC_CONNECTED mode, option 2 is more flexible for activation/deactivation of LP-WUS monitoring. Details are FFS.

Proposal 2: For Option 1-1, the time offset can be configured by SIB or RRC signalling. Details are FFS.

Proposal 3: From the RAN2 perspective, Option 1-2-1 can be excluded and Option 1-2-2 needs to be further discussed.

Proposal 4: To stop PDCCH monitoring triggered by LP-WUS, a PDCCH skipping indication or a timer can be considered.

- References

- RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR).

- RAN2#126 chairman notes.

- RAN1#116 chairman notes.

- RAN1#116bis chairman notes.